RPD$075066001

RSH$0505

CHP$U650  CHARGE (VMS & ULTRIX version) - SAYER

TPF$

CEB$

EQUILIBRIUM CHARGE STATE DISTRIBUTIONS

BLP$

CHARGE is an interactive program for prediction of equilibrium

charge state distributions in carbon foil and dilute gaseous

strippers. The predictions are based on semi-empirical

formulas developed previously (1,2) for the most probable

charge  Q0,  width  RHO,  and skewness  EPS as functions of

projectile atomic number Z  and velocity  Bc. The assumed

distribution was of the form:

CY1$

2

-0.5 t / (1 + EPS*t)

F(Q) = F(Q0) e

where

F(Q) = intensity for charge Q

t  =  (Q - Q0)/RHO

RHO = width

EPS = asymmetry parameter

A summary of the stripping formulas is given on page 2.

BLP$

If the target Z is input, the ionization is computed from the

carbon formulas on page 2 and then corrected for target Z dependence

(ref 3).  The correction is about 2-5% for Ni and 5-10% for Au.

CYB$

Type:  /usr/hhirf/charge   ;To start execution on DECstation

Type:  @U1:[MILNER]CHARGE  ;To start execution on VAX

BLP$

You will be prompted for the projectile atomic number, mass,

minimum energy, and maximum energy.   The maximum energy may be

omitted if predictions for only one projectile energy are desired.

The results will be output to your terminal and to \b\charge.log

if lon\\ is requested. For example:

CY1$

orph20>\b\charge\\

Enter projectile  Z, A, (target Z) --------->\b\ 12 24\\

Enter MIN and MAX projectile energy in MeV ->\b\ 20 200\\

Do another case ?  Y or N ------------------>\b\ n\\

BLP$

produces the output shown on page 3.   Note that the requested energy

range of 20 to 200 MeV was truncated because the range of the

semi-empirical formula for the most probable charge was exceeded.

CY1$

---------------------------------------------------------------------------

ODP$

(1)....R. O. Sayer, Revue de Physique Appliquee 12, 1543 (1977).

ODP$

(2)....R. O. Sayer and L. B. Maddox, "Predictions of Equilibrium

Charge State Distributions:  A Charge State Handbook,"

Internal Lab Memo, 1978.

ODP$

(3)....K. Shima, T. Ishihara, and T. Mikumo, Nucl. Inst. Meth. 200,

605 (1982).

TPF$

CEN$

\b\Semi-Empirical Stripping Formulas (Carbon Foils)\\

[ iZ = most probable charge,  B = v/c ]

CYB$

For  Z  =  6  thru  15  ...................................................

CY1$

-0.421  0.890

-55.8 Z       B              (0.15 < i < 0.95)

i = 1 - e

0.51          0.20

RHO = 0.38 Z    (i(1 - i))

EPS = 0

CYB$

For  Z  greater than  15  ................................................

CY1$

-0.380  0.860

-47.3 Z       B         (0.15 < i < 0.95)

i = 1 - 1.03 e

0.45          0.26

RHO = 0.48 Z    (i(1 - i))

EPS/RHO = 0.0007 Z  -  0.7

CEN$

\b\Semi-Empirical Stripping Formulas (Dilute gases)\\

[ iZ = most probable charge,  B = v/c ]

CYB$

For  Z  greater than  5  ..................................................

CY1$

0.55          0.27

RHO = 0.35 Z    (i(1 - i))

EPS/RHO = 0.17 + 0.0012 Z  -  3.3 B

CYB$

For  Z  =  6  thru  15  ...................................................

CY1$

-0.530  0.992

-49.5 Z       B         (0.03 < i < 0.07)

i = 1 - 1.02 e

-0.530  0.992

-86.8 Z       B         (0.07 < i < 0.90)

i = 1 - 1.08 e

CYB$

For  Z  greater than  15  .................................................

CY1$

-0.506  0.996

-49.5 Z       B         (0.03 < i < 0.07)

i = 1 - 1.02 e

-0.506  0.996

-80.1 Z       B         (0.07 < i < 0.90)

i = 1 - 1.08 e

TPF$

CYB$

Equilibrium Charge State Predictions for Mg- 24 Ions in: CARBON  FOIL

CY1$

E =  20.0  38.0  56.0  74.0  92.0 110.0 128.0 146.0 164.0 MeV

E/A =  0.83  1.58  2.33  3.08  3.83  4.58  5.33  6.08  6.83

Q0 =   8.3   9.5  10.1  10.5  10.8  11.0  11.2  11.3  11.4

RHO =  0.98  0.93  0.89  0.85  0.82  0.79  0.77  0.75  0.73

EPS =  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00  0.00

QAVG=   8.3   9.5  10.1  10.5  10.8  11.0  11.2  11.3  11.4

Q = 5  0.14

6  2.65  0.04

7 17.13  1.23  0.09

8 39.04 12.10  2.61  0.58  0.14  0.04  0.01

9 31.30 37.54 20.30  9.44  4.32  2.02  0.97  0.48  0.24

10  8.83 36.68 44.48 38.65 29.94 22.24 16.30 11.95  8.79

11  0.88 11.28 27.49 40.08 47.22 50.20 50.57 49.44 47.52

12  0.03  1.09  4.79 10.52 16.96 23.25 29.00 34.12 38.63

CYB$

Equilibrium Charge State Predictions for Mg- 24 Ions in: DILUTE GASES

CY1$

E =  20.0  38.0  56.0  74.0  92.0 110.0 128.0 146.0 164.0 182.0 MeV

E/A =  0.83  1.58  2.33  3.08  3.83  4.58  5.33  6.08  6.83  7.58

Q0 =   7.3   8.8   9.6  10.1  10.5  10.8  11.0  11.1  11.2  11.4

RHO =  0.93  0.89  0.84  0.80  0.76  0.72  0.69  0.66  0.64  0.61

EPS =  0.04 -0.01 -0.04 -0.07 -0.09 -0.10 -0.12 -0.13 -0.14 -0.14

QAVG=   7.3   8.7   9.5  10.0  10.4  10.6  10.8  11.0  11.1  11.2

Q = 4  0.03

5  1.57

6 15.95  0.39  0.02

7 40.95  6.46  0.71  0.12  0.03

8 31.82 31.32  9.14  2.51  0.80  0.30  0.13  0.06  0.03  0.02

9  8.69 43.38 37.67 20.51 10.22  5.21  2.80  1.58  0.93  0.57

10  0.94 16.67 41.85 49.58 43.36 33.64 25.03 18.44 13.65 10.20

11  0.05  1.72 10.19 25.52 40.95 51.75 57.26 58.76 57.72 55.27

12        0.05  0.42  1.77  4.62  9.05 14.63 20.87 27.29 33.60

QIT$