RPD$075066001

CHP$U310  LEMO (VMS & ULTRIX version) - WTM

RSH$0505

TPF$

SP1$

CEB$

LEMO (List-tape Examine, Modify, Output)

DBL$

CY1$

SECTION  CONTENTS

010  INTRODUCTION

020  LIST OF COMMANDS

030  DISCUSSION OF CERTAIN COMMANDS

040  DISCUSSION OF COMMANDS - RELATED TO MODIFY-COPY (PRESCAN)

050  MULTI-TAPE PROCESSING - MOCO - CONCURRENT ONLY

060  HOW TO EXAMINE A TAPE

070  HOW TO RESTORE EVENT HANDLER PROGRAMS TO DISK

080  COPYING ASCII FILES TO AND FROM TAPE

090  HOW TO CONVERT A NON-HHIRF TAPE TO HHIRF STANDARD (L002)

100  BYTE-SWAPPING

110  PRESCAN (MODIFY - COPY)

120  CHIL PROGRAMMING FOR PRESCAN - EXAMPLE

130  PRESCAN IN BRIEF

140  HOW TO CREATE A CUSTOMIZED PRESCAN PROGRAM

150  OPERATIONS WHICH CAN MODIFY THE DATA STREAM

155  BUFFERED TAPE COPY OPERATIONS

160  USER PROCESSING OF RAW      EVENTS  - VIA REBUF

170  USER PROCESSING OF RAW DATA BUFFERS - VIA USERMOC

180  COPYING EVENT-DATA FILES TO TAPE AND ADDING HEADERS

190  LOGICAL UNITS AND COMMON BLOCKS

SIN$

CYB$

---------------------------------------------------------------------------

Type:  LEMO               ;To start on CONCURRENT

Type:  @U1:[MILNER]LEMO   ;To start on VAX with no     LOGIN.COM entry

Type:  LEMO               ;To start on VAX with        LOGIN.COM entry

Type:  /usr/hhirf/lemo    ;To start on DECstation with no .login entry

Type:  lemo               ;To start on DECstation with    .login entry

Type:  HELP               ;For directory to commands

---------------------------------------------------------------------------

TPF$

SP2$

SEC$.010  INTRODUCTION

BLP$

LEMO is a tape processing utility program which can be customized

(via USERSUB, USERCMP and other support routines) to produce a

prescan program. The stock version of LEMO can do a number of

useful things: Some of these are:

ODP$

(1)....Any tape containing records

no longer than 32768 bytes, may be examined (records read and

displayed) and/or copied.

ODP$

(2)....Input data records may be byte-swapped for VAX compatibility.

ODP$

(3)....List data tapes recorded in HHIRF L002-format may be examined

by reading records and displaying or printing portions thereof

in "event format" (see SEC# U310.030).

ODP$

(4)....The text records from L002-format tapes (which normally contain

the Event Handler program) can be restored to disk.

It is then ready for assembly - if it ever was.

ODP$

(5)....A limited amount of prescan selection can be carried out with

stock LEMO (see SEC# U310.110 for discussion & U310.120 for an example).

ODP$

(6)....Certain non-HHIRF tapes may be converted to HHIRF L002 format

(see SEC# U310.090)

ODP$

(7)....The usual tape control functions (forward and backward spacing

of records and files, rewind etc.) are provided.

ODP$

(8)....Up to three output data streams can be produced in the MODIFY

COPY mode.

CY1$

---------------------------------------------------------------------------

See SEC# U350 - CHIL, for information on CHIL programming, USERSUBS

and user COMMAND PROCESSORS for customized prescan tasks.

---------------------------------------------------------------------------

See SEC# U310.150 through SEC# U310.170 for a discussion of other non-CHIL

user-supplied customizing routines, REBUF and USERMOC.

---------------------------------------------------------------------------

TPF$

SP1$

SEC$.020  LIST OF LEMO COMMANDS

CY1$

\bu\Commands for Assigning Input and Output Tapes\\

IN  MXXX:    - Specifies tape (MXXX:) for INPUT

OUX MYYY:    - Specifies tape (MYYY:) for OUTPUT-X (X=1,3)

MXXX:,MYYY: Denote MTL1:,MSA0:,MUB0:,rmt0:,rmt1:, etc

\bu\Commands for Tape Control Operations\\

RDI  N       - Read    N records from INPUT

RDOX N       - Read    N records from OUTPUT-X  (X=1,3)

FRI  N       - Forward N records on   INPUT

FROX N       - Forward N records on   OUTPUT-X  (X=1,3)

BRI  N       - Backup  N records on   INPUT

BROX N       - Backup  N records on   OUTPUT-X  (X=1,3)

FFI  N       - Forward N files   on   INPUT

FFOX N       - Forward N files   on   OUTPUT-X  (X=1,3)

BFI  N       - Backup  N files   on   INPUT

BFOX N       - Backup  N files   on   OUTPUT-X  (X=1,3)

RWI          - Rewind INPUT

RWOX         - Rewind                 OUTPUT-X  (X=1,3)

BTI          - Go to BOTTOM of INPUT  (to DBL EOF, Backup 1 F)

BTOX         - Go to BOTTOM of OUTPUT (to DBL EOF, Backup 1 F)

CLI          - Close INPUT

CLOX         - Close                  OUTPUT-X  (X=1,3)

ULI          - Unload and Close INPUT tape

ULOX         - Unload and Close       OUTPUT-X  (X=1,3)

\bu\Commands for Data Display\\

PEV  IA,IB   - Print 16-bit word IA thru IB in EVENT   Format

DEV  IA,IB   - Disp  16-bit word IA thru IB in EVENT   Format

PZ   IA,IB   - Print 16-bit word IA thru IB in HEX     Format

DZ   IA,IB   - Disp  16-bit word IA thru IB in HEX     Format

PA   IA,IB   - Print 16-bit word IA thru IB in ASCII   Format

DA   IA,IB   - Disp  16-bit word IA thru IB in ASCII   Format

PI   IA,IB   - Print 16-bit word IA thru IB in INTEGER Format

DI   IA,IB   - Disp  16-bit word IA thru IB in INTEGER Format

PIF  IA,IB   - Print 32-bit word IA thru IB in INTEGER Format

DIF  IA,IB   - Disp  32-bit word IA thru IB in INTEGER Format

\bu\Commands for Finding and Displaying Headers (Titles)\\

FIND N       - Find HEADER # N (does 1 BACKSPACE if found)

DTIT         - Displays next TITLE & HEADER # & BACKSPACES

\bu\Commands for Interrupting the Process\\

SEND STOP    - Interrupts READ or COPY process (CONCURRENT)

Ctrl/C       - Interrupts READ or COPY process (VAX)

END          - Terminates program

CEN$

(continued on next page)

TPF$

SP1$

SEC$.020  LIST OF LEMO COMMANDS (continued)

CY1$

\bu\Commands Related to Command-file Operations\\

CMDF FIL.CMD - Assign FIL.CMD as CMD-file (not read yet)

CCMD         - Continue reading  CMDS from CMD-file

CLCM         - Continue with last CMD from file (backspaces)

CCON         - Continue reading  CMDS from VDT (Terminal)

MSG  TEXT    - Display TEXT (44 bytes) on VDT

\bu\Commands Related to Modify_Copy\\

MILF FIL.MIL - Read & process MIL-file (required for MOC)

UCOM TEXT    - Send TEXT to USERCMP

RECO LBYT    - Output-Recl-Bytes (DFLT=8192)(MOC mode only)

SWAB         - Request    byte-swap of input buffers (SEC# U310.100)

SWOF         - Request no byte-swap (default)

SHON         - Says byte-swap headers once more than data

SHOF         - Says byte-swap headers & data the same way (default)

MOC  N,M     - MODIFY-COPY (N-files/M-recs - 1st to occur)

MOCO N,M     - Same as MOC but waits on next INPUT or OUTPUT tapes

- (Not supported in VAX version)

MOCE N,M     - MODIFY-COPY (END on request complete)

INIT         - Resets MODIFY-COPY INPUT & OUTPUT buffers

ZBUC         - Zero total INPUT & OUTPUT buffer counters

\bu\Commands Related to Simple Copy Operations\\

COPY N       - Copy N files   from INPUT to OUTPUT-1

CREC N       - Copy N records from INPUT to OUTPUT-1

CC           - Continue COPY - saves file- or record-count

EOF          - Write EOF on OUTPUT-1 (not normally needed)

FCOP FIL.EXT - Copy FIL.EXT to OUTPUT-TAPE-1

TCOP FIL.EXT - Copy 1 file from IN-TAPE to FIL.EXT (new file created)

(To be used for 80 byte ASCII files only)

\bu\Commands Related to Event-File-to-Tape Copy & Header Creation\\

INFI  filname   - Specify input file for exam (RDI) & copy-to-tape

HTIT  TITLE     - TITLE contains title for next tape header

HNUM  HN        - HN specifies next tape header number to use

HOUT            - Outputs tape header and increments HN

SHON            - Says byte-swap headers once more than data

SHOF            - Says byte-swap headers & data the same way

CEN$

(continued on next page)

TPF$

SP1$

SEC$.020  LIST OF LEMO COMMANDS (continued)

CY1$

\bu\Miscellaneous and Non-standard Commands\\

STEX FIL.EXT    - Store text blks on FIL.EXT

ASLU LU,FIL.EXT - Asn LU to FIL.EXT (LU=0,1,2,3,4) (CONCURRENT only)

CLOU LU         - Close LU                         (CONCURRENT only)

L001 NSKIP,NPPE - Specify non-HHIRF    input tape (see SEC# U310.090)

L002            - Specify HHIRF-format input tape (default)

\bu\Commands Related to User Processing of Raw Buffers & Events\\

UPON NPRAW,RECL - Turn User-processing ON (see SEC# U310.170)

- NPRAW = Max # of raw parameters (for user only)

- RECL  = Input data record length in bytes

UPON            - Turn User-processing ON (NPRAW=0, RECL=8192 bytes)

UPON NPARU      - Turn User-processing ON (         RECL=8192 bytes)

UPOF            - Turn User-processing OFF (default)

RBON            - Enable  REBUF calls (see SEC# U310.160)

RBOF            - Disable REBUF calls (default)

TPF$

SP1$

SEC$.030  DISCUSSION OF CERTAIN COMMANDS

RND$09

ODP$

FIND.....N attempts to find HEADER # N by searching forward on the

INPUT tape (TITLES and HEADER numbers are displayed along the way):

If found, LEMO backs up one record.

If  not found, LEMO will read past the first

Double-EOF, and back up one File Mark.

ODP$

BTOX.....Advances OUTPUT-X (X=1,3) past the first Double-EOF and backs

up one File Mark (i.e. positions properly for appending).

Header titles and numbers are displayed along the way.

ODP$

BTI......Does the same thing for the INPUT tape.

ODP$

DEV......PEV, DA, PA, DZ, PZ, DI, PI, DIF & PIF

are all commands which display on the terminal

or list on the printer some portion of the last record read from

tape (either the INPUT or OUTPUT). When you do a read (RIN or ROU),

LEMO tells you how many bytes were read but you must specify the

portion of the buffer to be displayed in half-words (16-bit words).

FIP$

In EVENT FORMAT (DEV or PEV), LEMO looks for a hex FFFF before

starting to accumulate the first EVENT to be displayed.

Therefore, the first EVENT in any record is usually not displayed

by DEV or PEV (you can see it via DZ or PZ, however).

ODP$

COPY.....N says COPY N-files from INPUT to OUTPUT. All that is

required is that records be no longer than 32768 bytes. File-marks

are copied and the OUTPUT tape is always positioned between Double

File-Marks on normal completion of a COPY request.

The OUTPUT tape is positioned ahead of Double File-Marks on

abnormal completion (via SEND STOP, input error, etc) of a COPY

request. The COPY is terminated (normally) if a Double-EOF or an

End-Of-Medium is encountered on the INPUT.

ODP$

CREC.....N says copy N-records from INPUT to OUTPUT. The same rules

apply as for COPY except that the OUTPUT tape is always left

positioned ahead of a Double File-Mark.

ODP$

CC.......Says continue previous COPY or CREC. It remembers how

many files or records have already been copied and continues the

count.

TPF$

SP1$

SEC$.040  DISCUSSION OF COMMANDS - RELATED TO MODIFY-COPY (PRESCAN)

ODP$

INIT.....Resets both the EVENT- and OUTPUT-buffers. Do this if you

have been doing some tests but are now ready to prescan for

real or any time you wish to make a clean start.

ODP$

MOC......N,M Starts the MODIFY-COPY process, where: N is the number

of files to process and M is the number of records to process.

The processing terminates on either N or M - the first to be

satisfied. The default values of N and M are 1 and 100,000,000

respectvely.

ODP$

MOCE.....N,M Differs from the MOC-command described above only in that

upon completion of the request, both INPUT and OUTPUT tapes are

unloaded and the program is terminated.

FIP$

The following table summarizes the different ways in which processing

may be terminated and the state of the OUTPUT-tape, OUTPUT-buffer

and EVENT-buffer for each:

CY1$

TERMINATION BY---   RESULTS IN -----------------------------------

Requested # Files   Flush OUT-BUF, 2-EOF, 1-BKFIL, Scrub-EV

EOM on INPUT        Flush OUT-BUF, 2-EOF, 1-BKFIL, Scrub-EV

Requested # Recs    Flush OUT-BUF, 2-EOF, 2-BKFIL,  Save-EV

SEND STOP                          2-EOF, 2-BKFIL,  Save-EV

Input Error         Flush OUT-BUF, 2-EOF, 2-BKFIL, Scrub-EV

MOCE                Flush OUT-BUF, 2-EOF, Unload Tapes, EXIT

FIP$

Where, Scrub-EV indicates that any partial-event which has not

been fully processed will be deleted from the EVENT-buffer and

Save-EV means that any partial-event will be retained for

subsequent MOC requests.

FIP$

NOTE: Processing is always terminated with two File-Marks being

written on all OUTPUT tapes.

RND$07

TPF$

SP1$

SEC$.050  MULTI-TAPE PROCESSING - MOCO - CONCURRENT ONLY

BLP$

The MOCO command is intended to make it easier for

a novice (or at least someone not familiar with your process) to

change tapes for you.

MOCO means MODIFY-COPY & CONTINUE (waits for input & output

tapes to be mounted).

CEB$

THIS IS HOW IT WORKS

BLP$

When intervention is required, by the requested number of files being

read from the input (or DBL-EOF or EOM encountered) or EOM detected

on the output, the program will:

CY1$

(1) Unload the tape which must be replaced (TELEX drives only),

(2) Type: WAITING FOR INPUT (or OUTPUT) TAPE,

(3) Try to read/write input/output tapes (trying every 2 sec),

(4) and flash the light on top of the drive.

BLP$

As soon as another tape is mounted and made ready, the program will:

CY1$

(1) Read/write one record from/on the new tape,

(2) Rewind the new tape,

(3) Turn off the flasher,

(4) and read the next command - normally from a command file.

BLP$

If the next command read from the command file is another MOCO,

MOCE, etc, it will process the tape without anything being typed

on the VDT. If the command file contains a long list of MOCO's,

the process will continue until you run out of tapes or the tape

drive screws up.

BLP$

A reasonable command file might look like:

CY1$

MOCO 100       ;Says process tape to end & wait for next

MOCO 100

-

-

MOCE 100       ;Process last tape and end

---------------------------------------------------------------------------

BLP$

To interrupt a tape-wait (light blinking) state and read the next

command from the VDT, type:

CY1$

SEND STOP

CEN$

(continued on next page)

TPF$

SP1$

SEC$.050  MULTI-TAPE PROCESSING - MOCO - CONCURRENT ONLY (continued)

CEB$

SUGGESTIONS

ODP$

(1)....Set up a command file to process a certain number of tapes

and then end gracefully.

ODP$

(2)....Turn the FLIP-SIGN on top of the tape drive to the card which

says .... NEXT TAPE WILL BE PROCESSED IN 10 SECONDS ....

ODP$

(3)....Fill out and attach one of the FLIP-SIGN labels provided.

ODP$

(4)....Carefully stack tapes to be processed in front of the

appropriate tape drive in the space labeled TAPES TO DO.

ODP$

(5)....Finished tapes should be stacked in the TAPES DONE space.

ODP$

(5)....Start processing with LEMO.

CYB$

When a flasher is observed to be active (blinking), do the following:

ODP$

(1)....Remove the processed tape and place it on the TAPES DONE stack.

ODP$

(2)....Load next tape from the TAPES TO DO stack and set it ON LINE.

CEB$

WHAT IF'S

ODP$

If....the drive drops ready on rewind but the light is blinking, things

should proceed normally - once you get the old tape unloaded & the

next tape mounted and on line.

ODP$

If....the tape runs off the end but the light is blinking,

things should proceed normally when you finally get the next tape

loaded.

ODP$

If....the drive becomes unavailable (for any reason) and the light

is NOT blinking, VDT intervention will be required.

CEB$

!!! CAUTIONS !!!

BLP$

Don't try to use a BLINKING drive for anything except to process the

next tape from the TAPES TO DO stack.

BLP$

If you have to unload the tape manually (because of drive

malfunction, etc), be carefull NOT to put the old tape ON LINE

because the program may start to process it AGAIN! before you

have time to press the RESET button.

TPF$

SP1$

SEC$.060  HOW TO EXAMINE A TAPE

CY1$

(1) Place the tape of interest on some tape drive.

(2) Type: LEMO

(3) Type: HELP

You can probably figure out what to do.

SEC$.070  HOW TO RESTORE EVENT HANDLER PROGRAMS TO DISK

BLP$

Position the INPUT tape such that the Header containing the program of

interest is the next one to be encountered (via FIND for example).

If the desired program is contained in the first

Header on the tape, type:

CY1$

RWI                        ;Rewinds the INPUT tape

STEX FILNAM.EVS            ;The extension .EVS is required by ADAC

BLP$

Where, FILNAM.EVS is the file on which the program is to be stored.

FILNAM.EVS must not already exist.

If you wish to restore the program contained in Header # N, Type:

CY1$

FIND N                     ;Finds Header # N

When (and if) the requested Header is found, Type:

STEX FILNAM.EVS

SEC$.080  COPYING ASCII FILES TO AND FROM TAPE

BLP$

LEMO may be used to write and read ASCII tapes for transfer to and

from other computers. The following commands are used:

CY1$

FCOP FILENAME  ;Copies FILENAME to OUTPUT-TAPE-1 (previously opened)

;Variable length records from FILNAME are de-tabbed and

;written as fixed length (80 byte) records on tape.

;(DEC's rules for FORTRAN tabs are used in de-tabbing)

TCOP FILENAME  ;Copies 1 file from INPUT-TAPE to FILENAME (created)

;Fixed length (80 byte) records from tape are

;written as variable length records on FILENAME.

TPF$

SP1$

SEC$.090  HOW TO CONVERT A NON-HHIRF TAPE TO HHIRF STANDARD (L002)

BLP$

Certain non-standard

tapes may be converted to L002 format under the MODIFY -

COPY mode if the following conditions are met:

ODP$

(1)....The length of event-by-event data records to be processed

must be different from all other records which will be encountered.

ODP$

(2)....All data records must be written in 16-bit mode.

ODP$

(3)....If the high-order bit of a data word in the input stream is set

it will be masked off (lost) in the converted data stream.

ODP$

(4)....Each data record must contain a fixed (integer) number of

events. i.e. events may not be split across record boundries.

ODP$

(5)....Each event must contain a fixed number of parameters.

ODP$

(6)....Bytes may be swapped if requested.

ODP$

(7)....A specified number of data words (record header words etc)

may be skipped at the beginning of each data record.

BLP$

Request processing of a non-standard (L001) format by typing:

CY1$

L001 NSKIP,NPPE

BLP$

Where, NSKIP denotes the number of record header words to skip before

reading events and NPPE the number of parameters per event.

If bytes are to be swapped and/or an output record length other than

8192 bytes is desired, use the following commands:

CY1$

SWAB          ;Requests byte-swap of input records

RECO  NBYTS   ;Specifies that the output record length is to be NBYTS

BLP$

A simple CHIL program of the following form will be required:

CY1$

$LSTL = RECL               ;Specify tape record length (bytes)

$NPR  = NPAR               ;Specify # parameters per EVENT (NPAR)

;Must include any calculated parameters

CALL USERSUB1        ;Use only if parameters are to be modified

;or created

CALL REPACK1 1,NPAR  ;Request that all parameters be saved

(See SEC# U310.130 for additional information on MODIFY-COPY operation)

TPF$

sp1$

SEC$.100  BYTE-SWAPPING

BLP$

The command SWAB causes the following byte-swapping actions:

ODP$

(1)....For L002 input, swaps bytes for all input records

(appropriately) in both COPY and MOC modes.

ODP$

(2)....For L001 input, swaps bytes of all input records (assuming

16-bit integers) in the COPY mode.

ODP$

(3)....For L001 input, swaps bytes of data records only in MOC mode.

BLP$

Under some circumstances, you may need to byte-swap the data and

not the headers or vice versa. In such cases, the command SHON turns

on an additional byte-swap for the header only. SHOF turns it off

and is the default.

SEC$.110  PRESCAN (MODIFY - COPY)

BLP$

Pre-Scanning (as interpreted here) involves the processing of an

INPUT data stream (from Mag Tape) to produce an OUTPUT data

stream (to Mag Tape). The processing may include

selection of certain events, selection of certain parameters,

modification of input parameters or creation of new parameters or

any combination of the preceding.

BLP$

A CHIL-based prescan task involves the use of LEMO (or a

customized version thereof) to control the process combined with a

CHIL program which aides in the selection of events and/or

parameters. Modification or creation of parameters will normally

require one or more user-supplied subroutines USERSUBS.

BLP$

The CHIL-based prescan program may include any legal CHIL statement

except for the H-statement (i.e. concurrent histogramming is not

supported). The CALL REPACK1 (not legal in histogramming programs)

is used to initiate the "saving of an event" into the output data

stream.

BLP$

In the MODIFY-COPY mode, records are read from the input tape,

events are expanded and passed (one at a time) to the USERSUBS.

BLP$

Input records which are not of the length specified in the CHIL

program ($LSTL-Statement) are simply copied to all output tapes,

thus, Headers are copied automatically.

If the record length and/or the Max # of Parms for the output tape

is specified differently from the input, the primary Header is

modified appropriately.

BLP$

If an End-of-Medium is encountered on an output tape and the

recording is continued on a new tape, only the primary header

(not the text blocks containing the Event Handler program etc) will

be reproduced on the continuation tape.

CEB$

See following page for CHIL program example

TPF$

SP2$

SEC$.120  CHIL PROGRAMMING FOR PRESCAN - EXAMPLE

BLP$

Prescan program which selects only those EVENTS for which parameters

18 and 16 satisfy Free-Form-Gate number-1 from file DEC2.BAN.

If gate is satisfied, all parameters of event are saved.

CY1$

$LSTL        = 8192        ;Specify tape record length in bytes

$NPR 18                    ;Specify number of parameters

$LPR 1 TO  2 = 64          ;Specify length of parameters 1 & 2

$LPR 3 TO 18 = 2048        ;Specify length of parameters 3 to 18

$BAN (256) 1               ;Request Free-Form-Gate from BAN-file

;(X-length = 256, ID-number = 1)

$BAF DCE2.BAN              ;Give name of BAN-file

IFU(B(18,16 1)) 100  ;Test X,Y-parameters (18,16) against gate

;Skip it if gate not satisfied

CALL REPACK1 1,18    ;otherwise, save EVENT in output stream

; 1 is lowest  Parameter-# to save

;18 is highest Parameter-# to save

100 CONTINUE

TPF$

SP2$

SEC$.130  PRESCAN IN BRIEF

DBL$

CY1$

Create your CHIL program and save in file PRESCAN.CHL, for example.

Type:  CHIL PRESCAN

Type:  DEL PRESCAN.DIR               ;For CONCURRENT

Type:  DEL PRESCAN.DRR;*             ;For VAX

Don't create a HIS-file

Type:  LEMO

Type:  IN  MXXX:                     ;MXXX: denotes Input  Tape Unit

Type:  OU1 MYYY:                     ;MYYY: denotes Output Tape Unit

Type:  MILF PRESCAN.MIL              ;Reads & processes MIL-file

Type:  MOC                           ;Starts processing

SIN$

CEN$

End-of-File or End-of-Medium on Input Tape

BLP$

When an EOF or EOM is encountered on the input tape, LEMO writes

any partially filled output buffer and two File Marks onto all

output tapes and backs up one File Mark.

To continue with the next input file, type the command MOC.

To continue with the next input tape, type RWI, mount next input

tape and type MOC.

CEN$

End-of-Medium on Output Tape

BLP$

When an EOM is encountered on the output tape, LEMO backs up one

record, writes a File Mark, rewinds the output, instructs you to

mount a new tape, and pauses.

When you type: CONTINUE, LEMO writes the primary header (not the text

blocks) and the last output record (the one it was writing when it

hit EOM) onto the new tape, and continues processing.

CYB$

See SEC# U310.040 for more information on termination of processing.

TPF$

SP1$

SEC$.140  HOW TO CREATE A CUSTOMIZED PRESCAN PROGRAM

CY1$

\bu\For CONCURRENT System\\

ODP$

(1)....Use the editor to create the desired USERSUB, USERCMP and

other required routines and save all such routines on one file

(MYRUTS.FTN for example).

ODP$

(2)....Compile these routines to obtain an object file (MYRUTS.OBJ for

example) by typing:

CY1$

F7D MYRUTS    ;For a non-optimize compilation while de-bugging

or

F7O MYRUTS    ;For an optimized compilation after de-bugging

ODP$

(3)....Link your routines with the "main package" by typing:

CY1$

LEMOLNK  MYRUTS,MYLEMO   ;To produce a customized task

BLP$

Where, MYLEMO denotes any legal filename prefix (up to 8 characters)

for the task file to be produced.

The following CSS procedure loads and starts MYLEMO.

CY1$

L MYLEMO

AS 5,CON: ; AS 6,PR: ; AS 9,CON:

AS 8,LEMO.HEP/S

ST

$EXIT

CY1$

\bu\For the VAX\\

ODP$

(1)....Use the editor to create the desired USERSUBS, USERCMP and

any other required routines and save all such routines on one or more

files (F1.FOR, F2.FOR .. for example).

ODP$

(2)....Compile these routines to obtain associated object files

by typing:

CY1$

FOR F1                         ;To compile F1

FOR F2                         ;To compile F2 etc.

ODP$

(3)....LINK your routines with the MAIN PACKAGE by typing:

CY1$

LEMOLNK  "F1,F2,.." MYLEMO     ;To produce EXE-file MYLEMO

ODP$

(4)....Copy the COM-file LEMO.COM

to MYLEMO.COM and replace the line:

CY1$

RUN LEMO  with  RUN MYLEMO

CEN$

(continued on next page)

TPF$

SP1$

SEC$.140  HOW TO CREATE A CUSTOMIZED PRESCAN PROGRAM (continued)

CY1$

\bu\For the DECstations\\

BLP$

Use the make file \b\/usr/hhirf/lemo.make\\

(listed below) as a template for creating

your own make file for generating a customized \b\prescan\\ program.

CY1$

DIRA= /usr/hhirf/

DIRB= /usr/users/\b\milner/Dlemo/\\

OBJS= $(DIRA)lemo.o $(DIRB)\b\dummysubs.o\\

LIBS= $(DIRA)lemolib.a $(DIRA)milib.a $(DIRA)jblibf1.a $(DIRA)jblibc1.a

\b\lemo:\\ $(OBJS) $(LIBS)

f77 -O2 $(OBJS) $(LIBS) -o \b\lemo\\

BLP$

Assume that your username is \b\userdoe\\ and your customizing routines

are in your directory \b\Dlemosubs\\ and there are two files to be

included, \b\subsa\\ and \b\subsb\\ and you wish to produce a program

\b\mylemo\\. Copy the template file into your directory and

create a make file \b\mylemo.make\\ by changing the template

file as/where indicated by the \b\bold\\ face type. The generic result

is shown below.

CY1$

DIRA= /usr/hhirf/

DIRB= /usr/users/\b\userdoe/Dlemosubs/\\

OBJS= $(DIRA)lemo.o $(DIRB)\b\subsa.o\\ $(DIRB)\b\subsb.o\\

LIBS= $(DIRA)lemolib.a $(DIRA)milib.a $(DIRA)jblibf1.a $(DIRA)jblibc1.a

\b\mylemo:\\ $(OBJS) $(LIBS)

f77 -O2 $(OBJS) $(LIBS) -o \b\mylemo\\

TPF$

SP2$

SEC$.150  OPERATIONS WHICH CAN MODIFY THE DATA STREAM

BLP$

When running in the modify-copy (MOC) mode,

one or more of the following operations may be initiated which

can result in the modification of the data stream. Operations

are listed in the order that they will be carried out if requested.

The column headed CMD gives the run-time LEMO command which requests

the associated operation.

CY1$

CMD    OPERATION-----------------------------------------------------------

ODP$

SWAB...Swap bytes in data buffers (also HHIRF headers) (see SEC# U310.100).

ODP$

L001...Convert data buffers from L001 to L002 format (see SEC# U310.090).

ODP$

RBON...Call user-supplied routine BUFMAN event-by-event (see SEC# U310.160).

ODP$

UPOF...Process & output buffers via CHIL, USERSUBs (see SEC# U310.120).

This is the default.

ODP$

UPON...Process buffers via user-supplied routine USERMOC (see SEC# U310.170).

After the new output buffer is constructed, USERMOC must do one

of the following:

FIP$

(1)...Output buffers directly by calling routine UPVALL or:

FIP$

(2)...Process and output buffers via CHIL, USERSUBs, etc. by

calling routine PVALL.

FIP$

\b\Note:\\ A CHIL program is not required unless PVALL is called by

USERMOC. Thus, in the UPON, UPVALL mode, the user can do anything he

feels like with the data. Output buffers do not have to be L002 or

anything in particular.

FIP$

\b\USERMOC is not available in the Concurrent version.\\

SP3$

SEC$.155  BUFFERED TAPE COPY OPERATIONS

BLP$

The simple tape copy executed via the \b\COPY or CREC\\

commands is not buffered (i.e. input and output does not proceed

simultaneously). However, you can use the \b\MOC\\ command

with \b\UPON\\ and the default USERMOC routine (which just copies

buffers) to achieve a double buffered copy operation. This should

run at approximately twice the speed of the simple \b\COPY\\.

TPF$

SEC$.160  USER PROCESSING OF RAW EVENTS - VIA REBUF

BLP$

LEMO provides (via a user-supplied routine REBUF)

the user with the means to access and modify the raw

event-data prior to any subsequent CHIL or user-processing

(USERMOC) action.

If REBUF is enabled, the following operations occur.

ODP$

(1)....Buffers (records) are read from the input tape.

ODP$

(2)....Input buffers are scanned for events (number sequences

ending in hex-FFFF) and one raw event per call is passed

to REBUF. Events which are split across records are taken

care of.

ODP$

(3)....REBUF produces a new output data stream by storing new

packed events (including the FFFF) into an INTEGER*2 array NULIST.

ODP$

(4)....The new data stream (in NULIST) is then passed to

the usual unpacking and CHIL processing, including USERSUB calls etc.

BLP$

CHIL only sees the data and number of parameters in NULIST -

it doesn't know that there was a REBUF operation.

$LSTL (in the CHIL program) is used by the tape reading routine and

is unchanged.

CYB$

Type:  RBON      ;At run time to  enable REBUF calls

Type:  RBOF      ;At run time to disable REBUF calls (default)

CY1$

C     ---------------------------------------------------------------------

C     A do-nothing REBUF routine which just copies PBUF to NULIST

C

C     PBUF   - contains raw packed event (the FFFF is not included)

C     NP     = number of words in PBUF

C     NULIST - is the new output buffer which you load

C     NN     = number of words loaded into output buffer

C              LEMO resets NN to zero whenever a buffer is processed

C     ---------------------------------------------------------------------

SUBROUTINE REBUF(PBUF,NP,NULIST,NN)

INTEGER*2 NULIST(16384),PBUF(2000)

DO 10 I=1,NP

NN=NN+1

NULIST(NN)=PBUF(I)

10 CONTINUE

NN=NN+1

NULIST(NN)=X'FFFF'        !You must supply the end-of-event FFFF

RETURN

END

CEB$

Important Comment

BLP$

If you think that you may significantly

increase the abount of data (in going from input to output),

I may need to know about it.

I have shown the dimensions of PBUF and NULIST to be that

defined in the main program. NULIST is dimensioned to 4 times

the usual input record size of 8192 bytes.

LEMO does not check the fullness of NULIST on an event-by-event basis

but processes NULIST after each input buffer is massaged into it.

As things stand, you would be able to expand the input data by almost

a factor of 4 (for 8192 byte input records) - beyond that, you blow

the program!!

TPF$

SP1$

SEC$.170  USER PROCESSING OF RAW DATA BUFFERS - VIA USERMOC

BLP$

VAX and DECstation versions of LEMO provide users with the means

(via a user-supplied routine USERMOC) to do the following:

ODP$

(1)....Process raw input buffers into new buffers and subsequently

call UPVALL which can generates (1 to 3) output data streams

to tape with no CHIL involvment.

ODP$

(2)....Process raw input buffers into new buffers and subsequently

call the standard CHIL processor, PVALL, which then imposes any CHIL

conditions and generates output data streams to tape.

BLP$

At run time, calls to USERMOC are enabled/disabled as follows:

CY1$

UPON             ;Enable calling of USERMOC (NPRAW=0, RECL=8192 bytes)

UPON NPRAW       ;Enable calling of USERMOC (         RECL=8192 bytes)

UPON NPRAW,RECL  ;Enable calling of USERMOC

;NPRAW = Max # raw parms (for USERMOC if needed)

;RECL  = Input record length (bytes) if not 8192

UPOF             ;Disables USERMOC calls (enables PVALL calls - default)

BLP$

The following USERMOC illustrations do nothing except to

copy the input buffer INBUF into

an output buffer OUBUF with no modification. The first illustration calls

UPVALL to output into stream-1 while second calls PVALL for

subsequent CHIL processing and output.

CY1$

---------------------------------------------------------------------------

SUBROUTINE USERMOC(INBUF,NWDS)   ;\bu\Illustration-1\\

C                                      ;\b\default USERMOC\\

INTEGER*2 INBUF(*),OUBUF(16384)  ;NWDS=#words in INBUF

C

DO 10 I=1,NWDS                   ;Loop on # words in input buffer

OUBUF(I)=INBUF(I)                ;Set output = input

10 CONTINUE

ISTREAM=1                        ;Specify output stream-1

C

CALL UPVALL(OUBUF,NWDS,ISTREAM)  ;Call UPVALL to handle output

C                                      ;NWDS = # of words in OUBUF

RETURN                           ;and return

END

---------------------------------------------------------------------------

SUBROUTINE USERMOC(INBUF,NWDS)   ;\bu\Illustration-2\\

C                                      ;NWDS = # of words in INBUF

INTEGER*2 INBUF(*),OUBUF(16384)  ;Buffer dimensions

C

DO 10 I=1,NWDS                   ;Loop on # words in input buffer

OUBUF(I)=INBUF(I)                ;Set output = input

10 CONTINUE

C

CALL PVALL(OUBUF,NWDS)           ;Call PVALL for CHIL processing &

C                                      ;output (CHIL specifies stream-#)

RETURN                           ;NWDS = # of words in OUBUF

END

TPF$

SP1$

SEC$.180  COPYING EVENT-DATA FILES TO TAPE AND ADDING HEADERS

BLP$

The DECstation version of \b\lemo\\

accomodates the examining and copying of disk-files containing

event-list data. This feature is intended to be an aid to those

doing simulations on the DECstations. The following commands are

available:

CY1$

INFI  filname   - Specify input file for exam (RDI, DEV) & copy-to-tape

HTIT  TITLE     - TITLE contains title for next tape header

HNUM  HN        - HN specifies next tape header number to use

HOUT            - Outputs tape header and increments HN

BLP$

In addition, an example program found in

\b\/usr/users/milner/Develx/evelx.f\\

contains routines for opening an event-file (EVELOPEN) and for

writing data to it (EVELOUT). The "simulator" might (or might not)

wish to include these routines in his simulation source code. Feel

free to copy \b\evelx.f\\ to your directory for examination etc.

The routines are internally documented.

BLP$

The idea is this:

ODP$

(1)....You do your simulations and write the generated events

to a disk file.

ODP$

(2)....You may then use \b\lemo\\ to examine this file (read,

display in event-format, etc).

ODP$

(2)....You may also specify a tape header number and title,

output the header to tape and finally copy the entire file to tape

via the normal \b\copy\\ command.

BLP$

An typical file-to-tape copy session might look as follows:

CY1$

lemo>\b\infi  eventfile.dat           \\;open event-file for input

lemo>\b\ou    rmt1:                   \\;open rmt1: for output

lemo>\b\htit  simulation-3 no-gates   \\;title for tape header

lemo>\b\hnum  3                       \\;next header number on tape

lemo>\b\hout                          \\;output the header to tape

lemo>\b\copy  1                       \\;copy 1-file input-to-output

lemo>\b\end                           \\;end program

BLP$

\b\NOTE:\\ You can also use the header setup and output feature (hnum,

htit & hout commands) to add headers while copying one tape to another.

There is no provision, however, to delete or modify existing headers.

SEC$.190  LOGICAL UNITS AND COMMON BLOCKS

CY1$

CHIL-based PRESCAN programs may use - LOGICAL UNITS 5 thru 14

User supplied routines should NOT attempt to use these LOGICAL UNITS.

All CHIL-based tasks may use COMMON BLOCK labels /AAA/ through /ZZZ/

LEMO also uses /INNARDS/ and /ULOCA/

User supplied routines should not use these COMMON BLOCK labels .

QIT$