RPD$070066001

RSH$0707

CHP$380.  ASAP - WTM

TPF$

CEB$

AUTOMATIC SPECTRUM ANALYSIS PROGRAM

DBL$

CY1$

SECTION  CONTENTS

010  HOW TO GET STARTED

020  GENERAL FEATURES

030  PROGRAM OPERATION

040  SYNTAX

050  FIT SPECIFICATIONS - COMMAND LIST

060  INPUT QUANTITIES - DEFINITIONS

070  BACKGROUND COMPUTATION

080  PEAK FINDING

090  SUM AREA (SAREA) CALCULATION

100  CENTROID CALCULATION

110  GAUSSIAN AREA (GAREA) CALCULATION

120  NON-LINEAR SEARCH FOR PEAK POSITIONS AND WIDTHS

130  LISTING AND PLOTTING OF RESULTS

140  COMMENTS

SIN$

SEC$010  HOW TO GET STARTED

BON$

CY1$

Type:  ASAP                   ;To start on CONCURRENT

Type:  @U1:[MILNER]ASAP       ;To start on VAX

BOF$

BLP$

On the CONCURRENT, any plots are generated directly on the Trilog

specified by the TRL command (see \B\SEC# 380.050\\).

BLP$

On the VAX, plots are written to a disk file ASAP.PLT for subsequent

submission to the Trilog print queue. The command:

BON$

CY1$

@U1:[MILNER]TRIP ASAP

BOF$

BLP$

Enters this file into the TRL1: queue and deletes it subsequent to

plotting.

TPF$

SEC$020  GENERAL FEATURES

ODP$

(1)....ASAP is intended to be used for the QUICK, EASY and maybe

DIRTY analysis of high-resolution gamma-ray spectra.

ODP$

(2)....Processes data from HIS- or SPK-files.

ODP$

(3)....Finds peaks, estimates the background, and calculates AREAS by

two methods (a SUMMING method and GAUSSIAN fitting)

ODP$

(4)....Produces a table of PEAK locations, ENERGIES, AREAS,

estimated ERRORS, and PEAK-WIDTHS.

ODP$

(5)....Produces TRILOG plots showing the FIT and again

giving some of the information in (4).

ODP$

(6)....Spectra are analyzed in "sections" chosen by the program.

ODP$

(7)....Sections may be up to 256 channels in length and contain

up to 16 peaks.

ODP$

(8)....Up to 250 peaks may be processed in one FIT request.

ODP$

(9)....Spectra with up to 16384 channels may be processed.

ODP$

(10)...Plots are generated via a separate command which allows

you to plot subsets of the fitted region for more desirable

scaling, shorter plots, etc.

SEC$030  PROGRAM OPERATION

BLP$

The user specifies how the fitting is to be carried out by supplying a number

of FIT specifications.

These are entered as free-form NAME lists and may be given in any order.

Most of these list-elements have DEFAULT values (see \B\SEC# 380.050\\).

After the fitting conditions are specified, one or more FIT requests are

entered. Subsequently, some or all of the fitting conditions may be changed and

more FIT requests entered etc., etc.

\B\SEC# 380.050\\ gives a complete list of the FIT specifications available.

Some of these are described in more detail in \B\SEC# 380.060\\.

SEC$040  SYNTAX

BLP$

All specifications are entered as free-form NAME lists.

By that, I mean a list NAME followed by one or more data items

(numbers or alphanumeric specifiers).

In some cases there are no data items.

The list NAME and all data items must be separated

by one or more delimiters (space , ( ) or /)

BLP$

If more than one NAME list is to be entered on a given line, it must be

separated from the previous list by a semicolon.

A given NAME list must be completed on the same line on which it is started.

TPF$

SEC$050  FIT SPECIFICATIONS - COMMAND LIST

CY1$

CMD   DATA-------Definition (general commands) ------------

IN    FIL.EXT    Open data file (EXT = .SPK OR .HIS)

(on VAX, HIS-file names must include Disk name)

CMD   FIL        Open and process commands from FIL.CMD

CMD   FIL.EXT    Open and process commands from FIL.EXT

TRL   TRLX:      Trilog (TRLX:=TRLU:,TRLO:...) (dflt=TRL:) CONCURRENT

TIT   TITLE      Enter a Title for Trilog plots

END              Ends program

CMD   DATA-------Definition (associated with fitting) -----

KWD   KWD        See text (usually KWD=2*FWHM)     (DFLT KWD=8)

WCAL  FWA,FWB    Peak-width=FWA+FWB*SQRT(chan#)    (DFLT FWA,FWB=4,0)

BIAS  BIAS       Peak-detect threshold             (DFLT BIAS=3.0)

DEL   DX,WDNC    Step size for pos & width         (DFLT 0.0,0.05)

KFIT  KIND       Kind of FIT (GAUS OR SUMA)        (DFLT KIND=GAUS)

KBAK  KIND       Kind of BGD (FULL,FAST,ZERO)      (DFLT KIND=FULL)

BSTD  BSTD       See text (STD DEV for BGD)        (DFLT BSTD=2.0)

MPIS  N          Max # Peaks/section (MAX=16)      (DFLT MPIS=8)

VW    KIND       Width variation (NONE,LOCK,FREE)  (DFLT KIND=LOCK)

FIT   ID,LO,HI   FIT spect# ID   (chans LO thru HI)

FIT   ID         FIT spect# ID   (using previous    LO,HI specs)

FIT              FIT             (using previous ID,LO,HI specs)

CMD   DATA-------Definition (associated with ploting) -----

LIN              LINEAR scale for TRILOG plots

LOG              LOG    scale for TRILOG plots     (DFLT spec)

M20              Label/scale-mark every 20  chans

M50              Label/scale-mark every 50  chans

M100             Label/scale-mark every 100 chans  (DFLT spec)

PPC   PPC        Pixels/chan for plots (MAX=6)     (DFLT PPC=6)

ECAL  A,B        Energy calib - E=A+B*(chan#)      (DFLT A,B=0,1)

PLOT  LO,HI      Plot FIT results (channels LO thru HI)

SEND  STOP       Interrupts plot in progress (on CONCURRENT)

Ctrl/C           Interrupts plot in progress (on VAX)

SEC$060  INPUT QUANTITIES - DEFINITIONS

ODP$

KWD....KWD is a basic width such that 2*KWD will include all of the

stronger peaks (see the following sections for other ways KWD is used)

ODP$

WCAL...FWA,FWB: ASAP computes FWHM from the relation FWHM=FWA+FWB*SQRT(CHAN#).

IF FWA = FWB = 0.0, ASAP estimates FWA and FWB by fitting the widths

calculated from SAREAS and PEAK HEIGHTS.

If ASAP computes FWA and FWB and if KWD was initially set to 0,

ASAP sets KWD = 2*FWHM at mid-range of the FIT request.

CEN$

(continued on next page)

TPF$

SEC$060  INPUT QUANTITIES - DEFINITIONS (continued)

ODP$

BIAS...BIAS: BIAS is a PEAK HEIGHT selector (the number of standard

deviations above  BACKGROUND that a count must be

to be considered part of a PEAK)

Depending on the statistics, BIAS may need to be set to values

ranging from 2.5 to 10.0

ODP$

DEL....DX,WDNC:

DX is the fraction of a channel that PEAKS will be moved

at each step in the non-linear-search for best PEAK position

(DX= 0.0 says do no non-linear search on PEAK position).

A maximum of 10 attempts is made to find the best position.

WDNC is the step size (fraction of the initial value of FWHM)

used in the non-linear search for best peak width/s.

Adjustment of FWHM is limited to 0.3 to 3.0 times the starting

value.

If WDNC = 0.0  FWHM will not be changed.

CYB$

Peak positioning is not usually required - start with DX = 0.0

ODP$

ECAL...A,B:

A is the spectrum intercept and

B is the spectrum GAIN in Kev/channel (or whatever).

(A and B are only used for identification purposes).

ODP$

BSTD...A BACKGROUND computation parameter which effects SAREA only

and is probably best left alone.

SEC$070  BACKGROUND COMPUTATION (FOR SAREA ONLY)

BLP$

The BACKGROUND is calculated by least squares fitting the

spectrum in a number of overlapping sections.

10*KWD or 256 (whichever is smaller) channels are considered

in each FIT.

Each section is fitted three times- the first time to the form

BGD(I)=A+B*X(I) and the last two times to the form

BGD(I)=A+B*X(I)+C*X(I)*X(I).

after each fit any channel containing a count Y(I) is thrown out of

the next fit if (Y(I)-BGD(I)).GT.BSTD*SQRTF(BGD(I)).

(see the list of definitions)

This essentially throws away the PEAKS (at least that is what it is

supposed to do).

Only the center 1/2 of each section is retained except for the first

and last sections.

I.E. the first 1/2 of the second section overlaps the

last 1/2 of the first section etc.

You should probably use BSTD=2.0 (the DEFAULT value).

SEC$080  PEAK FINDING

BLP$

Peak finding is accomplished using a routine due to J. D. Larson.

This routine (used by RIP and STP) computes a "spectrum derivative",

looks for inflection points, locates the largest count near this

inflection point and tests against the peak detect threshold

specified (BIAS in this program). The differentation process uses

KWD/2 as an estimate of the basic peak FWHM and is sensitive (but

not extremely so) to this value.

It does what it does and does it rather well, I think.

TPF$

SEC$090  SUM AREA (SAREA) CALCULATION

BLP$

The SUM AREAS (SAREA'S) are calculated

by summing Y(I)-BGD(I) from I1 to I2 where

I1=(IP-KWD+1)

and

I2=(IP+KWD).

(see the list of definitions)

IP is the rough position of the peak.

I.E. Y(IP) is the maximum peak count.

If peaks are closer together than 2*KWD, the difference is split

to get the summing range.

If IDIFLO is the separation between IP and the next

peak below it and IDIFHI is the separation between IP and the next

peak above, the sum is done from I1 to I2 where,

CY1$

I1=(IP-MINF(KWD,IDIFLO/2)+1)

and

I2=(IP+MINF(KWD,IDIFHI/2)).

BLP$

Where MINF(A,B) means the smaller of A and B.

The quoted ERROR is given by,

CY1$

ERR=100*SQRTF(SUM(Y(I))+SUM(BGD(I)))/AREA

SEC$100  CENTROID CALCULATION

BLP$

The CENTROID or best peak position may be gotten by summing

as follows from I1 to I2.

CY1$

CENTROID=SUM(I*(Y(I)-BGD(I))**2)/SUM((Y(I)-BGD(I))**2   I=I1,I2

BLP$

Notice that this is not really the CENTROID, since the square of the

net count is used rather than the net count, but this method

seems to give a better peak

position under adverse conditions.

SEC$110  GAUSSIAN AREA (GAREA) CALCULATION

BLP$

Peaks which are well separated are fitted by the linear least squares

method to a single gaussian plus a linear background

over the range J1 to J2 where,

J1=IP-2*KWD

and

J2=IP+2*KWD.

If peaks are closer together than 2*KWD the range is extended to include

up to 16 peaks, however the DEFAULT maximum number is 8.

If peaks are still in the way, the range is reduced somewhat to exclude

them. you can tell what the fit range actually was

by looking at the plot (a vertical line is drawn at the range

end points).

The errors in the peak areas are calculated in the conventional way for

this type of fitting but is too involved to discuss here.

TPF$

SEC$120  NON-LINEAR SEARCH FOR PEAK POSITIONS AND WIDTHS

BLP$

If DX is entered (greater than 0),

an attempt is made to find the best positions for all peaks

in the section being fitted by positioning the peaks one

at a time, starting with the largest and working down to the

smallest, in steps of DX

(see the list of definitions)

channels per step.

If DX=0.0, no positioning will be done.

If no best position is found the attempts will be terminated

after 10 trys.

After positioning is completed an attempt may be made to find

the best peak widths.

Peak widths may be

held fixed (VW=NONE), varied together in a given FIT section

(VW=LOCK) or allowed to vary freely (VW=FREE).

Adjustment of peak widths is limited to 0.3 to 3.0 times

initial values.

If WDNC = 0.0 no width search will be done.

SEC$130  LISTING AND PLOTTING OF RESULTS

BLP$

Most of the output is self explanatory, with a few exceptions:

ODP$

(1)....XG of the table denotes the best position found in the positioning

operation.

ODP$

(2)....AFWHM denotes the adjusted FWHM.

ODP$

(3)....The output col labeled 100(S-G)..../G compares GAUSSIAN AREAS

(GAREA) and SUM AREAS (SAREA) and

flags (with ***) if the agreement is worse than the estimated

ERRORS predict.

ODP$

(4)....If you don't use GAUSSIAN analysis the GAREA col will contain the

approximate PEAK HEIGHT. (this is just an accident).

ODP$

(5)....Trilog plots and printer listings may be matched by noting

the file-name and exact date and time printed at the top of each.

SEC$140  COMMENTS

ODP$

(1)....I don't know how well this program will work for your data.

Only you can determine that.

It's not going to work very well if the whole region of interest

is just one mass of overlapping peaks.

ODP$

(2)....Performance will depend on the fitting conditions that you

specify, of course. The DEFAULTS are set to "reasonable" values

for an "average" Ge(Li) spectrum - whatever that is.

If it is possible to supply good width coefficients (FWA & FWB)

and keep the widths of all peaks in a section locked together

(VW LOCK), results should be more satisfactory.

ODP$

(3)....In most cases it is best not to set the maximum number

of peaks per section to too large a value.

It makes the program slow and gives poor results for the smaller

peaks in the section. You will just have to find out what works

(or doesn't work) for your data.

QIT$