RPD$070066001

SIN$

RSH$0202

CHP$NEW STUFF INDEX

TPF$

DBL$

CYB$

TAB....Contents   - Description --------------------------------------

(1)....CHIL       - Symbols (addition to documentation only)

(2)....PARAMETERS - Increase in number allowed to 2000

BANANAS    - Increase in number allowed to 880

(3)....DAM        - New Commands - FIND, WO, MARC, MARE, KRUD

SAM        - New Commands - FIND, WO

STU        - New Command  - RECL for long-record ASCII files

STP        - New Commands - SCON/SCOF turn scale markers ON/OFF

(4)....HISSUMS    - Summing procedure for signed 16-bit histograms

(5)....Exabyte    - Biting The Exabyte Bullet

Exabyte    - Status & unload commands for SCAN, LEMO & HACO

(6)....EXAB       - Exabyte-drive exercise program

(7)....FARC       - Fractional archiving of Concurrent files

(8)....BMX        - Big-Mother-FIFO exerciser

(9)....BATMO2     - Alternate full-feature data acquisition package

(10)...CAT        - Notes, fixes, upgrades and comments

(11)...REBUF      - Special process; User access to raw events

UPVALL     - Special process; User processing of raw buffers

SCANM      - Special process; User generated memory-histograms

SCANB      - Special process; User generated disk-  histograms

(12)...VAXPAK     - Symbols - easy way to define

QIT$

SIN$

(13)...VAXstation - For public use

(14)...DAMS/SAMS  - VAXstation versions of DAM & SAM

RPD$070066001

SIN$

CHP$320.  CHIL - WTM

RSH$1010

TPF$

SEC$160  LOOPS & SYMBOLS

BLP$

Symbols defined by the "Equate Directive" (SYM=EXPRESSION)

are called compile-time variables.

Such variables may be re-defined without restriction. Note: These are

not run-time variables! - THERE ARE NO RUN-TIME VARIABLES IN CHIL!!

Compile-time variables must always be defined in terms of numbers

and/or previously defined compile-time variables.

At a given place in a CHIL program, the value associated with a

particular symbol is always the same.

See SEC# 320.170 for a discussion of expressions.

BLP$

Loops are implemented so as to "look like FORTRAN". However, one

must keep in mind that "loops" are actually expanded by the CHIL

complier

and that all "loop parameters" are constants. Here are a couple of

examples.

CYB$

EXAMPLE-1: -----------------------------------------------------------

BLP$

Produce 72 1-D histograms of parameters 1,4,7...214 with

a gate (500,1000) set on associated parameters 2,5,8...215.

CY1$

DO 100 I=1,214,3

J=I+1

H(I) L(512) G(J,500,1000)

100 CONTINUE

CYB$

EXAMPLE-2: -----------------------------------------------------------

BLP$

Produce 72 1-D histograms of parameters 1,4,7...214 with

the requirement that a parameter named ETOT (parameter # 250)

be in the range 1000 to 2000.

Define parameter names and associated values as follows:

CY1$

$DIP NAI(72),ETOT(1)                 ;Define parameter names

$ASS NAI(1 TO 72)=1,3                ;Assign values to NAI

$ASS ETOT(1)=250                     ;Assign value  to ETOT

IFU(G(ETOT(1),1000,2000)) 200  ;Skip loop if gate un-satisfied

DO 100 I=1,72                  ;Loop on 72 NAI detectors

H(NAI(I)) L(256)               ;Histogram NAI(I)

100 CONTINUE

200 CONTINUE

RPD$070066001

SIN$

RSH$1010

CHP$Increase in #Parameters & #Bananas

TPF$

DBL$

CYB$

To:      HHIRF Computer Users

From:    W. T. Milner

Subject: Increase the number of parameters and bananas allowed

Progs: CHIL, SCAN, MO1, LEMO & MEMO  now support up to 2000 parameters

Progs: CHIL, SCAN, MO1, LEMO         now support up to  880 bananas

Progs: PLO, MILDO, DAM, BANDO        now support up to  880 bananas

VAXPAK programs (CHIL, SCAN4, SCAN16, LEMO, PLO, MILDO, DAM & BANCO)

have been modified similarly.

RPD$070066001

SIN$

RSH$1010

CHP$New Commands

TPF$

CYB$

The DAM Command - KRUD -----------------------------------------------

BLP$

A new command has been added to DAM to reduce the

agony associated with the display of large numbers of 1-D channels.

The command:

CY1$

KRUD  NC

BLP$

Where, NC denotes a number of channels to be summed together (and

normalized by dividing by NC) for display purposes only. For example,

if NC=10 you will see a display that differs from the normal one only

in that there only 1/10 as many histogram bins.

CYB$

The DAM Command - FIND -----------------------------------------------

BLP$

A peak find feature has been added to DAM. The

following new commands are accepted:

CY1$

FIND BIAS,IFWHM     ;Turn peak-find ON  (see definitions below)

FIND                ;Turn peak-find ON  (with defaults - see below)

NOFI                ;Turn peak-find OFF (default is OFF)

ODP$

BIAS...is the number of standard deviations above background that a

peak channel must be in order to be considered as part of a peak.

Useful values of BIAS are in the range 3 to 10.

The default value of BIAS is 5.0.

ODP$

IFWHM..is the approximate full-width at half-max (in channels)

of peaks in the region of interest.

This value is not very critical but should be within a factor of

2 or so of the correct value. The default value of IFWHM is 5.

BLP$

All peaks found within the display region will be marked & labeled

with the associated energy-calibration value (def#ault is the

same as channel number). Peak labels are integers (no decimals - to

minimize screen space used) so if you want labels to be in units of

keV, for example, you must enter CAL such that E(keV) is a whole

number.

CYB$

The DAM Command - MARC -----------------------------------------------

BLP$

A channel and energy marking feature have been added to DAM.

The commands defined below are displayed when you type: HELP MARK

CY1$

MARC I,J,K..   ;Mark channels I,J,K.. on current display

;Saves marked channels in mark-list for MALL etc below

;Channel markers are labeled with integer values

MARE X,Y,Z..   ;Mark energies X,Y,Z.. on current display

;Uses calibration from CAL specification

;Energy marker labels show 1 digit to right of decimal

;Saves marked energies in mark-list for MALL etc below

MALL           ;Display all markers from mark-list on current display

KMAR           ;Erase all markers & zero mark-list

ZMAR           ;Zero mark-list but do not erase current markers

TPF$

CYB$

The DAM Command - WO -------------------------------------------------

BLP$

A new command WO [means the same thing as WOA or WHOA - i.e the

opposite of GIDDUP (my preferred spellings)]

has been implemented to work within LOOPs for programs

DAM and SAM. Whenever the WO command is encountered

(within a LOOP only, otherwise it's illegal), the message:

CY1$

Type [RETURN] to CONTINUE--->

BLP$

will appear on the screen. This gives you an opportunity to look at

the display etc. before it gets wiped out. When you are finished

looking, press the [RETURN] key and it will continue.

BLP$

Before you ask - Yes, I could let you specify a time delay rather than

have you press the [RETURN] key but you would never be happy with the

delay that you chose and would just want some other "feature" to

overcome the disadvantages of delay feature. Anyway, if you don't like

it, it's use is not required.

CYB$

The SAM Command - FIND -----------------------------------------------

BLP$

A peak find feature has been added to SAM. The

following new commands are accepted:

CY1$

FIND BIAS,IFWHM     ;Turn peak-find ON  (see definitions below)

FIND                ;Turn peak-find ON  (with defaults - see below)

NOFI                ;Turn peak-find OFF (default is OFF)

ODP$

BIAS...is the number of standard deviations above background that a

peak channel must be in order to be considered as part of a peak.

Useful values of BIAS are in the range 3 to 10.

The default value of BIAS is 5.0.

ODP$

IFWHM..is the approximate full-width at half-max (in channels)

of peaks in the region of interest.

This value is not very critical but should be within a factor of

2 or so of the correct value. The default value of IFWHM is 5.

BLP$

If FIND is enabled, SAM will do a peak find within the display region

each time a D (or DX) command is given. An attempt will then be

made to add the newly found peaks to the internal peak library.

If a newly found peak is closer than 0.5*FWHM channels to an existing

library peak, it will not be added. Finally, all library peaks will

be marked on the display in the usual manner. No distinction is made

between "found peaks" and "manually entered peaks".

TPF$

CYB$

The SAM Command - WO -------------------------------------------------

BLP$

A new command WO [means the same thing as WOA or WHOA - i.e the

opposite of GIDDUP (my preferred spellings)]

has been implemented to work within LOOPs for programs

DAM and SAM. Whenever the WO command is encountered

(within a LOOP only, otherwise it's illegal), the message:

CY1$

Type [RETURN] to CONTINUE--->

BLP$

will appear on the screen. This gives you an opportunity to look at

the display etc. before it gets wiped out. When you are finished

looking, press the [RETURN] key and it will continue.

BLP$

Before you ask - Yes, I could let you specify a time delay rather than

have you press the [RETURN] key but you would never be happy with the

delay that you chose and would just want some other "feature" to

overcome the disadvantages of delay feature. Anyway, if you don't like

it, it's use is not required.

CYB$

The STU Command - RECL -----------------------------------------------

BLP$

A new command has been added to STU which facilitates the copying of

ACSII records of up to 252 bytes in length. For example, to copy

ASCII records of maximum length 132 bytes, enter the following command:

CY1$

RECL 132

BLP$

You will have to enter this command before copying the file of interest

both to and from tape. Once entered, RECL is unchanged until entered

again. The default record length for ASCII (anything other than spk or

his files) is still 80 bytes.

RECL is defined in the STU help-list.

CYB$

The STP Command - SCON/SCOF ------------------------------------------

CY1$

SCON                   Turns Scale-mark ON  (the default)

SCOF                   Turns Scale-mark OFF

RPD$070066001

SIN$

RSH$1010

CHP$330. PROCESSING UTILITIES

TPF$

CY1$

SECTION  CONTENTS

010  HISSUM   - DISPLAYS COUNTS FOR ALL HISTOGRAMS ON HIS-FILE

020  SPKSUM   - DISPLAYS COUNTS FOR ALL HISTOGRAMS ON SPK-FILE

030  SPKMOM   - COMPUTES MOMENTS OF 1-D HISTOGRAMS IN SPK-FILES

040  DDIR     - DISPLAYS DIRECTORY FOR SPK- OR HIS-FILE

050  BANDO    - BAN-FILE MANIPULATION PROGRAM

060  HEDDO    - PROG TO DISPLAY & MODIFY HIS- & SPK-FILE HEADERS

SEC$010  HISSUM   - DISPLAYS COUNTS FOR ALL HISTOGRAMS ON HIS-FILE

BLP$

HISSUM displays (and lists) the total number of counts in all

histograms (1-D & 2-D) in a HIS-file. Let USER.HIS denote the name

of the file to be examined.

CY1$

FOR 16-BIT/CHANNEL DATA TREATED AS UNSIGNED --------------------------

CY1$

Type:  HISSUM  USER       ;To display on terminal

Type:  HISSUM  USER,PR:   ;To display on terminal and line printer

Type:  HISSUM  USER,TRL:  ;To display on terminal and Trilog

FOR 16-BIT/CHANNEL DATA TREATED AS SIGNED ----------------------------

Type:  HISSUMS USER       ;To display on terminal

Type:  HISSUMS USER,PR:   ;To display on terminal and line printer

Type:  HISSUMS USER,TRL:  ;To display on terminal and Trilog

SEC$020  SPKSUM   - DISPLAYS COUNTS FOR ALL HISTOGRAMS ON SPK-FILE

BLP$

SPKSUM displays (and lists) the total number of counts in all

histograms in a SPK-file. Let USER.SPK denote the name

of the file to be examined.

CY1$

Type:  SPKSUM  USER       ;To display on terminal

Type:  SPKSUM  USER,PR:   ;To display on terminal and line printer

Type:  SPKSUM  USER,TRL:  ;To display on terminal and Trilog

RPD$070066001

SIN$

RSH$1010

CHP$XXX  Biting the Exabyte Bullet - II

TPF$

DBL$

CY1$

Section  Contents

010  Introduction

020  Loading & Unloading Tapes

030  Using the Exabyte as an Input Device

040  Using the Exabyte as an Output Device

050  Using the Exabyte in Data Acquisition

SIN$

SEC$010  Introduction

BLP$

An Exabyte tape drive is a hi-capacity data storage device which

is rather new but is rapidly spreading throughout our world.

We have 12 Exabyte drives on the

Concurrent system (i.e 4 per CPU).

There is also two on PHYVAX

two on the MIPS, and two on a DECstation-3100.

The purpose of this document is to introduce potential users to the

personality of this device.

Some general features are:

ODP$

(1)....It uses video record/playback technology and standard 8mm

video cassettes as the storage medium.

ODP$

(2)....One video cassette holds about 2 gigabytes of data and costs

$9.50. This is 13 times as much data as one 6250-bpi mag tape which costs

about $14.00.

ODP$

(3)....This is a streamer device whose maximum read/write speed is

246 kb/sec. This speed is equivalent to writing a 6250-bpi tape in

10 minutes.

ODP$

(4)....At reduced speed (anything less than the maximum - streamer

speed), the forward motion of the tape stops and restarts frequently,

however the drive keeps up with the requested throughput to within

a very small percentage (5% or less).

ODP$

(5)....We don't know how long a drive will last under our (or any

other) operating conditions. We have had several in heavy use on the

Concurrent system for about one year at this date (2/7/90) and have

not experienced undue difficulty.

ODP$

(6)....If you get an unrecoverable read error, you can't recover the data untiL

you have advanced past one file-mark.

TPF$

ODP$

(7)....Some things take a lot of time with the Exabyte system. It

may be helpful to be aware of the following delays.

DBL$

CY1$

Time to open door on an empty drive                  =   5 sec

Time to open door on a rewound tape                  =  20 sec

Time to open door on a fully wound tape              = 200 sec

Time to get ready-light after inserting tape         =  35 sec

Time from first read/write request until it does it  =  60 sec

Time to write   one file-mark                        =  12 sec

Time to back-up one file-mark (adjacient)            =  12 sec

Time to back-up one record                           =  forever

SIN$

FIP$

It takes "forever" to back-up one record because it backs-up

one file and reads forward to position the tape one record behind

where it was.

ODP$

(8)....There is a cleaning procedure for the drive but only skilled

personel are allowed to do it.

Don't even think about doing such a thing yourself!

ODP$

(9)....Sony (the only brand to use) 8mm video tape can be ordered by

Joan Copeland. They are currently stocked in stores but nobody knows

what may happen when Avid in fully implemented.

ODP$

(10)...You will find that they are slow to start but once started

they don't like to stop. If you are patient and gentle with them,

the results can be delightful. You may have encountered analogous

situations before.

TPF$

SEC$020  Loading and Unloading Tapes

CY1$

**********************************************************************

*                                                                    *

*   "============================================================"   *

*   "                                                            "   *

*   "      !----------------------------------------------!      "   *

*   "      !                                              !      "   *

*   "      !<------------------ WINDOW ------------------>!      "   *

*   "      !                                              !      "   *

*   "      !----------------------------------------------!      "   *

*   "                                                            "   *

*   "                                                            "   *

*   "<-------------------------- DOOR -------------------------->"   *

*   "                                                            "   *

*   "                                                            "   *

*   "                                                            "   *

*   "============================================================"   *

*                                                                    *

*        RRR  GGG            PUSH-BUTTON ------------>BBBBBB         *

*        RRR  GGG                                     BBBBBB         *

*        RRR  GGG<---------- GREEN-LIGHT              BBBBBB         *

*        RRR  GGG                                     BBBBBB         *

*         !<---------------- RED---LIGHT                             *

*                                                                    *

**********************************************************************

BLP$

A sketch of the front-view of an Exabyte tape drive is shown above.

It's a pretty simple device to use. There is door through which tapes

are loaded and unloaded, a window in the door for viewing the tape

which is inside, one push-button for opening the door, one green light

(right hand)

to indicate when the drive is "ready" and one red light (left hand)

used to indicate that some movement is taking place.

CYB$

Loading a Tape Into an Empty Drive -----------------------------------

ODP$

(1)....Push the button and wait (about 5 sec) for the door to open.

ODP$

(2)....Remove the cassette from it's case and insert it into the

carrage with the label-side up (reels are visible on top).

ODP$

(3)....Gently push the door until it latches.

ODP$

(4)....Wait (about 30 sec) for the green (right hand) "ready light" to come on.

It will still take about one minute

more (after you request the first read/write) for it to really

get going.

CYB$

Unloading a Tape -----------------------------------------------------

ODP$

(1)....Push the button and wait for the door to open. This takes

about 20 sec for a rewound tape. Otherwise,

it can take up to 3 minutes if the tape is wound to end-of-medium.

ODP$

(2)....When the door opens, remove the tape and reclose the door

or insert another tape as described in (2) through (4) above.

TPF$

SEC$030  Using the Exabyte as an Input Device

BLP$

The Exabyte is used just as you would use a mag tape with the following

qualifications:

ODP$

(1)....You should be aware of the normal time delays.

ODP$

(2)....You should not attempt to do a "back record" function.

ODP$

(3)....All SYSIO functions normally used for mag tapes are legal

for the Exabyte except for the unload function, Y'81'.

BLP$

SCAN and LEMO have been modified to support Exabyte.

Device names are XMT1:, XMT2:, etc.

SEC$040  Using the Exabyte as an Output Device

BLP$

All of the comments relating to use as an input device apply to output

as well.

In addition, there are some special limitations on when you can write

to an Exabyte tape. You can only write to a tape under one of the

following conditions:

ODP$

(1)....If the tape has been fully rewound.

ODP$

(2)....If the last tape operation was a "write".

ODP$

(3)....If the tape is positioned "just in front of" a file-mark AND

if that position was attained by means of a forward-file or a

backward-file operation. The LEMO command BTO puts the tape in such

a position provided that the tape was ended normally (i.e. by a double

file-mark).

SEC$050  Using the Exabyte in Data Acquisition

BLP$

In data acquisition, the Exabyte is assigned (opened) by one of the

following commands:

CY1$

X1  - opens XMT1:

X2  - opens XMT2:,  etc

BLP$

Because of the inability to recover data following an unrecoverable

read error until you advance past the next file-mark, I will

automatically write a file-mark (followed by a primary header)

after every 5000 records. Header numbers will be advanced as well.

BLP$

With mag tapes, I now write 2 file-marks and back-up 2 file-marks

every time you stop the acquisition so that the tape is always

properly terminated. I will not do this for Exabyte tapes because it

would take about 60 sec and I think that would be a nuisance. You

should remember to close the tape before you KILL the acquisition

system, etc.

RPD$070066001

SIN$

RSH$1010

CHP$Exabyte Status & Unload for SCAN & LEMO

TPF$

BLP$

Commands which return Exabyte status information have been added

to SCAN and LEMO. Also, any commands or conditions which normally

unload standard magnetic tapes will now unload Exabyte tapes as well.

The commands:

CY1$

STX         While in the "WHAT NOW?" state or

SEND STX    While processing data, will display & log (if LON):

ODP$

(1)....The number of recoverable read/write errors encountered on

the cassette tape so far,

ODP$

(2)....The number of recoverable errors per megabyte read/written and

ODP$

(3)....The number of

megabytes remaining on the tape (measured from it's current position).

BLP$

Implementation of the status and

unload features requires the use of extended SVC1 LINK options.

This means that you may (or may not) have to do something!!

ODP$

(1)....If you only use standard SCAN & LEMO (no USERSUBs) and you

use SCAN.CSS from the system account, you should just take note of

the STX and SEND STX (Exabyte status commands) and be aware that

Exabyte drives now unload under the same conditions as standard

9-track drives (MTH1:, etc).

ODP$

(2)....If you use the standard SCAN.TSK/S but have a customized

SCAN.CSS, you should reconstruct your SCAN.CSS (or whatever you call

it)

using the one from the system as a template, otherwise HELP won't

work!

ODP$

(3)....If you use a customized SCAN task (with USERSUBs, etc), you

must (before relinking) reconstruct any customized "SCANLNK procedure"

(using SCANLNK.CSS/S as a template) and check to see if any of your user

routines call routine SYSIO. If you call SYSIO, you must make sure

that the parameter block IPBLK (the first SYSIO argument) is

dimensioned at least 6 and that IPBLK(6)=0,  which it will be (by

default) unless you have set it to something else.

Reconstruct any customized "SCAN.CSS" as described in item-(2).

ODP$

(4)....If you use a customized LEMO task (with USERSUBs, etc), you

must (before relinking) reconstruct any customized "LEMOLNK procedure"

(using LEMOLNK.CSS/S as a template) and check to see if any of your user

routines call routine SYSIO. If you call SYSIO, you must make sure

that the parameter block IPBLK (the first SYSIO argument) is

dimensioned at least 6 and that IPBLK(6)=0,  which it will be (by

default) unless you have set it to something else.

BLP$

If you are using someone else's program and don't know what any of

this means, you should call it to the attention of the "creator"

of your program. Until such time as you need to re-LINK, you should

only have to worry about item-(2).

RPD$070066001

SIN$

RSH$1010

CHP$Exabyte Status & Unload for HACO

TPF$

CY1$

To:       HHIRF Data-acquisition users

Subject:  Standardized HACO & Exabyte status & unload

From:     W. T. Milner

BLP$

Commands which return Exabyte status information have been added to

HACO. Also, any commands or conditions which normally unload standard

magnetic tapes will now unload Exabyte tapes as well.  The commands:

CYB$

STX         displays information without logging,

LSTX        displays and records (on BATMAMA.LOG) the following:

ODP$

(1)....The number of recoverable read/write errors encountered on the

cassette tape so far,

ODP$

(2)....The number of recoverable errors per megabyte read/written and

ODP$

(3)....The number of megabytes remaining on the tape (measured from

current position).

CYB$

All other HACO versions (HACOX & HACOXX) have been deleted and system

account FIG-files have been up-dated. You should reconstruct your own

FIG-files using those on the system as templets.

CEB$

Note!!!

BLP$

If you allow the output tape to run to end-of-medium, it

will unload automatically (good) but when you put in another tape and

try to open it again it will still think that it is at EOM and will

abort the open (not good!).

In data acquisition, I have defined a "soft end-of-medium" using

the Exabyte status information. This should prevent you from ever

getting a hardware EOM status and thus eliminate the problem

mentioned above.

CEB$

The Command - ?BUF

BLP$

The command ?BUF will display the number of buffers written to tape

and histogrammed - since the last ZBUC command.

It can be typed any time - you don't have to be stopped.

RPD$070066001

SIN$

RSH$1010

CHP$EXAB - Exabyte Exerciser

TPF$

CY1$

BLP$

Program EXAB is intended to be used to evaluate the performance of

Exabyte tape systems.

The objective is to determine maximum writing speed, writing speed

at less than full-speed request (dead time at sub-streamer speed),

tape utilization efficiency, error recovery capability, etc.

The following program features are available:

ODP$

(1)....Tapes may be written with program generated data records.

ODP$

(2)....Tapes may be examined (records read and displayed).

ODP$

(3)....Tape control functions (rewind, forward-file, backward-file,

forward-record, unload, etc) are provided.

ODP$

(4)....Time delays may be inserted between any or all write operations

including data-records, file-marks and backward-file functions.

ODP$

(5)....The write operation is specified in terms of a record length

LREC, a number of records to write per write-loop NREC, a number of

file-marks to write following the completion of a write-loop NEOF,

a number of back-files to do following the writing of file-marks NBAK

and finally the number of write-loops to execute NLOO.

ODP$

(6)....A repeat command RON specifies that when an end-of-medium is

encountered on the output tape, it will be rewound and the whole

process repeated forever until you stop it or something breaks.

CYB$

Type:  EXAB                 ;and then study the HELP

BLP$

A list of program commands and definitions of the quantities displayed

and logged are given on the next page.

TPF$

CYB$

Definition of EXAB Commands ------------------------------------------

CY1$

CMD  Data        ;Meaning or action ----------------------------------

END              ;Ends program

HELP             ;Displays about half of commands

HLP1             ;Displays other half of commands

OUTA  XXXX:      ;Assigns tape XXXX: for output (and input)

OUTA  XXXX:      ;ASSIGN TAPE XXXX: FOR OUTPUT

NLOO  IV         ;#WRITE-LOOPS TO DO

NREC  IV         ;#RECORDS/WRITE-LOOP

NEOF  IV         ;#FILE-MARKS/WRITE-LOOP

NBAK  IV         ;#BACK-FILES/WRITE-LOOP

DLA0  IV         ;DELAY(MS) BETWEEN RECORDS

DLAA  IV         ;DELAY(MS) BEFORE  FILE-MARKS

DLAB  IV         ;DELAY(MS) BEFORE  BACK-FILES

DLAC  IV         ;DELAY(MS) BETWEEN WRITE-LOOPS

RWO              ;REWINDS   OUTPUT TAPE

DAT              ;DISPLAYS  CURRENT PARAMETERS

STX              ;SHOWS #ERRS & #KB-LEFT FOR EXABYTE

SBUF             ;SHOWS FULL SCSI-STATUS BUFFER

RUN              ;STARTS RUN  (WRITING TO TAPE)

READ             ;STARTS READ (SAME LOOPS AS RUN)

SEND STOP        ;INTERRUPTS RUN

SEND STX         ;SHOWS #ERRS & #KB-LEFT WHILE RUNNING

SEND SBUF        ;SHOWS FULL STATUS BUF  WHILE RUNNING

RWO              ;REWIND OUTPUT TAPE

ULO              ;UNLOAD                OUTPUT TAPE

BTO              ;GO TO BOTTOM       OF OUTPUT TAPE

FRO   IV         ;FORWARD IV RECORDS ON OUTPUT TAPE

BRO   IV         ;BACKUP  IV RECORDS ON OUTPUT TAPE

FFO   IV         ;FORWARD IV FILES   ON OUTPUT TAPE

BFO   IV         ;BACKUP  IV FILES   ON OUTPUT TAPE

RDO   IV         ;READ    IV RECORDS ON OUTPUT TAPE

DA    IA,IB      ;DISPLAY WORDS IA THRU IB IN ASCII

DI    IA,IB      ;DISPLAY WORDS IA THRU IB IN INTEGER

DZ    IA,IB      ;DISPLAY WORDS IA THRU IB IN HEX

DEV   IA,IB      ;DISPLAY WORDS IA THRU IB AS EVENTS

DIF   IA,IB      ;DISPLAY WORDS IA THRU IB AS FULLWDS

RON/ROF          ;REPEAT-FLAG ON/OFF (DFLT=OFF)

LON/LOF          ;LIST-FLAG ON/OFF (DEFAULT = OFF)

CYB$

Definition of Quantities Displayed/Logged During Write-Loop ----------

CY1$

LUP#  =  Number of write-loops executed

NREC  =  Number of records/write-loop

T     =  Time(sec) since beginning of write-loop

#RKB  =  Number of KB remaining on tape

TER   =  Total number of recoverable errors encountered so far

DER   =  Number of recoverable encountered since last log

E/MB  =  Number of errors/megabyte

RPD$070066001

SIN$

RSH$1010

CHP$Fractional ARChiving - FARC

TPF$

CY1$

To:      Concurrent (Perkin-Elmer) Computer Users

From:    W. T. Milner

Subject: Concurrent Disk Space

BLP$

As we all know, disk space is a continuing problem. The nanny system

is not working very well, in this regard, and I have no reason to

think that it ever will again. We have too many users

(both inside, outside and transients) for us to use the "mother"

approach (i.e. looking for people, calling, asking and giving

special consideration to this and that).

BLP$

We have implemented a Fractional ARChiving (FARC - pronounced FARK)

procedure which should enable us to

keep the system operating without so much personal contact and

case-by-case analysis and delay.

When any disk (especially system disks) become more than about 90%

allocated, it's usage will be reduced to around 70% by a

procedure which:

ODP$

(1)....Selects enough files for archiving to release required space.

ODP$

(2)....Does a backup of these files and deletes them from disk.

ODP$

(3)....Keeps a FARC log of the CPU, disk, date & files archived.

BLP$

Files to be archived will be selected using an algorithm based on

the number of megabytes and the number of files

associated with a given account and the last dates accessed.

The procedure will involve the following steps:

ODP$

(1)....A global greed-factor GGF(#files,#megabytes), which considers

all disks and all CPUs, will be computed for all accounts.

ODP$

(2)....A local greed-factor LGF(#files,#megabytes), which considers

only the disk of interest, will be computed for all accounts.

ODP$

(3)....A vulnerability-factor VF is associated with each account.

VF will range from 0.0 for system and on-line accounts to 1.0

for outside user accounts.

ODP$

(4)....An exposure-factor EF=GGF*LGF*VF will be computed for all

accounts and the account with the largest EF will be chosen.

The lucky file within the chosen account will be that with the largest

SIZE*AGE product. Where, AGE=#days-since-last-access.

ODP$

(5)....Subsequently, all LGFs and EFs will be re-computed

and the next lucky account and file will be chosen, etc.

CYB$

Comments on File Selection -------------------------------------------

ODP$

(1)....You can exert some control over which files

get lucky but only your greed and vulnerability factors determine

the number of megabytes selected from your account.

CEN$

(continued on next page)

TPF$

ODP$

(2)....If you are a normal user, FARC will attempt to get space from

you by only choosing indexed files with RECL=2048 (SPK-files)

and contiguous files (mostly HIS- and TSK-files). If you don't

have enough files of this type, you will be put in a catagory where

all files are FARCable.

ODP$

(3)....PSF-files are not charged to you in computing greed-factors

but small ones may be deleted without backup and large ones

re-named (with a new name) to account-0, again without backup.

Otherwise,

FARC doesn't pay any attention to filename extensions, so don't

try to rename a 131K sector HIS-file to X.CSS to escape.

ODP$

(4)....The last-date-accessed has an effect on the order that files

are selected for a given account but not on the total number of

megabytes FARCed. So don't bother to set up some kind of program to

open all of your files every so often as you used to do for the VAX

"scratch disk" - it won't help.

ODP$

(5)....Files which have any protection keys set, will be the last

to be selected from a given account, however, only the order of

selection will be modified, not the total number of megabytes FARCed.

ODP$

(6)....In computing greed-factors, it is the fraction of the total

space and number of files associated with your account that matters;

not the absolute values.

That is, what matters is your relative greed.

CYB$

Current Algorithm ----------------------------------------------------

CY1$

GGF    = (GNUMF/GSUMF + 3.0*GNUMB/GSUMB)/GGFMAX  and

LGF    = (LNUMF/LSUMF + 3.0*LNUMB/LSUMB)/LGFMAX  where:

GNUMF  = Global # of files     for a given account

GSUMF  = Global # of files     for all     accounts

GNUMB  = Global # of megabytes for a given account

GSUMB  = Global # of megabytes for all     accounts

LNUMF  = Local  # of files     for a given account

LSUMF  = Local  # of files     for all     accounts

LNUMB  = Local  # of megabytes for a given account

LSUMB  = Local  # of megabytes for all     accounts

GGFMAX = Maximum value of GGF

LGFMAX = Maximum value of LGF

Global refers to all disks on all CPUs, Local to the disk being FARCed

VF     = 0.0  for system  accounts

VF     = 0.25 for group   accounts

VF     = 0.50 for normal  accounts

VF     = 1.00 for outside accounts

BLP$

On occasion, you may

feel that your files have been unfairly FARCed but then nobody ever

said that an algorithm was fair!

RPD$070066001

SIN$

RSH$1010

CHP$FARC ALGORITHM

TPF$

DBL$

CY1$

To:      FARC Fearfuls

From:    W. T. Milner

Subject: FARC Algorithm

BLP$

Before implementing the FARC procedure, I asked 9 users (Beene,

Lee, Varner, Shapira, Ferguson, Carter, Halbert, Hensley and Auble

(Backtash was out of town))

for comments and suggestions. Six people said

that they liked the idea, two people gave no response and one asked

how he could compute his own GREED factor.

Three people made suggestions which I incorporated. Nobody objected

to the process.

BLP$

Now, if you think that the current algorithm is unfair, you may

suggest a different one. Remember, that it must be based on information

that I have access to (#files, #megabytes, last date accessed and

account status). Also, there must be a single suggestion agreed to

(and signed) by all HHIRF group leaders. If you want certain people

or groups to have special status (immune or low vulnerability), that

must be agreed to by all group leaders or approved by R. L. Robinson.

BLP$

There are a couple of things to remember: (1) When we need to FARC a

disk, the space is going to come from somewhere. (2) We are not

going to make real-time adjustments except for on-line users.

RPD$070066001

SIN$

RSH$1010

CHP$FARC Operations - FARCOP

TPF$

First, compute new global greed factors unless you are unable to

do it for some reason. Otherwise, just use the current ones. Use

the following procedure:

ODP$

(1)....From Act-10 on all three machines, Type: GREED. This will

take a while since it has to do a D F,-:-.-/*.  Also, realize that

it will require up to 4000 sectors on the System Volume to do the D F.

If you don't have this much space, delete something or modify

GREED.CSS to use another Volume.

ODP$

(2)....XFER the GREED output files (GREED.CPA & GREED.CPB) from their

respective CPUs to Act-10 on CPU-C.

ODP$

(3)....From Act-10 on CPU-C, Type: FARCGGF. This program reads

the three files just XFERed and computes a global greed factor file

FARC.GGF.

ODP$

(4)....XFER FARC.GGF to Act-0 on all CPUs.

ODP$

(5)....Tweak FARC.CMD/0 on the system you are about to FARC. That is,

check #megabytes to FARC (first line), and other entries for currency.

ODP$

(6)....Do the actual FARCing by Typing: FARC PXXX:,MXXX: where, PXXX:

denotes SYSA:, SYSB:, PAK2: etc. and MXXX: denotes MTH1:, MTH2: etc.

ODP$

(7)....If everything goes OK with the BACKUP, delete the selected

files by typing: FARC.DEL.

CYB$

Summary --------------------------------------------------------------

DBL$

CY1$

(1) Run GREED from Act-10 on all CPUs by Typing: GREED

(2) XFER GREED.CPA/10 & GREED.CPB/10 to Act-10 on CPU-C

(3) From Act-10 on CPU-C Type:  FARCGGF

(4) XFER FARC.GGF/10 from CPU-C to Act-0 on all CPUs

(5) Check FARC.CMD/0 on system about to be FARCed

(6) FARC it from the system console by Typing:  FARC PXXX:,MXXX:

(7) Delete files from the system console by Typing:  FARC.DEL

SIN$

CYB$

Definition of FARC.CMD Flags -----------------------------------------

DBL$

CY1$

IMMU - Says Vulnerability Factor = 0.00 (default = 0.5)

GR   - Says Vulnerability Factor = 0.25

OUT  - Says Vulnerability Factor = 1.00

DRY  - Says Vulnerability Factor = 1000.0

ALL  - Says all files potentially lucky (not just contig & SPK)

RPD$070066001

SIN$

CHP$BMX - Big-Mother(-FIFO) Xerciser

RSH$1010

TPF$

BLP$

BMX is a program for testing data acquisition hardware by acting as

a data stream monitor while running under BATMAMA with HACO & SACO.

General features are as follows:

ODP$

(1)....HACO and SACO (or HAC and STOPAC) function the same as in

normal data acquisition.

ODP$

(2)....BMX is sent the same data buffers that MO1 would receive.

i.e. you must run with HON (or HONA if you are also writing tape).

ODP$

(3)....BMX must be sent an INIT command before it will do anything.

ODP$

(4)....BMX will scan the first buffer that it receives subsequent to

an INIT command for an event pattern. All buffers will then be tested

against this pattern.

ODP$

(5)....Events sent to BMX must contain no more than 99 parameters

and be bounded by hex=FFFFs.

ODP$

(6)....All events must be identical (same pattern) but there is no

requirement to have hex-8000 IDs etc (only the hex-FFFF is required).

ODP$

(7)....BMX prints the very first buffer it gets

and the first bad buffer (plus 2 subsequent buffers) following

an INIT command.

ODP$

(9)....BMX displays & logs (on BATMAMA.LOG) the first 50 errors it

finds subsequent to an INIT. The display/log counter is reset

after every 5000 buffers (i.e. allows another 50 error messages).

ODP$

(10)...BMX keeps a count of #buffers, #good-bufs, #bad-bufs, #errors.

The PEEK command displays/logs these counters - INIT resets them.

ODP$

(11)...BMX always attempts to re-sync after a parameter mismatch.

CY1$

COMMAND LIST ..........................................................

INIT    ;Resets all counters & flags & defines terminal to which

;error (and other) messages are to be sent

HELLO   ;Redefines terminal to which display messages are to be sent

NUEV    ;Requests new event pattern scan - and that is all

PEEK    ;Displays & logs buffer & error counters

TYPICAL COMMANDS FOR RUNNING BMX ......................................

*BATMO                  ;Start BATMO as usual

>FIG BMX.FIG/0          ;Standard BMX FIG-file

>HACF BMX.EVS/10        ;Must contain correct CRATE# & SLOT# for AUX

;Also, here is where you can set RECL

>LDAV BMX.EVO/10        ;Event-handler prog to generate test pattern

>HONA                   ;Turn on histogramming

>INIT                   ;Initialize BMX (tell it to get ready)

>RUN                    ;Start the acquisition

>PEEK                   ;Display & log buffer & error counters

>S                      ;Stop the acquisition

>KILL ALL               ;Kill it

RPD$070066001

SIN$

RSH$1010

CHP$BATMO2

TPF$

CY1$

To:       HHIRF data takers

From:     W. T. Milner

Subject:  A second full-feature data acquisition package

BLP$

A second "full feature" data acquisition package (initiated via

BATMO2) has been implemented on CPUs A & C. There are some differences

between the BATMO2 package and the BATMO (primary package).

ODP$

(1)....The BATMO2 system reads data VIA camac from a Kinetic Systems

FIFO Model 3841 using LAMs (just like KIT & CAT). A LAM grader

should be provided as well.

ODP$

(2)....The BATMO2 system uses common segment GBLB for the storage of

1 megachannel of in-core histograms. DAM2 is a task just like DAM

except that it has access to the said in-core histograms.

For other display programs, you have to HUP it to see it.

ODP$

(3)....The templet FIG-file (MO2.FIG/0) does not load RIPAC. It is

not expected that this package will normally be used with a

Chromatics terminal.

BLP$

The following files and segments are used:

DBL$

CY1$

BATMO2.CSS/S       ;Loads BATMO2

BATMO2.TSK/S       ;BATMO2 task communicates with BATMOM2

BATMOM2.TSK/S      ;Executive task of package (like BATMAMA)

BATMOM2.ERR/S      ;The error file

BATMOM2.LOG/P      ;The log   file  (like BATMAMA.LOG)

MO2.TSK/S          ;The monitor task (like MO1)

MO2.FIG/S          ;Templet FIG file for loading package with MO2

MOLNK2.CSS/S       ;For linking customized task for BATMO2 package

HAC2.TSK/S         ;The acquisition task (reads via CAMAC like CAT)

HACFIL2.TMP/S      ;Temp file used by HAC2

SEQUM2.DAT/S       ;Header sequence no. & title file used by HAC2

STOPAC2.TSK/S      ;Task to stop HAC2, etc

GBLB.SEG/S         ;Segment for in-core histograms

QQQ2.IMG/S         ;Data common for HAC2, STOPAC2 & MO2

MOMCOM2.IMG/S      ;Communication common for all tasks & BATMOM2

MMM2.IMG/S         ;Used by MO2 only

RPD$070066001

SIN$

RSH$1010

CHP$Cat Notes

TPF$

DBL$

CY1$

To:       HHIRF CAT lovers and/or haters

From:     W. T. Milner

Subject:  CAT & KIT - Upgrades and Comments

CEB$

Upgrades

ODP$

(1)....A CNAF command has been added. See HELP INIT.

ODP$

(2)....A run time specification has been provided. See HELP OPER.

The elapsed time is displayed when acquisition is stopped.

ODP$

(3)....A buffer counter has been added. See HELP INIT for how to

zero it and HELP OPER for how to display. The counter is displayed

automatically when acquisition is stopped.

Buffers are defined as the number of 8192-byte units of data read.

ODP$

(4)....Both CAT and KIT now support full CHIL features (Bananas and

gate lists were not previously supported). The NUBAN command

(but not the NUGAT command) is also supported.

FIP$

Up to 8M 16-bit channels are allowed but the system CSSs for CAT and

KIT only provide 75kb of extra space. If CHIL says you need more,

make your own CSS and provide it.

If you are generating more than a few hundred k-channels, you should

probably provide a CAT.PSF larger than the default 2048 sectors.

CEB$

Comments

BLP$

If histogramming is turned on (via HON), ALL buffers are histogrammed.

It is like HONA in the more general acquisition package. There is

no "histogram what you can without throttling the input data" mode.

RPD$070066001

SIN$

RSH$1010

CHP$User-supplied Subroutine - REBUF

TPF$

DBL$

BLP$

Users of Concurrent

SCAN and LEMO processing packages can gain access to and

modify the raw event data prior to any CHIL action by means of a

user-supplied routine REBUF.

If calls to REBUF are enabled, the following sequence of operations

will take place:

ODP$

(1)....Buffers (records) are read from the input tape.

ODP$

(2)....The said input buffer is scanned for events (any sequence of

numbers ending with hex-FFFF) and one raw event per call is passed

to routine REBUF. Events which are split across records are taken

care of - the user doesn't have to worry about it.

ODP$

(3)....REBUF produces a new output data stream by storing a new

packed event (just like an Event Handler program - including the

FFFF) into an INTEGER*2 array NULIST, for example.

ODP$

(4)....The new data stream (contained in NULIST) is then passed to

the usual unpacking and CHIL processing, including USERSUB calls etc.

BLP$

CHIL only sees the data in NULIST - it sees the number of parameters

in NULIST - it doesn't know that there was a REBUF operation.

$LSTL (in the CHIL program) is used by the tape reading routine and

is unchanged.

SIN$

CYB$

Type:  SCANLNK MYRUTS,MYSCAN   ;To link a SCAN task with REBUFability

Type:  LEMOLNK MYRUTS,MYLEMO   ;To link a LEMO task with REBUFability

Type:  RBON                    ;At run time to  enable REBUF calls

Type:  RBOF                    ;At run time to disable REBUF calls

;RBOF is the default

An example REBUF and argument definitions are given on next page

TPF$

CY1$

$PROG REBUF

C

C     ----------------------------------------------------------------

C     A do-nothing REBUF routine which just copies PBUF to NULIST

C

C     PBUF   - contains raw packed event (the FFFF is not included)

C     NP     = number of words in PBUF

C     NULIST - is the new output buffer which you load

C     NN     = number of words loaded into output buffer

C              I reset NN to zero whenever a buffer is processed

C

C     you must supply the FFFF to the output stream. I could do it

C     but if you do it, then you have the option of creating more

C     than one event. If you think that you may significantly

C     increase the abount of data (in going from input to output),

C     I may need to know about it - see Important Comment below.

C     ----------------------------------------------------------------

C

SUBROUTINE REBUF(PBUF,NP,NULIST,NN)

C

INTEGER*2 NULIST(16384),PBUF(2000)

C

DO 10 I=1,NP

NN=NN+1

NULIST(NN)=PBUF(I)

10 CONTINUE

NN=NN+1

NULIST(NN)=X'FFFF'

RETURN

END

----------------------------------------------------------------------

CEB$

Important Comment

DBL$

BLP$

I have shown the dimensions of PBUF and NULIST explicitly to be that

defined in the main program.

Note: that NULIST is dinensioned 16384 (i.e. 32768 bytes) or 4 times

the normal input record size of 8192 bytes. NULIST is processed

after each input buffer (of normally 4096 wds) is massaged into it.

I do not check the fullness of NULIST on an event-by-event basis.

As things stand, you would be able to expand the input data by almost

a factor of 4 (for 8192 byte input records) - beyond that, you blow

the program!!

RPD$070066001

SIN$

RSH$1010

CHP$UPVALL - User Processing of Raw LEMO Buffers

TPF$

BLP$

The VAX version of LEMO has been modified such that users are now

able (by means of a user-supplied routine UPVALL) to do the following:

ODP$

(1)....Process raw input buffers into (1 to 3) output data streams

with no CHIL involvment.

ODP$

(2)....Process raw input buffers into new buffers and subsequently

call the standard CHIL processor PVALL which then imposes any CHIL

conditions and generates output buffers.

FIP$

In either case, UPVALL must generate legal L002 output buffers.

BLP$

At run time, calls to UPVALL (rather than PVALL) are enabled and

disabled by the following commands:

CY1$

UPON              ;Enable calling of UPVALL (NPRAW=0, RECL=8192 bytes)

UPON NPRAW        ;Enable calling of UPVALL (         RECL=8192 bytes)

UPON NPRAW,RECL   ;Enable calling of UPVALL

;NPRAW = Max # raw parms (for UPVALL if needed)

;RECL  = Input record length (bytes) if not 8192

UPOF              ;Disables calls to UPVALL (enables calls to PVALL)

;the default

BLP$

The communication with UPVALL (which has been kept symmetric with

PVALL) is via two arguments, COMMON/PPP/, and two auxillary

entry points for initialization. Study the example routine listed on

the following pages as a guide for writing your own UPVALLs.

This routine just copies input buffers into output buffers with no

modification. Also, it is the "dummy routine" linked into the standard

LEMO.

BLP$

Note in the example that indexing in the output buffer is on the basis

of the first element being element-0 rather than element-1. You may

or may not want to do it this way but you should note how it is done

in the example and not get confused.

TPF$

CY1$

C$PROG UPVALL

C

SUBROUTINE UPVALL(INBUF,NWDS)

C

IMPLICIT INTEGER*4 (A-Z)

C

INTEGER*2 INBUF(*)

C

COMMON/PPP/ OUPTR(3),OOFSET(3),OUSIZ(3),OUBUF

C

COMMON/ULOCA/ NPAR,RESYN    !Local COMMON (stuff to save)

C

INTEGER*2 OUBUF(0:4)

C

DATA RESYN/'YES '/

C

C     *************************************************************

C     A demonstration UPVALL that just copies input buffers into

C     output Stream-1

C     *************************************************************

C     INBUF     - Contains input buffer to be processed

C     NWDS      = # of 16-bit words in INBUF

C

C     OUPTR(J)  - Contains output buffer pointer   for Stream-J

C               - Initialized to 0 by main prog & PUTPUT

C

C     OOFSET(J) - Contains current offset in OUBUF for Stream-J

C               = -1 says undefined

C               =     0  for Buffer-1 of Stream-1

C               = 16384  for Buffer-2 of Stream-1

C               = 32768  for Buffer-1 of Stream-2

C               = 49152  for Buffer-2 of Stream-2

C               = 65536  for Buffer-1 of Stream-3

C               = 81920  for Buffer-2 of Stream-3

C

C     OUSIZ(J)  = Output buf size (usually 4096)   for Stream-J

C

C     OUBUF     = Contains output Buffers-(1&2) for Streams-(1,2&3)

C

C     RESYN     = 'YES ' requests resync on next event

C     RESYN     = 'NO  ' says continue processing without resync

C

C     JPAR      = Maximum # of parameters/event if you need it

C     NPAR      = JPAR

C     *************************************************************

CEN$

(continued on next page)

TPF$

CY1$

C     OOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOO

C     ENTRY for processing  (main ENTRY)

C     OOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOO

C

J=1                                  !Set Stream-1

OOF=OOFSET(J)                        !Buffer offset in OUBUF

IF(OOF.LT.0) STOP 'NO OUT BUF'       !Tst for un-set

OEND=OOF+OUSIZ(J)                    !End-of-Buffer

OOF =OOF+OUPTR(J)                    !Current pointer value

C

DO 100 N=1,NWDS                      !Loop on # INBUF WDS

OUBUF(OOF)=INBUF(N)                  !Load output buffer

OOF=OOF+1                            !Inc output pointer

IF(OOF.LT.OEND) GO TO 100            !Tst for full

C

OOFSET(J)=-1                         !Un-set output buffer

C

CALL PUTPUT(J)                       !PUTPUT will set

C                                          !OUPTR(J) = 0

C                                          !OUSIZ(J) = buf size

C                                          !OOFSET(J)= OUBUF offset

C

OOF=OOFSET(J)                        !Buffer offset in OUBUF

IF(OOF.LT.0) STOP 'NO OUT BUF'       !Tst for un-set

OEND=OOF+OUSIZ(J)                    !OUPTR(J) = 0 now

C

100 CONTINUE

RESYN='NO  '                         !Request no resync

OUPTR(J)=OOF-OOFSET(J)               !Save out-pntr for later

RETURN

C

C     OOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOO

C     ENTRY for setup and initialization

C     OOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOO

C

ENTRY UPVSETUP(JPAR)

C

NPAR=JPAR                            !Max # of parameters

OOFSET(1)=-1                         !Set no OUBUFs

OOFSET(2)=-1                         !Set no OUBUFs

OOFSET(3)=-1                         !SEt no OUBUFs

RESYN='YES '                         !Request resync

RETURN

C

C     OOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOO

C     ENTRY for event-scrub

C     OOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOOO

C

ENTRY UPRSCRUB

C

RESYN='YES '                         !Request resync

RETURN

END

RPD$070066001

SIN$

RSH$1010

CHP$SCANM - In-memory Histogramming (Concurrent version)

DBL$

TPF$

BLP$

SCANM is an all in-memory histogramming package with the following

features and restrictions.

ODP$

(1)....A proper DIR-file and HIS-file must exist.

ODP$

(2)....The main program calls user-supplied routine HISSUB(IBUF,N)

with an INTEGER*2 event buffer IBUF

which may contain a raw buffer from

tape (default) or a single packed event

(requested by the CMD SEBU).

If a single event buffer is chosen, N is the number of words in

IBUF (the FFFF is not passed). For the raw tape buffer case, N is

the buffer length (normally 4096).

ODP$

(3)....Histogramming is accomplished by user-supplied routines via

calls to routine COUNT1(ID,IX,JY): where, ID is a histogram ID-number

in the range 1 to 8000. IX and JY are X- and Y-channel numbers in the

range 0 to whatever. For 1-D histograms, JY is ignored but must be

supplied.

ODP$

(4)....SCANM uses non-resident global common GBLX. The maximum

histogram size is 1048576 16-bit channels.

ODP$

(5)....No protection against out-of-range indices, data, etc. is

afforded by the main package - you are on your own!

CYB$

SCANLNKM MYSUBS,SCANM       ;Links it

SCANM    HISFIL             ;Runs  it

RPD$070066001

SIN$

RSH$1010

CHP$SCANM - In-memory Histogramming (VAX Version)

DBL$

TPF$

BLP$

SCANM is an all in-memory histogramming package with the following

features and restrictions.

ODP$

(1)....A proper DRR-file and HIS-file must exist.

ODP$

(2)....The main program calls user-supplied routine HISSUB(IBUF,N)

with an INTEGER*2 event buffer IBUF

which may contain a raw buffer from

tape (default) or a single packed event

(requested by the CMD SEBU).

If a single event buffer is chosen, N is the number of words in

IBUF (the FFFF is not passed). For the raw tape buffer case, N is

the buffer length (normally 4096).

ODP$

(3)....Histogramming is accomplished by user-supplied routines via

calls to routine COUNT1(ID,IX,JY): where, ID is a histogram ID-number

in the range 1 to 8000. IX and JY are X- and Y-channel numbers in the

range 0 to whatever. For 1-D histograms, JY is ignored but must be

supplied.

ODP$

(4)....SCANM generates histograms in COMMON/HIS1/. The maximum

histogram size is 1048576 16-bit channels for the current dimension

of array INTEGER*4 MEMBUF(524288).

ODP$

(5)....No protection against out-of-range indices, data, etc. is

afforded by the main package - you are on your own!

CYB$

SCANLNKM MYSUBS SCANM       ;Links it

SCANM    HISFIL             ;Runs  it

RPD$070066001

SIN$

RSH$1010

CHP$SCANB - User-driven 2-phase Histogramming

DBL$

TPF$

BLP$

SCANB is a special 2-phase histogramming package in which all

histogram addresses are calculated by a user-supplied routine HISSUB.

The following features and restrictions apply:

ODP$

(1)....Proper DIR, MIL, and HIS-files must exist.

ODP$

(2)....The main program calls user-supplied routine HISSUB(IBUF,N)

with an INTEGER*2 event buffer IBUF

which may contain a raw buffer from

tape (default) or a single packed event

(requested by the CMD SEBU).

If a single event buffer is chosen, N is the number of words in

IBUF (the FFFF is not passed). For the raw tape buffer case, N is

the buffer length (normally 4096).

ODP$

(3)....Histogramming is accomplished by user-supplied routines via

calls to routine COUNT1(ID,IX,JY): where, ID is a histogram ID-number

in the range 1 to 8000. IX and JY are X- and Y-channel numbers in the

range 0 to whatever. For 1-D histograms, JY is ignored but must be

supplied.

ODP$

(4)....Unlike the standard CHIL-based SCAN package which gets

memory space, SCANB is characterized by a fixed maximum

size which is determined by the value of a parameter CLY.

The following values of CLY (defined in routines - MAIN, INITUM,

RESET, PHASE1 & PHASE2) are required for the sizes shown:

SIN$

CY1$

UP TO  1 MEGACHANNEL FOR PARAMETER (CLY=32)

UP TO  2 MEGACHANNEL FOR PARAMETER (CLY=64)

UP TO  4 MEGACHANNEL FOR PARAMETER (CLY=128)

UP TO  8 MEGACHANNEL FOR PARAMETER (CLY=256)

UP TO 16 MEGACHANNEL FOR PARAMETER (CLY=512)

UP TO 32 MEGACHANNEL FOR PARAMETER (CLY=1024)

ODP$

(5)....No protection against out-of-range indices, data, etc. is

afforded by the main package - you are on your own!

CY1$

SCANLNKB MYSUBS,SCANB       ;Links it

SCANB    HISFIL             ;Runs  it

RPD$070066001

SIN$

RSH$1010

CHP$VAXPAK Symbols - Easy Way to Define

TPF$

BLP$

The easiest way to define the full set of symbols (procedures) used

in VAXPAK (DAM, SAM, LEMO, LEMOLNK, etc.) is to include the following

entry in your LOGIN.COM:

CY1$

@VAXPAK:MILNAMES

----------------------------------------------------------------------

The symbols listed below are defined:

----------------------------------------------------------------------

ADAC      == "@U1:[MILNER]ADAC"

ASAP      == "@U1:[MILNER]ASAP"

ASSWIPE   == "@U1:[MILNER]ASSWIPE"

BANCO     == "@U1:[MILNER]BANCO"

CCF       == "@U1:[MILNER]CCF"

CHIL      == "@U1:[MILNER]CHIL"

DAM       == "@U1:[MILNER]DAM"

DAMS      == "@U1:[MILNER]DAMS"

DC        == "@U1:[MILNER]DC"

DDP       == "@U1:[MILNER]DDP"

DDT       == "@U1:[MILNER]DDT"

FUNKYFIT  == "@U1:[MILNER]FUNKYFIT"

KINEQ     == "@U1:[MILNER]KINEQ"

LEMO      == "@U1:[MILNER]LEMO"

LEMOLNK   == "@U1:[MILNER]LEMOLNK"

MILDO     == "@U1:[MILNER]MILDO"

PLO       == "@U1:[MILNER]PLO"

RUL       == "@U1:[MILNER]RUL"

SAM       == "@U1:[MILNER]SAM"

SAMB      == "@U1:[MILNER]SAMB"

SAMS      == "@U1:[MILNER]SAMS"

SCAN16    == "@U1:[MILNER]SCAN16"

SCAN4     == "@U1:[MILNER]SCAN4"

SCANLNK16 == "@U1:[MILNER]SCANLNK16"

SCANLNK4  == "@U1:[MILNER]SCANLNK4"

STOPI     == "@U1:[MILNER]STOPI"

STOPX     == "@U1:[MILNER]STOPX"

STP       == "@U1:[MILNER]STP"

STU       == "@U1:[MILNER]STU"

TDX       == "@U1:[MILNER]TDX"

TRIL      == "@U1:[BEENE]RSC"

TRIP      == "@U1:[MILNER]TRIP"

XAM       == "@U1:[MILNER]XAM"

----------------------------------------------------------------------

RPD$070066001

SIN$

RSH$1010

CHP$VAXstation-2000 for Public Use

TPF$

DBL$

CY1$

From:     W. T. Milner

To:       HHIRF Computer Users

Subject:  VAXstation-2000 use

BLP$

The clustered VAXstation-2000 located in the Concurrent room is

available for general use. Since only two users can be logged on

this machine at a given time, only local logins are recommended.

Some special features and limitations of this system are listed

below:

ODP$

(1)....DAMS & SAMS (VAXstation versions of DAM & SAM) can be run

CY1$

By typing:   @U1:[MILNER]DAMS

and          @U1:[MILNER]SAMS,   respectively.

ODP$

(2)....The display is pretty fast.

ODP$

(3)....The only graphic hard-copy is via the print-portion-of-screen

workstation command. See the Workstation Users Guide for details.

Printout is on the VAX-room Laser printer.

ODP$

(4)....See the attached supplementary notes on DAMS & SAMS.

ODP$

(5)....Refer to the MicroVMS Workstation User's Guide (little

black book) for general instructions on how to use the workstation.

RPD$070066001

SIN$

RSH$1010

CHP$DAMS & SAMS (Vaxstation DAM & SAM)

TPF$

CY1$

Section  Contents

010  Introduction

020  SAMS - Deficiencies

030  SAMS - Differences

040  SAMS - Comments

050  DAMS - Deficiencies

060  DAMS - Differences

070  DAMS - Graphics Screen Configurations

080  DAMS - Graphics Screen Color Mapping

090  DAMS - Comments on Bananas

100  General comments on Hard Copy

110  Implementation

SEC$010  Introduction

BLP$

SAMS & DAMS are Vaxstation versions of SAM & DAM. These programs

have been re-written to use the standard UIS graphics library

provided with the Vaxstation. I have done all development on a

Vaxstation-2000 (15 inch color monitor) but the programs should

work on Microvax-II/GPX's and the Vaxstation-3000 series, etc.

if they support color or grey-scale; but I have only tested the

software on the Vaxstation-2000.

SEC$020  SAMS - Deficiencies

ODP$

(1)....The GFIT fitting method has not been implemented.

ODP$

(2)....There is no programmable screen-copy (i.e. no HC OR TF

commands). It has to be done via

the "Print (Portion of) Screen" feature of the workstation.

SEC$030  SAMS - Differences

BLP$

The main difference between SAMS and the Tektronix version is the

screen-copy business mentioned above. You also have control over

the size and position of the graphic window. Two default sizes

have been provided - one sized for interactive work and one sized

for screen-copy.

CY1$

Type:  HELP WIND    ;For a description of related commands

SEC$040  SAMS - Comments

BLP$

SAMS works very much like the Tektronix version (SAM). Study the

HELP (note: the new peak-find feature) and you probably won't

have much trouble.

TPF$

SEC$050  DAMS - Deficiencies

ODP$

(1)....The ZCOL & ZS commands have not beem implemented.

ODP$

(2)....There is no programmable screen-copy (i.e. no HC command)

It has to be done via

the "Print (Portion of) Screen" feature of the workstation.

ODP$

(3)....The 1-D displays have no useful window banner labels

because I haven't decided what would be useful and possible.

ODP$

(4)....Output to the dialog window [KB] is not always sized

properly for 1:1 screen copy.

SEC$060  DAMS - Differences

BLP$

There are four main areas of difference between DAMS and

DAM (the Tektronix version).

ODP$

(1)....Program controlled screen-copy (the HC command) is not

available as described above.

ODP$

(2)....There is much more flexibility available for setting up

graphics windows.

ODP$

(3)....There is much more flexibility available for color or

grey scale mapping.

ODP$

(4)....The 1-D stacked mode has been implemented somewhat differently.

ODP$

(5)....The Vaxstation version is a hellofalot faster.

SEC$070  DAMS - Graphics Screen Configurations

BLP$

The sizes and locations of display windows are controlled by one

or more of the commands - FIG, FIGF or FIGI.

CY1$

FIG ID - Sets the number and size of display windows to that specified

by configuration number ID. A given configuration ID number

may specify up to 20 display windows. The default configuration

library contains ID-numbers 1 thru 16. Type: HELP FIG for a

display indicating the default sceen configurations or just

Type: FIG 1, FIG 2 ... FIG 16 and see what you get.

FIGF FILENAME - Requests that a new configuration library be read from

a file named FILENAME. The file DISK$USER1:[MILNER.VX5]FIG3.DAT

which contains the default configuration is listed on the next

next two pages. The **** in col-1 of the table denote comment

lines and are ignored in processing.

TPF$

CY1$

****SCREEN-CONFIGURATION TABLE FOR DAMS

****----------------------------------------------

****FIG-ID    X0(CM)    Y0(CM)     W(CM)     H(CM)

1       1.0      10.0      18.0      16.0

****

2       0.0      11.5      16.0      16.0

2      16.5      11.5      16.0      16.0

****

3       1.0       6.5      18.0      10.0

3       1.0      17.5      18.0      10.0

3      19.5      14.5      13.0      13.0

****

4       0.0       6.5      16.0      10.0

4       0.0      17.5      16.0      10.0

4      16.5       6.5      16.0      10.0

4      16.5      17.5      16.0      10.0

****

5       1.0       6.0      16.0       6.5

5       1.0      13.5      16.0       6.5

5       1.0      21.0      16.0       6.5

5      17.5      13.0      14.5      14.5

****

6       0.0       6.0      16.0       6.5

6       0.0      13.5      16.0       6.5

6       0.0      21.0      16.0       6.5

6      16.5       6.0      16.0       6.5

6      16.5      13.5      16.0       6.5

6      16.5      21.0      16.0       6.5

****

7       1.0       6.5      16.0       4.5

7       1.0      12.0      16.0       4.5

7       1.0      17.5      16.0       4.5

7       1.0      23.0      16.0       4.5

7      17.5      13.0      14.5      14.5

****

8       0.0       6.5      16.0       4.5

8       0.0      12.0      16.0       4.5

8       0.0      17.5      16.0       4.5

8       0.0      23.0      16.0       4.5

8      16.5       6.5      16.0       4.5

8      16.5      12.0      16.0       4.5

8      16.5      17.5      16.0       4.5

8      16.5      23.0      16.0       4.5

****

9       1.0       6.0      16.0       3.5

9       1.0      10.5      16.0       3.5

9       1.0      15.0      16.0       3.5

9       1.0      19.5      16.0       3.5

9       1.0      24.0      16.0       3.5

9      17.5      13.0      14.5      14.5

TPF$

CY1$

****

10       0.0       6.0      16.0       3.5

10       0.0      10.5      16.0       3.5

10       0.0      15.0      16.0       3.5

10       0.0      19.5      16.0       3.5

10       0.0      24.0      16.0       3.5

10      16.5       6.0      16.0       3.5

10      16.5      10.5      16.0       3.5

10      16.5      15.0      16.0       3.5

10      16.5      19.5      16.0       3.5

10      16.5      24.0      16.0       3.5

****

11      00.0      10.0      32.0      17.5

****

12       0.0       8.0      32.0       9.0

12       0.0      18.0      32.0       9.0

****

13       0.0       6.0      32.0       6.5

13       0.0      13.5      32.0       6.5

13       0.0      21.0      32.0       6.5

****

14       0.0       6.0      32.0       4.5

14       0.0      11.5      32.0       4.5

14       0.0      17.0      32.0       4.5

14       0.0      22.5      32.0       4.5

****

15       8.0       2.5      25.0      25.0

****

16       0.0       6.0      21.5      21.5

----------------------------------------------------------------------

FIG-ID = Configuration ID number to be associated with window.

X0(CM) = X-coordinate of lower left corner of window in cm.

Y0(CM) = Y-coordinate of lower left corner of window in cm.

W(CM)  = Width  of window in cm.

H(CM)  = Height of window in cm.

Note: If you have a 15-inch monitor, a cm will be about 15/19 times

the length of an actual cm.

----------------------------------------------------------------------

FIGI   Restores the configuration library to the default state.

TPF$

SEC$080  DAMS - Graphics Screen Color Mapping

BLP$

The colors (or grey scale) is controlled by means of a color map

which has 16 entries that specify (red, green, blue) intensities.

The default table used by DAMS (file CMAP1.DAT) is listed below:

CY1$

----------------------------------------------------------------------

0  0.00   0.00   0.00     ;  0 - WINDOW BGD

1  0.00   0.00   0.50     ;  1 - 0.5 BLUE

2  0.00   0.00   0.70     ;  2 - 0.7 BLUE

3  0.00   0.00   1.00     ;  3 - 1.0 BLUE

4  0.40   0.00   0.00     ;  4 - 0.4 RED

5  0.70   0.00   0.00     ;  5 - 0.7 RED

6  1.00   0.00   0.00     ;  6 - 1.0 RED

7  0.00   0.40   0.00     ;  7 - 0.4 GREEN

8  0.00   0.70   0.00     ;  8 - 0.7 GREEN

9  0.00   1.00   0.00     ;  9 - 1.0 GREEN

10  0.80   0.80   0.80     ; 10 - 0.8 WHITE

11  1.00   1.00   1.00     ; 11 - 1.0 WHITE

12  0.00   0.00   0.00     ; 12 - KEYBOARD BGD

13  1.00   1.00   1.00     ; 13 - KEYBOARD & WINDOW FOREGND

14  0.00   0.00   0.00     ; 14 - SCREEN BGD

15  1.00   1.00   0.00     ; 15 - BLACK COMPLIMENT COLOR

----------------------------------------------------------------------

BLP$

Different color mapping may be accomplished by the CMAP command as

shown below:

CY1$

CMAP 1          ;Maps from an internal table which contains the same

;data as CMAP1.DAT (this is the default mapping).

CMAP 2          ;Maps from another internal table which contains the

;same data as file CMAP2.DAT.

CMAP 3          ;Maps from another internal table which contains the

;same data as file CMAP3.DAT.

CMAP 4          ;Maps from another internal table which contains the

;same data as file GSMAP.DAT. This is a grey scale.

CMAP FILENAME  ;Processes a file FILENAME of the structure shown above

;and maps as specified therein.

BLP$

Display something like a 2-D histogram and type CMAP 1, CMAP 2, etc.

and see what happens.

TPF$

SEC$090  DAMS - Comments on Bananas

BLP$

In DAM an open banana is yellow and a closed banana is red. In

the case of DAMS, open and closed bananas are the same color

(yellow) but the closed banana has a line segment connecting the

first and last points (i.e. looks closed).

SEC$100  General Comments on Hard Copy

BLP$

For 1:1 screen printing, it is probably best to set up the workstation

such that:

CY1$

Color Conversion Method = Grey Shades

Ribbon or Toner Saver   = Negative Image

BLP$

It is possible to use the RV (reverse video) command and the positive

image mode but it is a bit more tricky and I don't suggest it.

BLP$

I have set up the display windows sized for 1:1 screen printing to

a LN03 laser printer (WINB in SAMS and FIG 1 in DAMS) with the

expectation that the [KB] screen will be located at the extreme lower

left corner of the workstation.

SEC$110  Implementation

BLP$

The VSPAK tape

that you receive will contain all of the files that I have

on my Vaxstation-2000, whether you want them or not - it is just a

lot easier for me to do that. Since I am still running 4.6 on the

Vaxstation, the EXEs may not work and you may have to re-link.

To do this, type the following:

CYB$

@SAMSLNK     ;To link SAMS

@DAMSLNK     ;To link DAMS

CEB$

Final Note

BLP$

Of course, you will probably need to change DISK and [DIRECTORY]

names in files like LOGIN.COM, DAMS.COM, SAMS.COM, DAMSLNK.COM,

SAMSLNK.COM, etc. The same sort of things you did for VAXPAK

implementation.

CY1$

Good Luck,

W. T. Milner

QIT$