RPD$070066001

CHP$250.  EVENT-HANDLER - WTM

RSH$0505

TPF$

SP1$

CEB$

DATA ACQUISITION WITH THE EVENT-HANDLER

(CONCURRENT CAMAC SYSTEM)

DBL$

CY1$

SECTION  CONTENTS

010  HOW TO ASSEMBLE A PROGRAM

020  GENERAL FEATURES OF THE SYSTEM

025  READOUT & CONTROL OF UP TO 4 CRATES WITH ONE EVENT HANDLER

030  BASIC INSTRUCTION LIST AND DEFINITIONS

040  MACRO INSTRUCTION LIST AND DEFINITIONS

050  DIRECTIVES TO THE ASSEMBLER ("EQUATE", LOOP, ENDLOOP)

060  ASSEMBLY-TIME VARIABLES

065  STATEMENT LABELS

070  EXPRESSIONS

080  SYNTAX RULES

090  PROGRAMMING EXAMPLES

110  HOW TO SCREW UP WITHOUT REALLY TRYING

120  INSTRUCTION SET (BIT-PATTERN)

SIN$

SEC$010  HOW TO ASSEMBLE A PROGRAM

BLP$

To assemble source file USER.EVS to produce an object file

USER.EVO, type:

CY1$

ADAC USER          - For assembly but no listing

ADAC USER,PR:      - For assembly and listing on printer

ADAC USER,FILDEV   - For assembly and listing on FILDEV

ADAC USER,PR:,FULL - For assembly and expanded printout

TPF$

CEB$

EVENT HANDLER FRONT PANEL

CY1$

-----------------------

!                     !

!                ()   !

RESET ----!--[ ]          [()]--!---- MASTER

BUTTON    !                     !     SWITCH

!                     !

!        INPUTS       !

!                     !

!  (1)    (2)    (3)  !

!  TTL    TTL    TTL  !

!                     !

!  (4)    (5)    (6)  !

!  TTL    TTL    TTL  !

!                     !

!    ((7))   ((8))    !

!     NIM     NIM

!                     !

!                     !

!   () - BUSY --------!----

!   () - STOP         !   !

!   () - WAIT         !   !FRONT PANEL

!   () - NAF          !   !LED'S

!   () - OUT          !   !

!   () - NO Q --------!----

!                     !

!                     !

!     ( )----(( ))----!---- EVENT

!                     !

INTERRUPT ----!----( )       (( ))--!---- HOLD

!                     !

!         ( )---------!---- BUSY

!                     !

!   (( ))-----(( ))---!---- BUSY

!                     !

!         ( ) +5      !

!                     !

!                     !

!       OUTPUTS       !

!                     !

!  (1)    (2)    (3)  !

! 50ohm  50ohm  50ohm !

!                     !

!  (4)    (5)    (6)  !

!  TTL    TTL    TTL  !

!                     !

!    ((7))   ((8))    !

!     NIM     NIM     !

!                     !

-----------------------

()  --- DENOTES LITTLE OL' LEDS

( ) --- DENOTES "LITTLE GOLD" CONNECTOR

(( ))--- DENOTES LEMO CONNECTOR

CHP$250.  EVENT-HANDLER - ROAD MAP - WTM

TPF$

CY1$

EXTERNAL OUTPUT REG (EXO)                  EXTERNAL INPUT REG (EX)

--------------------------------           ----------------------

[ 1] 50 OHM                  !---[ 1] TTL

SSET #    [ 2] 50 OHM                  !   [ 2] TTL

[ 3] 50 OHM   8 FRONT        !   [ 3] TTL      8 FRONT

SCLR #    [ 4] TTL      PANEL          !   [ 4] TTL      PANEL

[ 5] TTL      OUTPUTS        !   [ 5] TTL      INPUTS

SCMP #    [ 6] TTL                     !   [ 6] TTL

[ 7] NIM                     !   [ 7] NIM

LOAD #    [ 8] NIM                     !   [ 8] NIM

--------------------------------       !   ----------------------

!   [ 9] GOOD  Q

!   [10] EVENT BUSY

SKIP EX.NONE.#             !   [11] WAIT

SKIP EX.ANY.#              !   [12] STOP

---------------------------!   [13] NAF   BUSY

Tests 16-bits of EX or EX2 !   [14] OUT   BUSY

!   [15] EXO-4

!---[16] EXO-5

--------------------  ---------------------

!  16-BIT AUX MEM  !  ! 24 BIT MEMORY REG !

!  AD = 0 to 511   !  !        #          !

--------------------  ---------------------

!    !          !------------MOV  #,CAX (LO-16>LO-16)

!    !          !       !----MOV  #,TXR (LO-16>LO-16)

STOX AD --!    !          !       !  !-OUT  #     (LO-16>LO-16)

!    !          !       !  !-MERG #     (13-16>13-16)

RECX AD -------!          !       !  !            (01-12 same )

!    !          !       !  !

--------------------      !       !  !     ------------  ------

! MOV UCAX,CAX     !<-----!       !  !---->!----------!->!FIFO!

! moves HI 8-bits  !              !------->!          !  !    !

! to    LO 8-bits  !                       !          !  !    !

! zeros bits 9-16  !-OUT  CAX (24 BITS)--->!----------!->!    !

!------------------!                       !          !  !    !

NAF  !                  !-OUT UCAX (HI-8>LO-8)->!----------!->!    !

<----->! 24 BIT CAMAC REG !                       ! XFER REG !  !FIFO!

CNAF !        CAX       !-MOV CAX,TXR (24)----->!    TXR   !  !    !

! CAX = CA1 or CA2 !                       !          !  !    !

!                  !->!---MOV CAX,PAT      !          !  !    !

!------------------!  !                    !          !  !    !

! INCX incs   CAX  !<---!-MOV PAT,CAX   !->!--------->!->!    !

! DECX decs   CAX  !  ! !               !  ! XFER REG !  !    !

! SPLX splits CAX  !  ! !               !  !   TXR    !  !FIFO!

--------------------  ! !               !  ------------  ------

!       (24)! !(16)           !

SKIP CAX.NONE.#       ! !               !----- OUT PAT (16)

SKIP CAX.ANY.#        ! !               !

SKIP CAX.LT.#         ! !               !

SKIP CAX.GT.#        ----------------------    SKIP PAT.NONE.#

! 24 BIT PATTERN REG !--- SKIP PAT.ANY.#

Tests LO 16-bits     !        PAT         !    SKIP PAT.LT.#

of CAMAC REG (CAX)   ----------------------    SKIP PAT.GT.#

SKIP UPAT.NONE.#          Tests LO 16-bits

SKIP UPAT.ANY.#           of PAT

Tests HI 8-bits of PAT

CHP$250.  EVENT-HANDLER - WTM

TPF$

SEC$020  GENERAL FEATURES OF THE SYSTEM

ODP$

(1)....All data input to the system is through CAMAC -

either directly from CAMAC modules

or from non-CAMAC modules coupled through CAMAC input registers.

ODP$

(2)....All data are acquired in the Event-by-Event mode.

ODP$

(3)....The user must set up external logic to provide one (or possibly

more) EVENT logic pulse(s) for all events which are to be accepted

(or considered).

ODP$

(4)....A programmable micro-processor (Event-Handler) is used to read

the digital data associated with an event from multiple CAMAC

addresses and load it into a FIFO.

CEN$

SOME GENERAL CONSIDERATIONS FOR EVENT-HANDLER PROGRAMMING

BLP$

Although the main function of the Event-Handler is to read data associated

with an EVENT from the CAMAC dataway (possibly test it) and

load it into a FIFO, there are a number of other functions which

it must perform. Some of the most important of these are:

ODP$

(1)....Detect the occurrence of an EVENT. This may be accomplished

by testing bit-10 of the external input register: This bit is set

automatically by a strobe to one of the EVENT inputs on the front

pannel or by the SETB command.

ODP$

(2)....Detect the fact that the FIFO is 3/4 or more full by testing

bit-11 (WAIT BIT) of the EXT-REG and discontinue loading of the

FIFO until the "Too-Full" condition is cleared up.

ODP$

(3)....Detect the fact that the host computer (CONCURRENT) is telling you

to stop "stuffing the FIFO" by testing bit-12 (STOP BIT) of the EXT-REG

and discontinue the acquisition of data until this condition is

cleared up.

ODP$

(4)....Tell the outside world (experiment) that the system is busy when

you wish to discontinue acquiring data. This is accomplished

by the SETB (Set Busy) command which sets a level at the three

BUSY output connectors on the front pannel.

The experiment hardware must, of course, sense this busy and

disable the acquisition.

TPF$

SP3$

SEC$025  READOUT & CONTROL OF UP TO 4 CRATES WITH ONE EVENT HANDLER

BLP$

Jim McConnell has modified all locatable Event Handlers and AUXs so

as to support the control and readout of up to four CAMAC crates.

Since there is only one crate address bit available in the Event

Handler instruction word, a BANK switching technique which involves

both new hardware and automatically inserted BANK setting instructions

has been implemented. Programmers who don't use SPB instructions

don't have to be concerned

with the BANK setting except to realize that setting the BANK (0 for

crates 1 & 2 and 1 for crates 3 & 4) requires one instruction and

takes about 500 nanoseconds. \b\Those who use SPBs must read the

paragraph below - BANK Setting With SPBs.\\

ADAC only inserts BANK setting

instructions where required so it is best to read groups of ADCs,

TDCs, etc. from crates in the same BANK rather than reading alternately

from, say crates 1 & 3 or 1 & 4 or 2 & 3 or 2 & 4. Crates are

referenced in the Event Handler program as 1,2,3,4 (i.e. just an

extension of the original 1,2 reference scheme). See the SEC# 250.30

and SEC# 250.40 for complete instruction definitions.

CEB$

BANK Setting With SPBs

BLP$

If an \b\SPB\\ is used to execute a section of code which may do

CAMAC or BANK dependant operations,

then on return from the \b\SPB\\ the BANK may be incorrectly set

for operations following the \b\SPB\\.

It would be somewhat difficult (and probably not

worth the effort) to take care of this problem automatically with ADAC.

Therefore, instructions for manually setting BANKS 0 and 1

\b\(BNK0 and BNK1)\\ have been added. The appropriate one of these should

be used following any \b\SPB\\ which can cause the BANK to be set incorrectly

for operations following the \b\SPB\\.  Note the following example:

CY1$

C1=1              ;CRATE-1

C2=2              ;CRATE-2

C3=3              ;CRATE-3

C4=4              ;CRATE-4

N =10             ;SLOT-10

A =1              ;ADDRESS-1

CROUT C1,N,A,0    ;READ FROM CRATE-1 (BANK-0, DEFAULT)

CROUT C3,N,A,0    ;READ FROM CRATE-3 (BANK-1, SET AUTOMATICALLY)

SPB   READ2       ;READ FROM CRATE-2 VIA SPB

BNK1              ;MANUALLY SET BANK-1 FOR FOLLOWING OPERATION

CROUT C4,N,A,0    ;READ FROM CRATE-4 (BANK-1, SET MANUALLY)

....

....

READ2  CROUT C2,N,A,0    ;READ FROM CRATE-2 (BANK-0, SET AUTOMATICALLY)

....

....

BRUR              ;RETURN FROM SPB

TPF$

SEC$030  BASIC INSTRUCTION LIST AND DEFINITIONS

CY1$

INSTRUCTION           ;MEANING OR DESCRIPTION

NOP                   ;NO-OP

SETB                  ;SET BUSY - front panel output

CLRB                  ;CLR BUSY - front panel output

SSET  #               ;Selective set        External Output Reg (EXO)

SCLR  #               ;Selective clear      External Output Reg (EXO)

SCMP  #               ;Selective compliment External Output Reg (EXO)

LOAD  #               ;Load # (8 bits) into External Output Reg (EXO)

;* * * LOAD REPLACES ALL BITS! * * *

NAF   N,A,F           ;Do NAF  - data to or from CA1 implied

CNAF  C,N,A,F         ;Do CNAF - (C=1,2,3,4)-data to/from CA1,CA2,CA3,CA4

NAF  (P),N,A,F        ;Do NAF  - (proceed)

CNAF (P),C,N,A,F      ;Do CNAF - (proceed)

NAF  (S),N,A,F        ;Do NAF  - (short   CAMAC cycle - No S2)

CNAF (S),C,N,A,F      ;Do CNAF - (short   CAMAC cycle - No S2)

NAF  (N),N,A,F        ;Do NAF  - (shorter CAMAC cycle - No S1 or S2)

CNAF (N),C,N,A,F      ;Do CNAF - (shorter CAMAC cycle - No S1 or S2)

NAF  (PS),N,A,F       ;Do NAF  - (short   CAMAC cycle & proceed)

CNAF (PS),C,N,A,F     ;Do CNAF - (short   CAMAC cycle & proceed)

NAF  (PN),N,A,F       ;Do NAF  - (shorter CAMAC cycle & proceed)

CNAF (PN),C,N,A,F     ;Do CNAF - (shorter CAMAC cycle & proceed)

MOV  CAX,PAT          ;MOV contents of CAX to Pattern  Reg (PAT)

MOV  PAT,CAX          ;MOV contents of PAT to CAX (16 bits)

MOV  CAX,TXR          ;MOV contents of CAX to Transfer Reg (TXR)

MOV  UCAX,CAX         ;MOV HI 8-bits to LO 8-bits (zero bits 9 - 16)

MOV  #,TXR            ;MOV # to TXR (16 bits)

MOV  #,CAX            ;MOV # to CAX (16 bits)

OUT  CAX              ;Output CAX (MOV to TXR & XMIT to FIFO)(24 bits)

OUT  #                ;Output #   (MOV to TXR & XMIT to FIFO)(16 bits)

OUT  UCAX             ;MOV HI 8-bits to LO 8-bits of TXR & XMIT

;(HI 8-bits of TXR set to zero)

OUT  PAT              ;MOV LO 16-bits of PAT to TXR & XMIT

OUT  SPEC             ;Special OUT - not yet supported.

MERG #                ;MOV bits 13-16 of # to bits 13-16 of TXR & XMIT

;    (bits 1-12 unchanged)

INCX                  ;Increment CAMAC register CAX

DECX                  ;Decrement CAMAC register CAX

SPLX                  ;"Split"   CAMAC register CAX

STOX AD               ;Store CAX in AUX memory at address AD

RECX AD               ;Recall AUX memory location AD & store in CAX

----------------------------------------------------------------------

CAX  denotes CA  (implies CA1),  CA1,  CA2,  CA3  or CA4

INCX denotes INC (implies INC1), INC1, INC2, INC3 or INC4

DECX denotes DEC (implies DEC1), DEC1, DEC2, DEC3 or DEC4

SPLX denotes SPL (implies SPL1), SPL1, SPL2, SPL3 or SPL4

All NAF & CNAF instructions support an optional "last field"  T  which

denotes a Totalizer Module register no. (T = 0,1,2,3).

----------------------------------------------------------------------

TPF$

SEC$030  BASIC INSTRUCTION LIST (CONTINUED)

CY1$

SKIP EXX.ANY.#      ;Skip next INS if any set bits in EXX  & # match

SKIP CAX.ANY.#      ;Skip next INS if any set bits in CAX  & # match

SKIP PAT.ANY.#      ;Skip next INS if any set bits in PAT  & # match

SKIP UPAT.ANY.#     ;Skip next INS if any set bits in UPAT & # match

SKIP EXX.NONE.#     ;Skip next INS if no  set bits in EXX  & # match

SKIP CAX.NONE.#     ;Skip next INS if no  set bits in CAX  & # match

SKIP PAT.NONE.#     ;Skip next INS if no  set bits in PAT  & # match

SKIP UPAT.NONE.#    ;Skip next INS if no  set bits in UPAT & # match

SKIP CAX.LT.#       ;Skip next INS if contents of CAX < # (16 bits)

SKIP PAT.LT.#       ;Skip next INS if contents of PAT < # (16 bits)

SKIP CAX.GT.#       ;Skip next INS if contents of CAX > # (16 bits)

SKIP PAT.GT.#       ;Skip next INS if contents of PAT > # (16 bits)

BRU   DEST          ;Branch to Destination - Dest may be of form:

;/ # / @+# / @-# / NAME / NAME+# / NAME-# /

I, highly recommend that you use only the form

BRU  NAME. Otherwise, you can really screw

up if you don't know exactly what you are doing.

SPB   DEST          ;SPB = STORE POSITION and BRANCH - Similar to

;CALL in Fortran. DEST same form as for BRU.

BRUR                ;Return from subroutine

DLAY  #             ;Wait for #*(0.1 micro-sec) up to 409.6 micro-sec

BNK0                ;Set BANK-0 (for CA1 or CA2, etc operations)

BNK1                ;Set BANK-1 (for CA3 or CA4, etc operations)

----------------------------------------------------------------------

EXX denotes EX (implies EX1), EX1, EX2, EX3 or EX4

CAX denotes CA (implies CA1), CA1, CA2, CA3 or CA4

----------------------------------------------------------------------

TPF$

SEC$040  MACRO INSTRUCTION LIST AND DEFINITIONS

CY1$

IF(EXX.ANY.#)ADDR     ;If set bits in EXX  match any  in #, GO TO ADDR

IF(CAX.ANY.#)ADDR     ;If set bits in CAX  match any  in #, GO TO ADDR

IF(PAT.ANY.#)ADDR     ;If set bits in PAT  match any  in #, GO TO ADDR

IF(UPAT.ANY.#)ADDR    ;If set bits in UPAT match any  in #, GO TO ADDR

IF(EXX.NONE.#)ADDR    ;If set bits in EXX  match none in #, GO TO ADDR

IF(CAX.NONE.#)ADDR    ;If set bits in CAX  match none in #, GO TO ADDR

IF(PAT.NONE.#)ADDR    ;If set bits in PAT  match none in #, GO TO ADDR

IF(UPAT.NONE.#)ADDR   ;If set bits in UPAT match none in #, GO TO ADDR

IF(CAX.LT.#)ADDR      ;If contents of CAX < # (16 bits) GO TO ADDR

IF(PAT.LT.#)ADDR      ;If contents of PAT < # (16 bits) GO TO ADDR

IF(CAX.GT.#)ADDR      ;If contents of CAX > # (16 bits) GO TO ADDR

IF(PAT.GT.#)ADDR      ;If contents of PAT > # (16 bits) GO TO ADDR

SPAT  N,A,F           ;Read 24-bit WD and save in Pattern Reg (PAT)

CSPAT C,N,A,F         ;Read 24-bit WD and save in Pattern Reg (PAT)

ROUT  N,A,F           ;Read data and XMIT to FIFO

CROUT C,N,A,F         ;Read data and XMIT to FIFO

RSTO  N,A,F,AD        ;Read into CA1 & store LO-16 in AUX memory at AD

CRSTO C,N,A,F,AD      ;Read into CAC & store LO-16 in AUX memory at AD

INCX  AD              ;Increment AUX memory AD using CAMAC reg CAX

DECX  AD              ;Decrement AUX memory AD using CAMAC reg CAX

----------------------------------------------------------------------

EXX  denotes EX  (implies EX1),  EX1,  EX2,  EX3  or EX4

CAX  denotes CA  (implies CA1),  CA1,  CA2,  CA3  or CA4

INCX denotes INC (implies INC1), INC1, INC2, INC3 or INC4

DECX denotes DEC (implies DEC1), DEC1, DEC2, DEC3 or DEC4

All NAF & CNAF generating instructions support an optional first-field

(S), (P), (N), (PS), or (PN). (See first page of SEC# 250.030)

All NAF & CNAF generating instructions support an optional last-field

T which denotes a Totalizer Module register no. (T = 0,1,2,3)

----------------------------------------------------------------------

CEN$

(See next page for list of MACRO expansions)

TPF$

SEC$040  MACRO INSTRUCTION LIST (EXPANSIONS)

BLP$

Each macro-instruction generates two or more basic instructions

as indicated below.

CY1$

MACRO-INSTRUCTION      BASIC INSTRUCTIONS GENERATED BY ASSEMBLER

-----------------      -----------------------------------------------

SPAT   N,A,F           NAF   N,A,F

MOV   CA1,PAT

CSPAT  C,N,A,F         CNAF  C,N,A,F       (C must be 1,2,3 or 4)

MOV   CAX,PAT       (CAX=CA1,CA2,CA3 or CA4)

ROUT   N,A,F           NAF   N,A,F

OUT   CA1

CROUT  C,N,A,F         CNAF  C,N,A,F       (C must be 1,2,3 or 4)

OUT   CAX           (CAX=CA1,CA2,CA3 or CA4)

RSTO   N,A,F,AD        NAF   N,A,F

STO   AD

CRSTO  C,N,A,F,AD      CNAF  C,N,A,F       (C must be 1,2,3 or 4)

STOX  AD            (STOX=STO1,STO2,STO3 or STO4)

INCX   AD              RECX  AD            (uses CAMAC reg CAX)

INCX                (incremented value in CAX)

STOX  AD

DECX   AD              RECX  AD            (uses CAMAC reg CAX)

DECX                (decremented value in CAX)

STOX  AD

IF(REG.OP.MSK)ADDR     SKIP  REG.IOP.MSK   (for OP = ANY ,NONE)

BRU   ADDR          (    IOP= NONE,ANY )

IF(REG.LT.VAL)ADDR     SKIP  REG.GT.VAL-1  (REG=CA1,CA2,CA3,CA4 or PAT)

BRU   ADDR

IF(REG.GT.VAL)ADDR     SKIP  REG.LT.VAL+1  (REG=CA1,CA2,CA3,CA4 or PAT)

BRU   ADDR

----------------------------------------------------------------------

CAX  denotes CA  (implies CA1),  CA1,  CA2,  CA3  or CA4

INCX denotes INC (implies INC1), INC1, INC2, INC3 or INC4

DECX denotes DEC (implies DEC1), DEC1, DEC2, DEC3 or DEC4

All NAF & CNAF generating instructions support an optional first-field

(S), (P), (N), (PS), or (PN). (See first page of SEC# 250.030)

All NAF & CNAF generating instructions support an optional last-field

T which denotes a Totalizer Module register no. (T = 0,1,2,3)

----------------------------------------------------------------------

TPF$

SEC$050  DIRECTIVES TO THE ASSEMBLER ("EQUATE", LOOP, ENDLOOP)

CY1$

SYM=EXPRESSION          Directs assembler to evaluate EXPRESSION

and assign the resulting value to SYM.

LABL  LOOP    NLOOP     Directs the assembler to "replicate" the set

of statements bounded by the LOOP- and

ENDLOOP-directives. Do it NLOOP times.

ENDL  ENDLOOP           Defines End-of-Loop.

SEC$060  ASSEMBLY-TIME VARIABLES

BLP$

Symbols defined by the "Equate Directive" (SYM=EXPRESSION)

are called assembly time

variables or just assembly variables.

Such variables may be re-defined without restriction. Note: These are

not run-time variables! - THERE ARE NO RUN-TIME VARIABLES!

Assembly variables must always be defined in terms of numbers

and/or previously defined assembly variables.

Defining expressions may include "Bit-Lists".

SEC$065  STATEMENT LABELS

BLP$

The label on the ENDLOOP directive as well as any other labels

Defined between the LOOP and ENDLOOP directives are undefined

outside of the loop and may be used in other loops.

All other statement labels, including those on the LOOP directive

Are defined to all parts of the program and, therefore, must be

unique.

CEB$

* * * * LABELING OF THE "EQUATE" DIRECTIVE IS NOT ALLOWED * * * *

TPF$

SEC$070  EXPRESSIONS

BLP$

The assembler supports simple expressions which are evaluated left

to right. REPEAT!! EVALUATED! LEFT! TO! RIGHT!

Let "V" represent a single value (number or previously defined symbol).

Let "S" represent an algebraic sum of "V's".

Expressions of the following type are legal:

CY1$

A=V

A=S

A=S*V+S                Means: A=(S)*V+S

A=S/V+S                Means: A=(S)/V+S

A=S/V*V+S              Means: A=((S)/V)*V+S

A=MOD(S,S)             No additional terms allowed

(same argument definition as in fortran)

A=S+[S,S,..]           Where [ ] encloses a list of "bits"

DBL$

CY1$

NUMERICAL EXAMPLES:

ASSIGNMENT             RESULT

A=10                   A=10

B=A+4                  B=14

C=A+B-9                C=15

D=100/B                D=7

E=A+B+C/6              E=6

F=A+B-C*7              F=63

G=A+B-C/6*A            G=10

H=A+B-C/6*A+C+10       H=35

I=MOD(A+B,7)           I=3

J=MOD(A+B,E-1)         J=4

K=[A+6,3]              K=8004 (HEX)

L=[A+6,3]+40H          L=8044 (HEX)

M=[2,1]                M=3

N=[A+6,M]              N=8004 (HEX)

CEB$

* * * PARENTHESES ARE NOT ALLOWED EXCEPT IN THE "MOD STATEMENT" * * *

SIN$

TPF$

SEC$080  SYNTAX RULES

ODP$

(1)....All statement labels must start in col-1 and be no greater than

8 characters in length.

ODP$

(2)....All instructions (Basic and Macro) must start after col-1. At

least one blank must separate any statement label and the

instruction field.

ODP$

(3)....At least one blank must separate the instruction and operand

fields.

ODP$

(4)....Imbedded blanks are allowed in (but removed from) the IF-MACRO,

expressions and operand fields of all types. I.e. you can type it

any way you wish but I will re-format it to be the way I like it.

ODP$

(5)....The symbol being defined by an EQUATE may start in any column

and be up to 8 characters in length.

ODP$

(6)....All comment fields must be preceded by a semicolon.

ODP$

(7)....A completely blank line will show up as a blank line in the

assembly listing and otherwise be ignored.

ODP$

(8)....All numbers are decimal by default.

ODP$

(9)....Hex numbers are specified by a trailing "H" (8000H for example)

The first character must be a decimal integer (0 to 9)

ODP$

(10)..."Bit Expressions" are lists of bit-numbers enclosed in [ ].

Example: [16,1,2] produces 8003H.

ODP$

(11)...Bit-Lists are not allowed in elements of a N,A,F or C,N,A,F

ODP$

(12)...Expressions enclosed by < > within a comment-field are evaluated

and replaced by a 3-digit decimal integer in the assembly listing.

this is for labeling purposes only: any errors will be ignored.

CEB$

* * * EXPRESSIONS INCLOSED BY < > MAY CONTAIN NO IMBEDDED BLANKS * * *

TPF$

SEC$090  PROGRAMMING EXAMPLES

BLP$

Loops are initiated by the LOOP directive and terminated by the

ENDLOOP (equivalent to CONTINUE in fortran).

An example is shown below:

CY1$

SPAT 1,1,0              ;READ & STORE PATTERN REGISTER

B=0                     ;INIT PATTERN REG BIT COUNTER

AA=-1                   ;INIT ADC SUB-ADDR CNTR

AT=-1                   ;INIT TDC SUB-ADDR CNTR

DET=0                   ;INIT DETECTOR COUNTER

ID=-2                   ;INIT PARM ID #

NADC=10                 ;SLOT FOR ADC (ADC IN 10 & 11)

NTDC1=12                ;SLOT FOR TDC1 (TDC1 IN 12 & 13)

NTDC2=7                 ;SLOT FOR TDC2 (TDC2 IN 7  &  9)

DSA=1                   ;SLOT INC FOR ADC

DST1=1                  ;SLOT INC FOR TDC1

DST2=2                  ;SLOT INC FOR TDC2

CA=-1                   ;CNTR FOR ADC  SLOT SWITCHING

CT1=-1                  ;CNTR FOR TDC1 SLOT SWITCHING

CT2=-1                  ;CNTR FOR TDC2 SLOT SWITCHING

LU1   LOOP 16

B=B+1

AA=MOD(AA+1,12)

AT=MOD(AT+1,8)

DET=DET+1

ID=ID+3

CA=CA+1

CT1=CT1+1

CT2=CT2+1

NA=CA/12*DSA+NADC

NT1=CT1/8*DST1+NTDC1

NT2=CT2/8*DST2+NTDC2

IF(PAT.NONE.[B])LEND    ;TEST PATTERN REG BIT <B>

OUT 8000H+ID            ;OUTPUT PARAMETER ID <ID>

ROUT NA,AA,0            ;N,A  = <NA>,<AA>

ROUT NT1,AT,0           ;N,A = <NT1>,<AT>

ROUT NT2,AT,0           ;N,A = <NT2>,<AT>

LEND  ENDLOOP                 ;END OF LOOP

CEB$

* * * * NESTED LOOPS ARE NOT SUPPORTED * * * *

TPF$

SEC$090  PROGRAMMING EXAMPLES (continued)

CY1$

TITLE LINE MUST BE PRESENT

CSYS=2    ;SYSTEM (BRANCH) CRATE-#

CAUX=08   ;CRATE-# CONTAINING EVENT HANDLER

NFIF=18   ;SLOT-# FOR FIFO

RECL=8192 ;MAG TAPE RECORD LENGTH (BYTES)

NPAR=34   ;# OF PARAMETERS (FOR YOUR INFO ONLY)

NAUX=22   ;SLOT-# FOR EVENT-HANDLER AUX

NSUC=19   ;SLOT-# FOR SUCK-CONTROLLER

BPI=800   ;TAPE DENSITY (BITS PER INCH)

FORM=L002 ;L002 FORMAT (FOR INFO ONLY)

COM$

EXAMPLE EVENT-HANDLER PROGRAM

PGM$

S=12                    ;STOP BIT = 12

W=11                    ;WAIT BIT = 11

E=10                    ;EVENT BIT = 10

M=1                     ;MASTER BIT IN GATED LATCH

NAD1=5                  ;ADC1 IN SLOT 5

NLAT=2                  ;GATED LATCH IN SLOT 2

SETB                    ;SET BUSY TO INIT CAMAC

INIT  IF(EX.ANY.[S,W])INIT    ;WAIT FOR EXT HANGUPS TO END

CLR   NAF NAD1,12,11          ;CLEAR ADC

NAF NLAT,0,9            ;CLEAR LATCH

WAIT  IF(EX.ANY.[S,W])WAIT    ;WAIT FOR EXT STOPS AND WAIT STATES

CLRB                    ; * GO *

IDLE  IF(EX.NONE.[S,W,E])IDLE ;LOOP UNTIL SOMETHING HAPPENS

IF(EX.ANY.[S,W])STOP    ;STOP OR WAIT

IF(EX.NONE.[E])IDLE     ;? - WELL , GO BACK AND TRY AGAIN

SETB                    ;PREPARE TO READ OUT

DLAY 1000               ;WAIT FOR CONVERSIONS

SPAT NLAT,0,0           ;GET GATED LATCH SIGNAL

IF(PAT.NONE.[M])CLR     ;IF MASTER COUNTER DID NOT FIRE,IGNORE

OUT  8001H              ;EVENT HEADER

OUT  CA                 ;LATCH TO FIFO

A=-1                    ;SET UP ADC READ LOOP

LOOP 8

A=A+1                   ;INC ADDRESS

ROUT NAD1,A,0           ;ADC TO FIFO

ENDLOOP

OUT 0FFFFH              ;END OF EVENT

BRU CLR                 ;GO AND RESET ADCS ETC

STOP  SETB                    ;STOP - SET BUSY

BRU WAIT                ;AND GO INTO WAIT STATE

END

TPF$

SEC$110  HOW TO SCREW UP WITHOUT REALLY TRYING

ODP$

(1)....Use the BRU @+-# instruction without taking into account the

fact that macros (SPAT, ROUT, CROUT, IF) and LOOPS generate more

than one instruction. Fix: DON'T USE "BRU @+-#" SYNTAX.!!!

ODP$

(2)....Use the SKIP instruction just preceding a MACRO or LOOP.

It will likely screw up for the reasons given in (1). Fix: DON'T

USE THE SKIP INSTRUCTION PRECEDING A MACRO OR LOOP.

ODP$

(3)....Failure to recognize that the "LOAD" instruction replaces bits

1 through 8 of the EX REG. Fix: Recognize that fact.

ODP$

(4)....Failure to remember that expressions are evaluated Left-to-Right.

Fix: Remember That!!

ODP$

(5)....Use the (P), (S), (N), (PS), (PN) forms of NAF of CNAF

preceding anything but an OUT instruction without checking with

Dave Hensley first.

Fix: Check with Dave before you try to get fast or fancy.

ODP$

(6)....Be happy-go-lucky about the use of Bit-Lists and other numbers

in the specification of masks etc.

CY1$

**********************************************************************

BLP$

In order to avoid confusion,

I suggest that symbol definitions (equates) always be made without

the use of Bit-Lists and

that masks always be specified completely in terms of bit

lists. This includes the operand field of the instructions - SSET,

SCLR, SCMP and LOAD.

BLP$

Although I always try to give a man enough room to screw up, I

personally believe that programming should be done in a fairly

uniform and conservative style.

CY1$

**********************************************************************

TPF$

SEC$120  BASIC INSTRUCTION LIST (BIT PATTERN)

CY1$

!2222!2111!1111!1110!0000!0000!

!4321!0987!6543!2109!8765!4321! HEX REPRESENTATION

------------------!----!----!----!----!----!----!---------------------

NOP               !0000!0000!    !    !    !    ! 0

!    !    !    !    !    !    !

NAF     N,A,F<,T> !0001!00TT!00NN!NNNA!AAAF!FFFF! 100000+N,A,F,T

!    !    !    !    !    !    !

NAF  (P)N,A,F<,T> !0001!10TT!00NN!NNNA!AAAF!FFFF! 180000+N,A,F,T

!    !    !    !    !    !    !

NAF  (S)N,A,F<,T> !0001!00TT!01NN!NNNA!AAAF!FFFF! 104000+N,A,F,T

!    !    !    !    !    !    !

NAF  (N)N,A,F<,T> !0001!00TT!10NN!NNNA!AAAF!FFFF! 108000+N,A,F,T

!    !    !    !    !    !    !

NAF (PS)N,A,F<,T> !0001!10TT!01NN!NNNA!AAAF!FFFF! 184000+N,A,F,T

!    !    !    !    !    !    !

NAF (PN)N,A,F<,T> !0001!10TT!10NN!NNNA!AAAF!FFFF! 188000+N,A,F,T

!    !    !    !    !    !    !

BRU   A           !0010!0101!0   !0AAA!AAAA!AAAA! 250000+A

!    !    !    !    !    !    !

BRUR              !0010!0000!0   !    !    !    ! 200000

!    !    !    !    !    !    !

SPB   A           !0010!1101!0   !0AAA!AAAA!AAAA! 2D0000+A

!    !    !    !    !    !    !

SPBR              !0010!1000!0   !    !    !    ! 280000

!    !    !    !    !    !    !

INTE  A           !0010!1111!0   !0AAA!AAAA!AAAA! 2F0000+A

!    !    !    !    !    !    !

INTR              !0010!1010!    !    !    !    ! 2A0000

!    !    !    !    !    !    !

SKIP  PAT.ANY.#   !0011!0000!####!####!####!####! 300000+#

!    !    !    !    !    !    !

SKIP UPAT.ANY.#   !0011!0001!0000!0000!####!####! 310000+#

!    !    !    !    !    !    !

SKIP  EXX.ANY.#   !0011!0010!####!####!####!####! 320000+#

!    !    !    !    !    !    !

SKIP  CAX.ANY.#   !0011!0011!####!####!####!####! 330000+#

!    !    !    !    !    !    !

SKIP  PAT.NONE.#  !0011!0100!####!####!####!####! 340000+#

!    !    !    !    !    !    !

SKIP UPAT.NONE.#  !0011!0101!0000!0000!####!####! 350000+#

!    !    !    !    !    !    !

SKIP  EXX.NONE.#  !0011!0110!####!####!####!####! 360000+#

!    !    !    !    !    !    !

SKIP  CAX.NONE.#  !0011!0111!####!####!####!####! 370000+#

!    !    !    !    !    !    !

SKIP  CAX.LT.#    !0011!1011!####!####!####!####! 3B0000+#

!    !    !    !    !    !    !

SKIP  CAX.GT.#    !0011!1111!####!####!####!####! 3F0000+#

!    !    !    !    !    !    !

SKIP  PAT.LT.#    !0011!1000!####!####!####!####! 380000+#

!    !    !    !    !    !    !

SKIP  PAT.GT.#    !0011!1100!####!####!####!####! 3C0000+#

----------------------------------------------------------------------

The optional T-field in NAF & CNAF is supported by NEW-DEAL AUX only

TPF$

SEC$120  BASIC INSTRUCTION LIST (BIT PATTERN) (CONTINUED)

CY1$

!2222!2111!1111!1110!0000!0000!

!4321!0987!6543!2109!8765!4321! HEX REPRESENTATION

----------------  !----!----!----!----!----!----!-----------------------

SSET  #           !0100!0011!1111!1111!####!####! 43FF00+#

!    !    !    !    !    !    !

LOAD  #           !0100!0011!####!####!####!####! 430000+257*#

!    !    !    !    !    !    !

SCLR  #           !0100!0011!$$$$!$$$$!0000!0000! 430000+256*$

!    !    !    !    !    !    !

SCMP  #           !0100!0011!$$$$!$$$$!####!####! 430000+256*$+#

!    !    !    !    !    !    !

!    !    !    !    !    !    !

MOV     #,CAX     !0100!0000!####!####!####!####! 400000+#

!    !    !    !    !    !    !

MOV     #,TXR     !0101!0000!####!####!####!####! 500000+#

!    !    !    !    !    !    !

MOV   CAX,PAT     !0100!0001!    !    !    !    ! 410000

!    !    !    !    !    !    !

MOV   PAT,CAX     !0100!0010!0   !    !    !    ! 420000

!    !    !    !    !    !    !

MOV   CAX,TXR     !0101!0001!    !    !    !    ! 510000

!    !    !    !    !    !    !

MOV  UCAX,CAX     !0100!0010!1000!1   !    !    ! 428000

!    !    !    !    !    !    !

!    !    !    !    !    !    !

MERG  #           !0101!1100!####!    !    !    ! 5C0000+4096*#

!    !    !    !    !    !    !

OUT   #           !0101!1000!####!####!####!####! 580000+#

!    !    !    !    !    !    !

OUT   CAX         !0101!1001!    !    !    !    ! 590000

!    !    !    !    !    !    !

OUT   PAT         !0101!1010!    !    !    !    ! 5A0000

!    !    !    !    !    !    !

OUT   UCAX        !0101!1010!1   !    !    !    ! 5A8000

!    !    !    !    !    !    !

!    !    !    !    !    !    !

DLAY  #           !0110!0000!    !@@@@!@@@@!@@@@! 60F000+@

!    !    !    !    !    !    !

CLRB              !0111!0000!    !    !    !    ! 700000

!    !    !    !    !    !    !

SETB              !0111!0001!    !    !    !    ! 710000

!    !    !    !    !    !    !

BNK0              !0111!0010!    !    !    !    ! 720000

!    !    !    !    !    !    !

BNK1              !0111!0011!    !    !    !    ! 730000

----------------------------------------------------------------------

(1) $ Denotes compliment of #

(2) CAX denotes: CA, CA1, CA2, CA3, CA4 (CA is same as CA1)

(3) EXX Denotes: EX, EX1, EX2, EX3, EX4 (EX is same as EX1)

(EX2,EX3,EX4 used to check "Q" or "NAF BUSY" for CAMAC AUX-2,3,4)

(4) CNAF takes same forms as NAF except that C must be given.

(5) EX2,CA2,EX4,CA4 and C=2 or C=4 result in bit-24 being set

(6) @@@@@ Denotes 2's compliment of ##### (i.e. negative)

----------------------------------------------------------------------

\B\(See following page for more NEW-DEAL AUX instructions)\\

TPF$

SEC$120  BASIC INSTRUCTION LIST (BIT PATTERN) (NEW-DEAL AUX ONLY)

CY1$

!2222!2111!1111!1110!0000!0000!

!4321!0987!6543!2109!8765!4321! HEX REPRESENTATION

------------------!----!----!----!----!----!----!---------------------

SPLX              !0001!0000!0000!0000!0000!0000! 100000

!    !    !    !    !    !    !

INCX              !0001!0000!0011!1100!0000!0000! 103C00

!    !    !    !    !    !    !

DECX              !0001!0000!0011!1110!0000!0000! 103E00

!    !    !    !    !    !    !

STOX M            !0001!0000!0011!100M!MMMM!MMMM! 103800+M

!    !    !    !    !    !    !

RECX M            !0001!0000!0011!101M!MMMM!MMMM! 103A00+M

!    !    !    !    !    !    !

----------------------------------------------------------------------

SPLX,INCX,DECX,STOX,RECX  denotes  SPL, INC, DEC, STO, REC  (uses CA1)

or  SPL1,INC1,DEC1,STO1,REC1 (uses CA1)

or  SPL2,INC2,DEC2,STO2,REC2 (uses CA2)

or  SPL3,INC3,DEC3,STO3,REC3 (uses CA3)

or  SPL4,INC4,DEC4,STO4,REC4 (uses CA4)

Use of CA2 or CA4 results in bit-24 being set.

----------------------------------------------------------------------

QIT$