RPD$075066001

CHP$U630  STOPX (UNIX & VMS versions) - AWES

TPF$

CEB$

STOPPING POWER - ENERGY LOSS - RANGE

BLP$

STOPX calculates stopping powers, energy loss in finite samples and

ranges for ions in elements, compounds and mixtures.

The formulas and parameters of J. F. Ziegler (given in

The Stopping and Ranges of Ions in Matter, Vols 3 & 5,

Pergamon Press, 1980.)

are used to calculate stopping powers which are integrated for

energy-loss and range calculations.

STOPX is a "prompting" program which requires little or no additional

documentation. A list of the STOPX commands are given below.

All input may be entered in either upper or lower case.

CY1$

---------------------------------------------------------------------------

CMD   LIST         MEANING

PROJ  P1,P2,P3,... Define stoppee. PI = element or Z.

EA    E1,E2,E3,... Define energy/nucleon of stoppee.

(increasing order unless E3=INCR.)

DEDX               Calc. stopping power Mev/(mg/cm**2)

RNGE               Calc. range mg/cm**2

ELOS               Calc. E-loss in a series of absorbers

LP                 Assign output to \b\stopx.log\\ on DECstation

Assign output to \b\STOPX.PRT\\ on VAX

CON                Assign output to console (default)

END                Exit (ends program)

FILE               Assign output file for energy losses.

ABSB               Define absorbers (prompted for each absorber)

---------------------------------------------------------------------------

Legal absorber declarations have the form:

ELEMENT MIXTURE  ["GAS" PRES] THICKNESS

For example:

.1*(C 1H4)+.9*40AR GAS 500. -30.  - Gas mix of 10% Methane & 90% Argon

- Pressure = 500 torr

- Thickness = 30cm

28SI  3.5                         - Silicon target

- Thickness = 3.5mg/cm**2

Note: A "negative" thickness is taken to be in units of cm.

A "positive" thichness is taken to be in units on mg/cm**2.

---------------------------------------------------------------------------

Type:  stopx             ;To start with \b\stopx\\ defined in your .login,

;.cshrc, or login.com files, otherwise,

Type:  /usr/hhirf/stopx  ;To start execution on a HHIRF DECstation  or

Type:  /home/upak/stopx  ;To start execution on a SPARCstation      or

Type: @U1:[MILNER]STOPX  ;To start execution on the HHIRF VAX

Type:  help              ;and then try to figure out what to do.

CEB$

(See next page for comments by W. T. Milner)

TPF$

SP2$

CEB$

Comments by W. T. Milner

DBL$

BLP$

On a number of occasions I have been asked questions concerning the

syntax of solid absorber definitions - namely, whether the element mix

should be entered as mg/cm**2 or as elemental concentrations.

Relative elemental concentrations are the appropriate values to enter.

Also, the program does not distinguish between compounds and mixtures.

Note: that for solid Silicon Dioxide, all entries in the list shown below

give the same results:

CYB$

1.0*(28si 16o2)                1.0 - thickness is 1.0 mg/cm**2

1.0*(28si 16o 16o)             1.0

1.0*28si + 1.0*16o2            1.0

1.0*28si + 1.0*16o + 1.0*16o   1.0

100*(28si 16o2)                1.0

100*28si + 100*16o2            1.0

100*28si + 100*16o + 100*16o   1.0

BLP$

It is probably best to always give solid thickness in terms of mg/cm**2

since the program doesn't "know" the density of all elements and certainly

not that of compounds and mixtures.

QIT$