RPD$075075001

CHP$U300  DAMM - GENERAL - WTM

TPF$

CY1$

Sec Page Contents

010   2  GENERAL      - Introduction and General Features

020   3  GENERAL      - Getting Started

030   4  GENERAL      - Assigning Input/Output Files

040   4  GENERAL      - Loop-Execution & Symbol-Definition

050   5  GENERAL      - Log File -  damm.log

060   5  GENERAL      - Comments on Hard Copy

070   5  GENERAL      - File ID-directories and Count-Sums

090   6  GENERAL      - Cursor Tracking Problems With Xterminals

100   6  GENERAL      - Changes in Cursor-Mode Commands

110   6  GENERAL      - Mouse Button Customizing

120   7  GENERAL      - Screen Setup and Color Mapping

130   8  GENERAL      - Display Delays (hangup problems)

135   8  GENERAL      - Commands Which Suspend or Abort Operations

140   8  GENERAL      - Zeroing Histograms in Shared Memory

150   9  DISPLAY      - 1-D Display

155  11  DISPLAY      - 1-D Display (Peak Finding/Logging)

160  12  DISPLAY      - 2-D Display

170  14  DISPLAY      - Special XY-Line Display

175  16  DISPLAY      - Label Generation and Annotation

200  18  MANIPULATION - Syntax Definitions

210  18  MANIPULATION - Setup

220  19  MANIPULATION - I/O     1-D Histograms

230  19  MANIPULATION - Gating  2-D Histograms

240  19  MANIPULATION - General 2-D Projections

250  20  MANIPULATION - Operations on Buffers-1 & -2

260  20  MANIPULATION - Show Data, Count-Sums etc (from Bufs-1 & -2)

270  20  MANIPULATION - Modify Buffer Contents

280  21  MANIPULATION - Printer Plots

290  21  MANIPULATION - Gain-Shifts & Compressions

300  22  MANIPULATION - HIS-file (Create, Expand, Copy, Add, Gain-Shift...)

310  25  MANIPULATION - HIS-file (Adding & Subtracting Complete HIS-files)

350  26  BANANAS      - Definition, Rules, Construction & Display

360  27  BANANAS      - Projections

400  28  FITTING      - Introduction & General Features

410  29  FITTING      - 1-Key (Cursor Mode) Commands

420  30  FITTING      - Setup Commands

430  31  FITTING      - Display of Data, Fits & Printer-Plots

440  32  FITTING      - FIT Execution Commands

450  32  FITTING      - FIT Parameters - Saving, Setting, Default

460  33  FITTING      - Commands to Control Relative Peak-intensities

470  34  FITTING      - FIT Specification Details

480  35  FITTING      - Peak Shapes

490  35  FITTING      - Gradient-Search Method (FIT  request)

500  36  FITTING      - Gaussian Method        (GFIT request)

510  37  FITTING      - Estimated Uncertainties in Peak Areas

520  37  FITTING      - Reading Fit Results from damm.log

530  37  FITTING      - Common Problems

540  38  FITTING      - Peak Shape vs Asymmetry (Log Plot)

550  39  CUSTOMIZING  - Screen Configuration

560  41  CUSTOMIZING  - Graphic Screen Color Mapping

570  42  CUSTOMIZING  - Graphic Screen Black & White Mapping

TPF$

SEC$.010  Introduction and General Features

BLP$

DAMM is a Display, Analysis and Manipulation Module which is

configured to be used on Alpha workstations running Digital Unix.

Older versions were configured for DECstations, SPARCstations and

VAXstations.

DAMM provides all of the features to be found in VAXPAK programs DAM,

SAM, TDX and XAM as well as a number of additional features.

Some general features are listed below.

CYB$

Display features ..........................................................

DBL$

CY1$

(1)....Works with Alphas, DECstations, SPARCstations & VAXstations.

(2)....Displays 1-D data from HIS- or SPK-files.

(3)....Displays 2-D data from HIS-files.

(4)....Hardcopy available via screen-copy to networked Laser printers.

(5)....A dialog record may be saved on a Log-file.

(6)....Supports Free-Form (Banana) gate construction.

(7)....Provides for total number of counts within a Banana.

(8)....Supports X- & Y-projections of Bananas (saved on DAMQ8Q.SPK).

(9)....Provides for peak sum, centroid and fwhm.

(10)...Provides for spectrum analysis (fitting - see SEC# U300.400).

SIN$

CYB$

General features ..........................................................

DBL$

CY1$

(1)....Reads 1-D histograms from either HIS-, SPK-files.

(2)....Extracts GATES (on parameters 1 or 2) from 2-D histograms.

(3)....Supports general projections of Bananas on arbitrary axis.

(4)....Forms linear combinations, gain-shifts etc. of 1-D histograms.

(5)....Forms linear combinations, gain-shifts etc. of 2-D histograms.

(6)....Does linear gain and intercept transformations by rebinning.

(7)....Does crunches (sums a specified number of channels together).

(8)....Lists and plots 1-D histograms on the line printer.

(9)....Shows directory (ID'S) contained in HIS-, SPK- & BAN-files.

(10)...Shows count-sums for all ID's in SPK- or HIS-files.

SIN$

CEN$

(continued on next page)

TPF$

CYB$

Program operation .........................................................

BLP$

The program is controlled by a set of commands (alphabetic

directives) and associated data-lists (numbers): I call these

command-lists. Input is free-form.

Command and list-element delimiters are BLANK , ( ) /

SEC$.020  GETTING STARTED

BLP$

The steps given below outline how I would do it. Of course,

you can do it any way that you choose or not at all.

ODP$

(1)....Log onto the workstayion, Xterminal, etc. in the usual manner.

ODP$

(2)....Open a window and move it to the lower left corner of screen.

CY1$

(3)....Type:  damm                ;to start program on any host

;where the path to the executable

;has been defined in your

;.cshrc, .login or login.com files

;otherwise,

Type:  /usr/hhirf/damm     ;to start on a HHIRF DECstation

Type   /home/upak/damm     ;to start on a HHIRF SPARCstation

(4)....Type:  H                   ;for HELP directory

(5)....Type:  H  ITEM             ;for help on directory ITEM

(6)....Type:  H  FIG              ;for screen configurations

(7)....Try a few FIG commands to get a feel for how they work.

ODP$

(8)....Note the fact that the display required for entering

fitting information (like peak positions, etc) is via the DS & DSX

commands rather than the general display commands D & DX.

ODP$

(9)....Learn to use the HELP facility. That will usually be more

up-to-date than this document.

ODP$

(10)...Filenames have been made case-sensitive for the UNIX version.

Where default extensions apply, upper case is assumed for VMS and lower

case is assumed for UNIX.

Acceptable standard extensions now include .spk, .SPK, .his, .HIS, .ban,

.BAN, .cmd & .CMD. Note: If the his-file extension is lower/upper case

then the drr-file extension must be lower/upper case. Also note:

CYB$

/usr/users/directory/subdirectory/filename   ;is an acceptable form but

../directory/subdirectory/filename   ;is NOT! (at least for now)

TPF$

SEC$.030  Commands for Assigning Input/Output Files

CY1$

IN   FIL.EXT    - Open N-file (EXT = SPK or HIS)

OU   FIL.EXT    - Open O-file (EXT = SPK or HIS) - OUTPUT for SPK only

OU   FIL.SPK,NEW- Create and open O-file (SPK-file for output)

QF   FIL.EXT    - Open Q-file (EXT = SPK or HIS) - for display only

RF   FIL.EXT    - Open R-file (EXT = SPK or HIS) - for display only

SF   FIL.EXT    - Open S-file (EXT = SPK or HIS) - for display only

BAN  FIL        - Open FIL.BAN for store, recall, proj, etc

BAN  FIL,NEW    - Create & open FIL.BAN for store, recall, etc

- (See below for how to specify variable FILENAMES)

CLO  F          - Closes F-file (where F = N, O, P, Q, R, S or BAN)

DFIL            - Displays data files currently open

CMD  FIL        - Open and process commands from FIL.CMD

CMD  FIL.EXT    - Open and process commands from FIL.EXT

LON/LOF         - Turn Log-output (to LU7) ON/OFF (default = OFF)

CYB$

Explanation of variables in FILENAMES .....................................

BLP$

One symbol (integer variable) may be incorporated in a FILENAME

specification as the following examples illustrate:

CY1$

Example-1 .................................................................

SYM=3

OU FIL"SYM".SPK  ;Opens FIL3.SPK

Example-2 .................................................................

I=0

LOOP 3

I=I+1

IN FIL"I".SPK    ;Opens (in succession) FIL1.SPK, FIL2.SPK, FIL3.SPK

.

ENDLOOP

SEC$.040  Commands Related to Loop-Execution & Symbol-Definition

CY1$

SYM = EXPRESSION - Define symbol (SYM) up to 100 symbols supported

- symbols: UIND CIND ULOC CLOC FIX NONE FITS ALL and

- COLR GREY DOTS LIVE BAN  M N O P Q R S X are reserved

- expression syntax is same as in CHIL

- no imbedded blanks are allowed in expressions

- symbols may contain up to 8 characters

DSYM             - Displays currently defined sumbols & values

LSYM             - Logs currently defined symbols & values on damm.log

ZSYM             - Resets symbol table (deletes all symbols)

LOOP N           - Starts LOOP (executed N-times) N=SYM or CONST

CMD  ....        - Nesting supported

CMD  ....        - # lines between 1st LOOP & matching ENDL = 100

ENDL             - Defines end-of-loop

- KILL (entered before END) kills LOOP

- Ctrl/C    - aborts loop-in-progress

- opening of CMD-file within a LOOP not allowed

CEN$

(continued on next page)

TPF$

CYB$

LOOP suspension - the WO command ------------------------------------------

BLP$

A command WO [means the same thing as WOA or WHOA - i.e the

opposite of GIDDUP (my preferred spellings)]

has been implemented to work within LOOPs.

Whenever the WO command is encountered, the message:

CY1$

Type [RETURN] to CONTINUE--->

BLP$

will appear on the screen. This gives you an opportunity to look at

the display etc. before it gets wiped out. When you are finished

looking, press the [RETURN] key and it will continue.

SEC$.050  Log File - damm.log

BLP$

The VMS version of DAMM always creates a new version of DAMM.LOG

while the UNIX version creates a new damm.log or appends to an

old version of damm.log if it exists.

If you enter the command LON, almost all dialog to and from the host

will be logged, otherwise, only certain "print commands" will produce

output to the log-file or device.

You may turn the log ON/OFF by entering LON/LOF.

SEC$.060  Comments on Hard Copy

BLP$

As I have defined the default the color mapping, it is best to set

up the Workstation to Print Screen in the negative image mode.

If you are printing

on something like a LN03 ScriptPrinter, anything you chose to print

will be scaled to fit on one page. If you are using an LN03, it may

come out on multiple pages (and you may miss some) unless you choose

a "Portion of Screen" that it likes.

SEC$.070  File ID-directories and Count-Sums

CY1$

DIR  KF   Displays a list of all ID's in file-KF, where

KF   left blank says input-file

KF = N     denotes   input-file

KF = O     denotes  output-file

KF = Q     denotes       Q-file

KF = R     denotes       R-file

KF = S     denotes       S-file

KF = BAN   denotes     BAN-file

LDIR KF   Logs a list of all ID's in file-KF on DAMM.LOG

DDIR KF   Displays ID's & # of non-zero channels for SPK-files

DDIR KF   Displays ID-directory in detail        for HIS-files

Also logs on DAMM.LOG if LON

DSUM KF   Displays count-sums of all ID's on file-KF

Also logs on DAMM.LOG if LON

TPF$

SEC$.090  Cursor Tracking Problems With Xterminals

BLP$

The software-generated full-window cursor displayed by DAMM when in

the "1-key cursor mode" requires a lot of real-time response from the

host computer for live tracking of the mouse. This works fine when the

host is a local workstation but does not work well for Xterminals hosted

by a busy VAX. The following commands are intended to alleviate this

problem. Type:

CYB$

CURT LIVE  ;for full-window cursor which tracks mouse "live" (default)

CURT X     ;for new cursor display only for mouse-click or key-press

;(works better for Xterminals hosted by busy VAX, etc.)

BLP$

Execute the desired CURT command and then FIG to make it take effect.

SEC$.100  Changes in Cursor-Mode Commands

BLP$

I have eliminated the destinction between upper and lower case in

all cursor-mode commands. The Shift- and Caps-lock keys have no

effect.

In order to do this and retain meaningful command names, it was

neccessary to use two keys for certain commands. These

commands (UP, UX, UW, UL, UH, UO, and UB) are defined below:

CY1$

P/UP   Add/Delete peak to Library (pos specified by cursor)

X/UX   Fix/Free  peak position (for displayed peak nearest to cursor)

W/UW   Fix/Free  peak width    (for displayed peak nearest to cursor)

L/UL   Fix/Free  Lo-Side ASYM  (for displayed peak nearest to cursor)

H/UH   Fix/Free  Hi-Side ASYM  (for displayed peak nearest to cursor)

O/UO   Turn peak  ON/OFF       (for displayed peak nearest to cursor)

B/UB   Add/Delete background point at cursor position

BLP$

Read UP as Unset Peak, for example. As usual, no carriage return

is used. There are also a few other changes in commands.

These are listed below:

CY1$

LF-ARROW    Set expand-region lo-limit

RT-ARROW    Set expand-region hi-limit

DN-ARROW    Pan DOWN - move picture so cursor-chan at right-screen

UP-ARROW    Pan UP   - move picture so cursor-chan at  left-screen

/           Display XCUR, YCUR, channel# & energy

SEC$.110  Mouse Button Customizing

BLP$

The Mouse Buttons can be used in place of some key-strokes

while in the 1-key cursor-mode.

Different button definitions are provided

for the three different types of displays (namely: the

displays resulting from the commands  D, DD & DS).

The following customizing commands are supported:

CY1$

BUD  L,M,R  ;Defines Left,Middle,Right buttons for cursor in  D-window

BUDD L,M,R  ;Defines Left,Middle,Right buttons for cursor in DD-window

BUDS L,M,R  ;defines Left,Middle,Right buttons for cursor in DS-window

BUD  L,H,S  ;Example (cursor in 1-D)   L-butt sets sum-reg lo-limit,

;M-butt sets sum-reg hi-limit,  R-butt requests S-sum

BUDD A,T,Z  ;Example (cursor in 2-D)   L-butt adds banana points,

;M-butt totalizes enclosed counts,  R-butt zots banana

CYB$

Buttons can't be set to  / or ;  -  set to  ? or :  instead

TPF$

SEC$.120  Screen Setup and Color Mapping

BLP$

Screen configuration (placement of graphics windows on the screen) and

color (or black & white) mapping is discussed in more detail in SEC#

U300.550, U300.560 and U300.570 (if you have a B&W monitor, you will

probably want to change the color mapping).

Here we give the usual list of commands

and a brief description of each.

CYB$

COMMANDS RELATED TO SCREEN SETUP & COLOR MAPPING ...........................

CY1$

FIGI           ;Set  screen configuration library to default

FIGF FILNAM    ;Read screen configuration library from FILNAM

FIG  N         ;Set current screen configuration to type-N

WIN  ID        ;Set subsequent displays to be in window-ID (dflt=1)

AXON ID        ;Enable  the drawing of axis for  window-ID (dflt)

AXOF ID        ;Disable the drawing of axis for  window-ID

CMAP           ;Set color map to default  ("takes" after next FIG)

CMAP FILNAM    ;Set color map from FILNAM ("takes" after next FIG)

REVV           ;Reverse all color specs   ("takes" after next FIG)

DLNS N         ;Set # disp-lines = N - for HELP, DDIR & DSUM

CURT LIVE      ;Set full-wind cursor to track mouse LIVE (default)

CURT X         ;New full-wind cursor generated via mouse-click or key

;The CURT command takes effect only after next FIG

SSI            ;Set screen to initial - erase all graphic windows

BLP$

Program \b\damm\\ has, by default, 16 pre-defined screen connfigurations

available. Each is referenced (via the FIG command) by an ID-number.

A list of the ID-numbers along with the associated configuration is

given below. Try a few \b\FIGs\\ and observe the results or if you

really want to get serious, see SEC# U300.550. In particular, if you

have an Xterminal with less than 1024 x 860 pixels, you will probably

need to modify the configuration table as described in SEC# U300.550.

CY1$

1-[]     2-[][]   3-[][]   4-[][]   5-[][]   6-[][]   7-[][]   8-[][]

[]       [][]     []       [][]     []       [][]

[]       [][]     []       [][]

[]       [][]

9-[][]  10-[][]  11-[--]  12-[--]  13-[--]  14-[--]  15-[][][]

[]       [][]              [--]     [--]     [--]     [][][]

[]       [][]                       [--]     [--]     [][][]

[]       [][]                                [--]     [][][]

[]       [][]                                         [][][]

16-[2D]  17-[2D]  18-[2D]  19-[2D]  20-[][][][]

[][][][]

[][][][]

[][][][]

[][][][]

CHP$U300  DAMM - DISPLAY - WTM

TPF$

SP1$

SEC$.130  Display Delay (hangup problems)

BLP$

Some display devices (X-terminals for example) may require a delay between

successive displays. If your device "hangs up" or produces "incomplete

displays" when executing LOOPS, you might try increasing the appropriate

delay. A range of 10 to 20000 miliseconds is accepted.

CY1$

DLAF MS        ;Set FIG-delay     to MS   milliseconds (default=1000)

DLAF           ;Set FIG-delay     to 1000 milliseconds

DLAD MS        ;Set Display-delay to MS   milliseconds (default=500)

DLAD           ;Set Display-delay to 500  milliseconds

SP1$

SEC$.135  Commands Which Suspend or Abort Operations

CY1$

WAIT  MS             ;Suspends execution for MS milliseconds

;(60,000 Milliseconds is the maximum allowed)

WO                   ;Suspends execution until you  type [RETURN]

;(it prompts you to: Type [RETURN] to continue

Ctrl/C               ;Interrupts (aborts) a FIT-in-progress

;Interrupts (aborts) a loop-in-progress

;Interrupts (aborts) a command-file being executed

BLP$

Note: If you interrupt a fit-in-progress (being executed within a command

file), this does not abort the command file execution. An additional

Ctrl/C will be required to abort the command file.

SP1$

SEC$.140  Zeroing Histograms in Shared Memory

BLP$

The following commands are provided for use when \b\damm\\ is used

to monitor histogram generation during data acquisition or tape scanning.

CY1$

Z  ON                ;Enable  Z-command

Z  OFF               ;Disable Z command (default)

Z  IDA IDB IDC ..    ;Says zero explicit histogram ID-list

Z  IDA to IDB        ;Says zero IDA thru IDB (in steps of 1)

Z  IDA to IDB INC    ;Says zero IDA thru IDB (in steps of INC)

For these commands to work, the following conditions are required:

CYB$

(A)....The his-file must be opened by DAMM via the IN cmd.

(B)....The his-file must be attached to a scan or monitor process

(i.e. the his-file must be in a shared memory segment)

CEB$

The disk-file is NOT zeroed!

TPF$

SP1$

SEC$.150  Commands Related to 1-D Display

CY1$

FIG  NF        ;Choose screen config-NF. See SEC# U300.120 & U300.550

;for screen configuration and color mapping commands

WIN  NW        ;Set subsequent displays to be in window-NW

LIN/LOG        ;Set display to linear/log  (default is linear)

PLON/PLOF      ;Turn peak logging ON/OFF (dflt OFF) see SEC# U300.155

ST /OV         ;Set to disp mult hist stacked/overlayed (dflt = OV)

CAL  A,B,C     ;Define energy calibration (E=A+B*Chan+C*Chan**2)

COL  I,J,K...  ;Defines color sequence for display

;For I,J.. = 1     2   3     4    5    6       7

;       COL= white,red,green,blue,yellow,magenta,cyan

GWID WID       ;Define cursor-mode sum-region width (channels)

DNOR LO,HI     ;Normalize displayed data to count-sum of chans LO,HI

DL   LO,HI     ;Set display limits (min,max chan#)

DMM  LO,HI     ;Set display limits (min,max counts)

;LO or HI = X  says use MIN or MAX data value

D    IDLST     ;Display histogram ID's contained in IDLST

DX   IDLST     ;Display IDLST (range defined by expand-region)

;-----------------------------------------------------------

;IDLST is of the form:

;KF C ID, C ID.. KF C ID, C ID..

;or

;KF C IDA to IDB,<INC> KF C ID ID ID ... KF C IDA to IDB ...

;-----------------------------------------------------------

;ID, IDA & IDB denote ID-numbers

;INC (optional) denotes an ID-number increment (DFLT=1)

;C is an OPTIONAL floating-point norm-coeff (DFLT=1.0)

;KF = M,N,O,P,Q,R,S (default is N) and denotes:

;MEM-BUF, IN-FIL, OU-FIL, PROJ-FIL, Q-FIL, R-FIL, S-FIL

;If IDLST omitted, uses previously defined IDLST

;-----------------------------------------------------------

SUML LO,HI     ;Define sum region for SUM command below

SUM  IDLST     ;Sum counts (LO,HI) (IDLST same as D except C illegal)

C              ;Enter cursor-mode

SSI            ;Set screen to initial - erase graphic windows

CEN$

(continued on next page)

TPF$

SP1$

SEC$.150  Commands Related to 1-D Display (continued)

CYB$

1-KEY CURSOR COMMANDS FOR 1-D DISPLAY .....................................

CY1$

<-- (LF-ARROW) ;Set expand-region lo-limit

--> (RT-ARROW) ;Set expand-region hi-limit

E              ;Expand display

UP-ARROW       ;Pan up   - move picture so cursor-chan at  left-screen

DN-ARROW       ;Pan down - move picture so cursor-chan at right-screen

;(you must be expanded to execute pan)

L              ;Set sum-region lo-limit

H              ;Set sum-region hi-limit

G              ;Set sum-region limits (LO=XCUR, HI=XCUR+WID-1)

V              ;Make expand-/sum-region markers visible/invisible (toggles)

M              ;Turn expand-/sum-region markers ON

J              ;Junk (delete) all expand-/sum-region markers

S              ;Display sum, centroid, fwhm (2.354*sigma) of sum region

;DATA(LO),DATA(HI) defines BGD for NETS

A              ;Display sum, centroid, fwhm (2.354*sigma) of sum region

;YCUR(LO),YCUR(HI) defines BGD for NETS

C              ;Draw peak-marker and display chan#  at cursor pos

P              ;Draw peak-marker and display energy at cursor pos

/ or ?         ;Display XCUR, YCUR, channel# & energy

'              ;Same as ? except forces logging (see SEC# U300.155)

Q              ;Quit cursor-mode (return to normal-mode)

See SEC# U300.110 or Type: \b\h mous\\ for use of Mouse Buttons.

TPF$

SEC$.155  Peak Finding/Logging

CY1$

The following commands control peak finding.

FIND BIAS,IFWHM     ;Turn peak-find ON  (see definitions below)

FIND                ;Turn peak-find ON  (with defaults - see below)

NOFI                ;Turn peak-find OFF (default is OFF)

ODP$

BIAS...is the number of standard deviations above background that a

peak channel must be in order to be considered as part of a peak.

Useful values of BIAS are in the range 3 to 10.

The default value is 5.0.

ODP$

IFWHM..is the approximate full-width at half-max (in channels)

of peaks in the region of interest.

This value is not very critical but should be within a factor of

2 or so of the correct value. The default value is 5.

BLP$

All peaks found within the display region will be marked & labeled

with the associated energy-calibration value (default is the

same as channel number). Peak labels are integers (no decimals - to

minimize screen space used) so if you want labels to be in units of

keV, for example, you must enter CAL such that E(keV) is a whole

number.

CY1$

See SEC# U300.430 for how peak-finding is used in fitting operations.

CY1$

\bu\Logging "found" and "marked" peaks on damm.log\\

BLP$

Peaks which either \b\found\\ via the FIND command above or

\b\marked\\ via the 1-key command (/ or ? or ;) may be logged

on damm.log. The following commands (independent of LON/LOF)

turns said logging ON and OFF.

CY1$

PLON           ;Turns peak logging (to damm.log) ON

PLOF           ;Turns peak logging (to damm.log) OFF (default)

BLP$

For \b\found peaks\\, damm.log may be read (skipping prog, date & time) as:

CY1$

READ(LU,10)IFLG,ID,CH,HEFT,(FILNAM(I),I=1,16)

10 (29X,A4,I10,2F10.0,2X,16A4)

Where: IFLG   = 'PEAK' for found peak log entry

ID     = Spectrum ID number

CH     = Peak location in channels

HEFT   = Peak heftiness

FILNAM = First 64 characters of spk- or his-filename

BLP$

For \b\marked peaks\\, damm.log may be read (skipping prog, date & time) as:

CY1$

READ(LU,10)IFLG,ID,CH,ENER,(FILNAM(I),I=1,16)

10 (29X,A4,I10,2F10.0,2X,16A4)

Where: IFLG   = 'MARK' for marked peak log entry

ID     = Spectrum ID number

CH     = Peak location in channels

ENER   = Peak "energy" from calibration constants

FILNAM = First 64 characters of spk- or his-filename

TPF$

SEC$.160  Commands Related to 2-D Display

CY1$

FIG   NF       ;Choose screen config-NF. See SEC# U300.120 & U300.550

;for screen configuration and color mapping commands

WIN   NW       ;Set subsequent displays to be in window-NW

ZLEV  N        ;Set # of color/grey-scale intensity levels to N

ZLEV           ;Set # of color/grey-scale intensity levels to 10 (dflt)

GRAS  I,J,K..  ;Set grey-scale values (range 0-100) & ZLEV (# entries)

;Must do after first FIG  ("takes" after next FIG)

ZINT  COLR     ;Set 2-D intensity mapping to Color (default)

ZINT  GREY     ;Set 2-D intensity mapping to Grey-scale

ZINT  DOTS     ;Set 2-D intensity mapping to Dot-matrix (3x3 or 5x5)

ZSON/ZSOF      ;Z-scale ON/OFF - displays cnts vs colors legend (dflt)

ZLIN/ZLOG      ;Set 2-D display to linear/log (default is log)

XC    LO,HI    ;Set min & max X-channel numbers for display

YC    LO,HI    ;Set min & max Y-channel numbers for display

ZMM   LO,HI    ;Set min & max counts/channel    for display & count-sum

ZMM   LO       ;Sets min & searches for max (semi-autoscale)

ZMM            ;Searches for min & max      (full-autoscale)

DD       ID    ;Display 2-D histogram ID from IN-file

DD    KF,ID    ;Display 2-D histogram ID from KF-file

DDX      ID    ;Display 2-D histogram ID from IN-file (expand region)

DDX   KF,ID    ;Display 2-D histogram ID from KF-file (expand region)

;Where KF = N,O,Q,R,S (default is N) and denotes:

;IN-FIL, OU-FIL, Q-FIL, R-FIL, S-FIL

;ID (and KF) omitted says use previously defined spec

DDID           ;Shows ID-number & Filename for current 2-D display

ZBL            ;Zero in-core BAN-library

LBL            ;List in-core BAN-ID numbers

C              ;Enter cursor-mode

SSI            ;Set screen to initial - erase graphic windows

CEN$

(continued on next page)

TPF$

SP1$

SEC$.160  Commands Related to 2-D Display (continued)

CYB$

1-KEY CURSOR COMMANDS FOR 2-D DISPLAY .....................................

CY1$

<-- (LF-ARROW) ;Set expand-region lo-left- limit

--> (RT-ARROW) ;Set expand-region hi-right-limit

V      ;Make expand-region markers visible/invisible (toggles)

J      ;Junk (delete) expand-region markers

E      ;Expand

1      ;Move display such that cursor is at lo-left

2      ;Move display such that cursor is at hi-left

3      ;Move display such that cursor is at hi-right

4      ;Move display such that cursor is at lo-right

Z      ;Zero  (open) active X,Y-list

A      ;Add point to active X,Y-list

D      ;Dele nearest point in active X,Y-list

M      ;Move nearest point in active X,Y-list to cursor pos

I      ;Insert a point in active X,Y-list at cursor position

L      ;List active X,Y-list (on VDT)

B      ;Draw active BAN & BAN's in in-core library

G      ;Prompt for ID & read into in-core BAN-library

O      ;Open nearest in-core BAN for modification

S      ;Prompt for ID & store in in-core library & on disk

R      ;Store  nearest BAN with original ID

F      ;Remove nearest BAN from in-core library & erase

K      ;Delete nearest BAN from in-core library and disk

T      ;Totalize counts in nearest BAN (active or not)

P      ;Totalize and & save X- & Y-projections on DAMQ8Q.SPK

/ or ? ;Display/(log if PLON)  X,Y-coordinates of cursor

'      ;Like ? but forces log (see \b\X,Y-log format\\ below)

Q      ;Quit cursor-mode (return to normal-mode)

See SEC# U300.110 or Type: \b\h mous\\ for use of Mouse Buttons.

---------------------------------------------------------------------------

\b\X,Y-log format-\\ damm.log may be read (skipping prog, date & time) as:

READ(LU,10)IFLG,ID,X,Y,(FILNAM(I),I=1,16)

10 (29X,A4,I10,2F10.0,2X,16A4)

Where: IFLG   = 'MAR2' for 2-D position mark entry

ID     = Histogram ID number

X      = X-position in channels

Y      = Y-position in channels

FILNAM = First 64 characters of spk- or his-filename

---------------------------------------------------------------------------

TPF$

SP2$

SEC$.170  Special XY-Line Display

BLP$

DECstation

\b\damm\\ provides for the display of one or more lists of

concatenated vectors (lines) onto any of the FIGed windows.

In order to use this feature, a ASCII file must be created

of the following form:

CY1$

XYDATA  ID  XLEN  YLEN

X Y

X Y

. .

. .

XYDATA  ID  XLEN  YLEN

X Y

X Y

. .

END-OF-FILE     Where:

ODP$

XYDATA is the ASCII string "xydata" in upper or lower case (XYDA is

enough).

ODP$

ID.....is an integer ID-number by which the X,Y data set which follows

will be referenced.

CY1$

XLEN...is the X-length basis for the following X-values (optional).

YLEN...is the Y-length basis for the following y-values (optional).

X......is a floating# (used as chan# for 1D & 2D displays).

(X is decoded via E-format & can contain up to 12 characters)

Y......is a floating# (used as chan# for 2D and count for 1D displays).

(Y is decoded via E-format & can contain up to 12 characters)

BLP$

If XLEN and YLEN are entered and "histogram lengths" are provided by the

displayed histograms, the X,Y data will be appropriately scaled to match

the histogram data contained in the window in which it is drawn.

"Histogram lengths" are provided by all his-file directories and by

spk-file entries which are copied from his-files. If either of these

"length entries" are missing, no scaling will be done. For 1D displays,

no attempt is made to scale Y.

CEN$

(continued on next page)

TPF$

SP1$

SEC$.170  Special XY-Line Display (continued)

CY1$

\bu\List of commands\\

XYF  filename     ;Opens XY-file and reads in all data

XYI               ;Displays XY IDs which have been read in

XYD  I J K L ...  ;Displays XY data for IDs (I J K L ..) into active

;window (default = window 1 or set by WIN command)

XYP  I J K L ...  ;Same as XYD except that XY-points are shown in

;addition to the connecting vectors

;XYD or XYP with no ID-list uses previous ID-list

XYC  KOL          ;Specifies the "color" for subsequent XY displays

;Legal values of KOL are:

;WHIT - white      - CMAP entry 34

;RED  - red        - CMAP entry 35

;GREE - green      - CMAP entry 36

;BLUE - blue       - CMAP entry 37

;RG   - red-green  - CMAP entry 38

;RB   - red-blue   - CMAP entry 39

;GB   - green-blue - CMAP entry 40

BLP$

Note: Displays are done in "complement mode" so that displaying the same

ID a second time will erase it, displaying a third time will show it, etc.

CY1$

\bu\Current Limits\\

Maximum number of IDs        in a file = 2048

Maximum number of XY entries in a file = 256,000

Maximum number of XY points  in a set  = 500

BLP$

These limits are rather arbitrary, and can easily be changed.

CY1$

\bu\Comments\\

ODP$

(1)....The XY display feature is for \b\display only\\. The XY data

file is \bu\not\\ another data type on a par with HIS-file or

SPK-file data. It cannot be analysed, summed, or combined with

other data, etc.

ODP$

(2)....If the XY data extend beyond the range of the display, it is

allowed to spill over into the "scale label" regions. It is easy enough

to prevent this but I am not sure if it would be more desirable.

ODP$

(3)....An ID label is drawn near to the XY-point which is "nearest" to

the center of the window. If multiple XY-lines are displayed,

there may be some confusion in identification. I will think about trying

to minimize this.

ODP$

(4)....All commands and specifications are case insensitive.

TPF$

SP1$

SEC$.175  Label Generation and Annotation

BLP$

\b\damm\\ provides a simple method for labeling and annotating

graphical displays. The following features are provided:

ODP$

(1)....Normal text labels may be generated either interactively or

by the usual processing of command files.

ODP$

(2)....The \b\Label Generating Commands\\ (LA7, LA8, LA9) described

below are used to generate up to three blocks of "label text".

ODP$

(3)....Each of the three "label blocks" can contain up to 10 lines of

text with each line containing up to 76 characters (80 - 4 command

characters).

ODP$

(4)....Each \b\damm\\ window supports up to 3 independent, active

(relocatable) label blocks.

ODP$

(5)....By "locking" labels already displayed and re-defining label

blocks, you can display as many labels as you like.

ODP$

(6)....Labels are positioned within graphics windows using the

1-key \b\Label Display Commands\\ described below.

ODP$

(7)....\b\Label Pointer Commands\\, defined below, may be used to make

close associations between labels and specific graphic features.

BLP$

\b\Note: LA7, LA8, LA9\\ label specifiers are used in order to make the

association with the label display keys \b\(7,8,9)\\ easier.

CYB$

Label Generating Commands -------------------------------------------------

CY1$

LA7  text   ;Adds line of "text" to LA7 label

LA8  text   ;Adds line of "text" to LA8 label

LA9  text   ;Adds line of "text" to LA9 label

LAZ7        ;Deletes all lines of   LA7 label

LAZ8        ;Deletes all lines of   LA8 label

LAZ9        ;Deleted all lines of   LA9 label

LAL         ;Displays current labels - LA7, LA8, LA9

LAC  KOL    ;Specifies label color for subsequent displays

;Legal values of KOL are:

;WHIT - white      - CMAP entry 34

;RED  - red        - CMAP entry 35

;GREE - green      - CMAP entry 36

;BLUE - blue       - CMAP entry 37

;RG   - red-green  - CMAP entry 38

;RB   - red-blue   - CMAP entry 39

;GB   - green-blue - CMAP entry 40

CEB$

(see next page for label display commands)

TPF$

SP2$

SEC$.175  Label Generation and Annotation (continued)

CYB$

Label Display Commands (1-key cursor-mode) --------------------------------

CY1$

\bu\Key\\      \bu\Action\\

7        Displays LA7 at cursor location (complement mode)

8        Displays LA8 at cursor location (complement mode)

9        Displays LA9 at cursor location (complement mode)

0        Locks previously displayed labels against subsequent change

(i.e. all labels displayed in a given window will be fixed)

(enables the drawing of additional labels)

It goes like this:

ODP$

(1)....The first strike of say key-7 displays LA7 with the

upper-left corner of an unclosed box at the cursor location.

ODP$

(2)....The next strike of key-7 will erase the LA7 label and the box.

ODP$

(3)....The next strike of key-7 will display LA7 in any new cursor

location. Etc, etc. and the same for key-8 and -9.

ODP$

(4)....Finally, key-0 will close any open boxes and lock in all current

labels within a given window. Additional labels are now enabled.

CYB$

Label Pointer Commands (1-key cursor-mode) --------------------------------

BLP$

The following 1-key commands provide for the interactive drawing of

concatenated straight-line segments. The idea is to provide a method of

associating a block of text with a very specific region in a display.

The following 1-key commands are used to draw and modify such lines.

CY1$

\bu\Key\\      \bu\Action\\

=        Adds a vector point to current list and draws point or line

-        Deletes last vector point in current list and erases line

;        Locks current vector list against subsequent change and

enables new list

CYB$

Comments:

ODP$

(1)....Each window supports an independent active vector list.

ODP$

(2)....The active vector-list associated with any window can contain

a maximum of 20 points.

CHP$U300  DAMM - MANIPULATION - WTM

TPF$

SEC$.200  Command Syntax - General Definitions

CY1$

B1   - Memory Buffer-1

B2   - Memory Buffer-2

ID   - The  ID-number of histogram to be read

NUID - Next ID-number to be assigned to output histogram

LO   - A first-channel-number (usually of a Gate)

HI   - A last- channel-number (usually of a Gate)

FAC  - A multiplication factor

Meaning of the individual command-characters ..............................

I    - Input  or read

O    - Output or write

A    - Add or accumulate

S    - Shift (gain shift)

GX   - Gate on X-parameter (i.e. parameter # 1)

GY   - Gate on Y-parameter (i.e. parameter # 2)

1    - Buffer-1

2    - Buffer-2

M    - Multiply

C    - Crunch

D    - Divide

SEC$.210  Commands for Setup (no immediate action)

CY1$

NUID IV    - Set next ID to be used to IV

IDST N     - Set ID-step to be used in implied I/O loops

(remains active until changed - default=1)

CRUN IVAL  - Sets standard crunch value to IVAL

GASP XI1,XI2,XF1,XF2,NCF - Standard gain shift specification

SIDA       - Says treat 16-bit HIS-file data as signed

USDA       - Says treat 16-bit HIS-file data as un-signed (default)

TPF$

SEC$.220  Commands for Input/Output of 1-D Histograms

CY1$

I    kf ID            Input to B1

IS   kf ID            Input to B1, gain shift B1

IA   kf ID,FAC        Input to B1, B2=B2+FAC*B1

ISA  kf ID,FAC        Input to B1, shift  B1, B2=B2+FAC*B1

IO   kf ID            Input to B1, output B1

ISO  kf ID            Input to B1, shift  B1, output B1

IO   kf IDA,IDB       Input to B1, output B1     (for ID=IDA,IDB)

ISO  kf IDA,IDB       Input to B1, shift, output (for ID=IDA,IDB)

O1                    Output B1

O2                    Output B2

SEC$.230  Commands for Gating 2-D Histograms

CY1$

GY   kf ID,LO,HI      Y-gate to B1

GYS  kf ID,LO,HI      Y-gate to B1, shift  B1

GYO  kf ID,LO,HI      Y-gate to B1, output B1

GYO  kf IDA,IDB,LO,HI Y-gate to B1,         output B1 (for ID=IDA,IDB)

GYSO kf ID,LO,HI      Y-gate to B1, shift & output B1

GYSO kf IDA,IDB,LO,HI Y-gate to B1, shift & output B1 (for ID=IDA,IDB)

GYA  kf ID,LO,HI,FAC  Y-gate to B1, B2=B2+FAC*B1

GYSA kf ID,LO,HI,FAC  Y-gate to B1, shift B1, B2=B2+FAC*B1

GX   kf ID,LO,HI      X-gate to B1

GXS  kf ID,LO,HI      X-gate to B1, shift  B1

GXO  kf ID,LO,HI      X-gate to B1, output B1

GXO  kf IDA,IDB,LO,HI X-gate to B1,         output B1 (for ID=IDA,IDB)

GXSO kf ID,LO,HI      X-gate to B1, shift & output B1

GXSO kf IDA,IDB,LO,HI X-gate to B1, shift & output B1 (for ID=IDA,IDB)

GXA  kf ID,LO,HI,FAC  X-gate to B1, B2=B2+FAC*B1

GXSA kf ID,LO,HI,FAC  X-gate to B1, shift B1, B2=B2+FAC*B1

O1                    Output B1

O2                    Output B2

SEC$.240  Commands for General 2-D Projections

CY1$

PJ   kf ID,BID,DEGR               PROJ TO B1

PJS  kf ID,BID,DEGR               Proj to B1, shift  B1

PJO  kf ID,BID,DEGR               Proj to B1, output B1

PJO  kf IDA,IDB,BIDA,BIDB,DEGR -  Proj to B1, output B1

(outer loop on BID, inner loop on ID)

PJSO kf ID,BID,DEGR               Proj to B1, shift & output B1

PJSO kf IDA,IDB,BIDA,BIDB,DEGR -  Proj, shift, output

(outer loop on BID, inner loop on ID)

PJA  kf ID,BID,DEGR,FAC           Proj to B1, B2=B2+FAC*B1

PJSA kf ID,BID,DEGR,FAC           Proj to B1, shift B1, B2=B2+FAC*B1

PJAL                              Project all bananas in currently open

BAN-file for HIS-files, ID's & DEGR'S

contained therin

O1                                Output B1

O2                                Output B2

\B\ID\\ denotes histogram ID, \b\BID\\ denotes Banana ID

(DEGR = Projection-axis angle in degrees)

\b\kf is an optional file-code = N,O,P,Q,R,S (default = N)\\

TPF$

SEC$.250  Commands for Operations on Buffer-1 & Buffer-2

CY1$

M1  XM             Multiply B1 by XM

M2  XM             Multiply B2 by XM

C1  ICRUN          Crunch B1  by ICRUN (standard crunch unchanged)

C2  ICRUN          Crunch B2  by ICRUN (standard crunch unchanged)

S1                 Shift  B1  by standard GASP

S2                 Shift  B2  by standard GASP

S1  XI1,XI2,XF1,XF2,NCF - Shift B1 as specified (standard GASP unchanged)

S2  XI1,XI2,XF1,XF2,NCF - Shift B2 as specified (standard GASP unchanged)

Z1                 Zero B1

Z2                 Zero B2

A12  FAC           B2=B2+FAC*B1

A21  FAC           B1=B1+FAC*B2

SWAP               Swap B1 & B2

M2D1 FAC           B2=(FAC*B2)/B1

MX12               B2=MAX(B1,B2) channel-by-channel

MX21               B1=MAX(B1,B2) channel-by-channel

MN12               B2=MIN(B1,B2) channel-by-channel

MN21               B1=MIN(B1,b2) channel-by-channel

O1                 Output B1

O2                 Output B2

SEC$.260  Commands which Show Data, Count-Sums etc (from Bufs-1 & -2)

CY1$

PR1                Print   Buffer-1

PR2                Print   Buffer-2

D1   LO,HI         Display Buffer-1 (channels LO thru HI)

D2   LO,HI         Display Buffer-2 (channels LO thru HI)

SUM1 LO,HI         Display sum of counts LO-thru-HI of B1

SUM2 LO,HI         Display sum of counts LO-thru-HI of B2

COMP NCH           Compare first NCH-channels of B1 & B2

(gives # counts and # mis-matches)

GEN  ID,KO,KX,NCH  Generate test spectrum in B1 (NCH channels)

Channel contents = KO+KX*(channel#+1)

SEC$.270  Commands which Modify Buffer Contents

CY1$

SET1 ICN,YV        Set channel ICN of B1 to YV

SET2 ICN,YV        Set channel ICN of B2 to YV

SET1 LO,HI,YV      Set channels LO-thru-HI of B1 to YV

SET2 LO,HI,YV      Set channels LO-thru-HI of B2 to YV

SET1 LO,HI,YA,YB   Set channels LO-thru-HI of B1 to YA-thru-YB

SET2 LO,HI,YA,YB   Set channels LO-thru-HI of B2 to YA-thru-YB

ADD1 ICN,YV        Add YV to channel ICN of B1

ADD2 ICN,YV        Add YV to channel ICN of B2

ADD1 LO,HI,YV      Add YV to channels LO-thru-HI of B1

ADD2 LO,HI,YV      Add YV to channels LO-thru-HI of B2

ADD1 LO,HI,YA,YB   Add YA-thru-YB to channels LO-thru-HI of B1

ADD2 LO,HI,YA,YB   Add YA-thru-YB to channels LO-thru-HI of B2

(i.e. a strait line)

MSK1 LO,HI,MSK     Mask (with MSK)   channels LO-thru-HI of B1

MSK2 LO,HI,MSK     Mask (with MSK)   channels LO-thru-HI of B2

(MSK is entered in hexadecimal)

TPF$

SEC$.280  Commands Related to Printer Plots

CY1$

SKRZ               Set to skip repeated-zeros for printer plots

PLRZ               Set to plot repeated-zeros for printer plots

PLG  ID,LO,HI,NCYC      - Input to B1 & LOG plot

PLN  ID,LO,HI,NCFS      - Input to B1 & LIN plot

PLG  IDA,IDB,LO,HI,NCYC - Input to B1 & LOG plot (for ID=IDA,IDB)

PLN  IDA,IDB,LO,HI,NCFS - Input to B1 & LIN plot (for ID=IDA,IBD)

PLG1 LO,HI,NCYC    Log    Printer-plot of Buffer-1

PLG2 LO,HI,NCYC    Log    Printer-plot of Buffer-2

PLN1 LO,HI,NCFS    Linear Printer-plot of Buffer-1

PLN2 LO,HI,NCYC    Linear Printer-plot of Buffer-2

(NCFS = # of counts full-scale for LIN plots)

(NCYC = # of cycles            for LOG plots)

SEC$.290  Discussion of Gain-Shifts and Compressions

BLP$

Gain shifts are specified by five parameters - XI1, XI2, XF1, XF2 and NCF.

XI1 and XI2 represent two locations (in channel-# units)

in the initial 1-D histogram and

XF1 and XF2 represent corresponding

locations in the final histogram (i.e. after the transformation).

That is:

CY1$

XF=A+B*XI

where,

B=(XF2-XF1)/(XI2-XI1)

and

A=XF1-B*XI1

BLP$

NCF gives the number of channels in the histogram after the transformation.

If NCF=0, the final #-of-channels is determined by the initial

#-of-channels NCI and the transformation specified.

If NCF=-1 the final #-of-channels is set equal to NCI.

Counts are redistributed into the final set of channels (bins) by assuming a

uniform distribution of counts in the initial bins.

Data shifted below channel-#-0 and above channel-#-NCF-1 are lost and gone

forever.

CY1$

Gain-shifts are always "in place"

CRUN IVAL (i.e. standard crunch) does it at "input time"

Data is kept internally as floating - is converted to fixed on output

All output from DAMM is 32 bits/channel

TPF$

SP1$

SEC$.300  HIS-file (Create, Expand, Copy, Add, Gain-Shift, etc)

BLP$

The commands given below may be used to \b\Create\\ a new HIS-file or to

\b\Expand\\ an existing HIS-file (by specifying additional entries) for

support of the operations described on the following pages of this section.

CY1$

HOU  fil.his     - Opens fil.his for histogram Output

- If non-existant, Prompts for permission to create it

HDIM N           - HIS dimension (N = 1 or 2) (required)

HWPC N           - half-wds/chan (N = 1 or 2) (required)

HPAR PX PY       - Parameter IDs              (reference only)

LRAW LX LY       - Raw    parm lengths        (reference only)

LSCL LX LY       - Scaled parm lengths        (reference only)

MINC MNX MNY     - Min chan#s for X & Y       (required)

MAXC MXX MXY     - Max chan#s for X & Y       (required)

XLAB labl-x      - X-label (ASCII)            (reference only)

YLAB labl-y      - Y-label (ASCII)            (reference only)

HTIT title       - Title   (ASCII)            (reference only)

HEDD             - Displays current data to be used for next his generation

HGEN HID         - Generates histogram HID using current data

BLP$

When the \b\HGEN\\ command is entered, the data associated with

\b\HDIM, MINC & MAXC\\ will be checked for legallity and

\b\HID\\, given in the \b\HGEN\\ command, will be checked for multiple

definition.

CY1$

\bu\Note for Input HIS-file\\

ODP$

o......If HIS-file is shorter than implied by DRR-file, the open

request is rejected.

CY1$

\bu\Notes for Output HIS-file\\

ODP$

o......If HIS & DRR files do not exist, you will be asked for

permission to create them.

ODP$

o......If DRR-file exists but HIS-file does not, you are asked for permission

to create the HIS-file and then extend it.

ODP$

o......If HIS-file exists but DRR-file does not, the open request is rejected.

CEN$

(continued on next page)

TPF$

SP1$

SEC$.300  HIS-file (Create, Expand, Copy, Add, Gain-Shift, etc) (continued)

BLP$

DAMM can copy, add (or subtract) and gain-shift 1-D or 2-D

histograms from an input HIS-file to an output HIS-file.

ODP$

(1)....All operations are from an INPUT-file and INPUT-ID (IDI)

to an OUTPUT-file and OUTPUT-ID (IDO).

ODP$

(2)....For HCOP and HADD operations, the output histogram must have

the same dimensions and ranges as the input histogram.

ODP$

(3)....For SHIF (gain-shift) and SHAD (gain-shift & add) operations,

the dimensions of the output histogram need not match the input.

ODP$

(4)....The number of bits-per-channel (16 or 32) need not be the

same for input and output.

ODP$

(5)....Gain-shifts are accomplished by converting the data to

floating point, rebinning (with count fractionation) and finally

converting back to integer.

ODP$

(6)....Fractional copies and adds are also done in floating point.

ODP$

(7)....Final conversion from floating point to integer involves the

addition of a random number whose range is 0.0 to 1.0.

This procedure results in slight differences in the total number

of counts for the input and output histograms.

CYB$

==========================================================================

The commands given on the preceeding page may be used to create and/or

expand the output HIS- and DRR-files as needed.

==========================================================================

CY1$

HOU  FIL.HIS               - Opens HIS-file for output

SNEG OFF                   - Turn OFF reset of neg 16bit out data (default)

SNEG IV                    - Says set negative 16-bit output data to IV

- (you MUST use SIDA mode for this to work!!)

GSX  XI1,XI2 XF1,XF2       - Defines   X-gain-shift (described below)

GSY  YI1,YI2 YF1,YF2       - Defines   Y-gain-shift (described below)

GSXOF                      - Turns X-gain-shift OFF (GSX turns it ON)

GSYOF                      - Turns Y-gain-shift OFF (GSY turns it ON)

HSTA                       - Shows files open & gain-shift data

HCOP kf IDI,IDO <,F>       - Copies F*IDI (input) to IDO (output)

(If F is not entered, F=1)

\b\Where kf is an optional file-code field = N,O,P,Q,R,S (default = N)\\

CY1$

===========================================================================

For \b\HCOP\\: If \b\IDO\\ does not exist, it will be created with the same

attributes (same header data) as \b\IDI\\.

===========================================================================

CEN$

(continued on next page)

TPF$

SP2$

SEC$.300  HIS-file (Create, Expand, Copy, Add, Gain-Shift, etc) (continued)

CY1$

HADD kf IDI,IDO <,FI><,FO> - Adds FI*IDI to FO*IDO

(If FI is not entered, FI=1)

(If FO is not entered, FO=1)

(If FO is entered, FI must be entered)

HDIV kf IDI,IDO <,FI>      - Divides FI*IDI by IDO & saves in IDO

SHIF kf IDI,IDO <,FI>      - Gain-shifts IDI & stores in IDO

SHAD kf IDI,IDO <,FI><,FO> - Gain-shifts IDI & adds   to IDO

\b\Where kf is an optional file-code field = N,O,P,Q,R,S (default = N)\\

HSET IDO,IV                - Sets IDO on output to IV

HRAN IDO,IV                - Sets IDO on output to IV (counting statistics)

HZOT IDO                   - Sets IDO on output to 0

CEB$

X- and Y-Gain-shifts

CY1$

X-gain-shifts are specified by the parameters - XI1, XI2, XF1 & XF2.

Y-gain-shifts are specified by the parameters - YI1, YI2, YF1 & YF2.

BLP$

For an X-gain-shift,

XI1 and XI2 represent two locations (in channel # units)

in the initial spectrum and

XF1 and XF2 represent corresponding

locations in the final spectrum (i.e. after transformation).

that is:

CY1$

XF=A+B*XI

where:   B=(XF2-XF1)/(XI2-XI1)

and      A=XF1-B*XI1

BLP$

The "final" # of channels is determined by the "initial" # of

channels and the transformation specified.

Counts are redistributed into the final set of channels (bins) by assuming a

uniform distribution of counts in the initial bins.

Data shifted out of the range of the final histogram are lost and

gone forever!

BLP$

The rules and procedures are identical for Y-gain-shifts.

CEB$

COMMENTS

ODP$

(1)....If gain-shift specifications are not given (or turned off),

bin-widths will be the same for output and input.

ODP$

(2)....Any data which does not fall within the ranges of the output

histogram will be lost (without comment).

ODP$

(3)....Data will be properly positioned in the output histogram

even if the ranges of the input and output are different.

That is, data will appear in that region of the output histogram

which overlaps the gain-shifted input histogram.

TPF$

SP1$

SEC$.310  HIS-file (Adding Subtracting Complete HIS-files)

CYB$

Commands which add & subtract identically structured  his-files .............

BLP$

The following commands support the addition and subtraction of complete

his-files. The associated drr-files must have identical structures.

The files may contain any combination of 1D, 2D, INTEGER*2 & INTEGER*4

histograms

CY1$

-----------------------------------------------------------------------------

FADI   inpfile.his  ;Opens input  his-file - .his & .drr must already exist

;cannot be same as output

FADO   outfile.his  ;Opens output his-file - .his & .drr must already exist

;cannot be same as input but must have same structure

FADN   outfile.his  ;Creates & initializes   outfile.his & outfile.drr

;with same attributes as inpfile.his & inpfile.drr

;outfiles .his & .drr must NOT already exist

FADZ                ;Zeros outfile.his

FADF                ;Displays FAD files which are open

FADD                ;Adds      inpfile.his to   outfile.his

FSUB                ;Subtracts inpfile.his from outfile.his

CLIF                ;Closes input  FAD-file if open

CLOF                ;Closes output FAD-file if open

Ctrl/C              ;Interrupts (aborts) an ADD-in-progress

;Interrupts (aborts) a ZERO-in progress

-----------------------------------------------------------------------------

NOTE: The input & output file assignments here are NOT the same as those

assigned by the damm commands IN, OU & HOU. However, you may open these

files for display by commands such as IN, QF, etc.

For example, you may do the following:

fadi infile.his  !Open infile.his for add/subtract input

fado oufile.his  !Open oufile.his for add/subtract output

in   infile.his  !Open infile.his for normal display etc.

qf   oufile.his  !Open oufile.his for normal display etc.

-----------------------------------------------------------------------------

CHP$U300  DAMM - BANANAS - WTM

TPF$

SEC$.350  Bananas - Definition, Rules, Construction & Display

BLP$

Free-form-gates (or Banana-gates - Bananas for short) are

2-D regions of arbitrary shape which are specified by a list

of X,Y-points (channel-# coordinates).

Each Banana on a given BAN-file is stored and recalled by means of an

identification number (ID #).

Attempts to store two Bananas with the same ID will be rejected.

The rules for Bananas are listed below:

CY1$

(1) Banana coordinates nust be given in CLOCKWISE order.

(2) The Banana is formed by connecting X,Y-points with strait lines.

(3) The last point is connected to the first by the program.

(4) No line segment of the Banana should intersect another.

(5) A maximum of 63 points may be specified for any one Banana.

(6) A maximim of 880 Bananas may be stored on a given BAN-file.

CYB$

Bananas may be displayed in two different forms (OPEN and CLOSED).

BLP$

A CLOSED Banana is one which has just been recalled from or stored

on a BAN-file (i.e. there is an exact image on disk). There may be

up to 20 CLOSED Bananas displayed at once.

You can do the following things with a CLOSED Banana:

CY1$

GET      -  recall from disk (prompted for ID)     by typing  G

OPEN     -  for modification (change to OPEN)      by typing  O

FORGET   -  delete from display                    by typing  F

KILL     -  delete from display and BAN-file       by typing  K

TOTALIZE -  counts contained within Banana         by typing  T

PROJECT  -  (X & Y) and save on DAMQ8Q.SPK         by typing  P

BLP$

An OPEN Banana is one which is open for creation or

modification. If the Banana is being newly created there will be no

corresponding image or partial image on a BAN-file.

Only one such Banana can exist at a given time.

You can do the following things with a OPEN Banana:

CY1$

ADD      -  X,Y-point at cursor position           by typing  A

INSERT   -  X,Y-point at cursor position           by typing  I

MOVE     -  nearest X,Y-point to cursor position   by typing  M

SAVE     -  on BAN-file (prompted for ID)          by typing  S

REPLACE  -  on BAN-file (with old ID)              by typing  R

ZERO     -  all X,Y-points                         by typing  Z

TOTALIZE -  counts contained within Banana         by typing  T

PROJECT  -  (X & Y) and save on DAMQ8Q.SPK         by typing  P

BLP$

All Banana references are made in cursor mode. ADD, INSERT, MOVE,

SAVE, REPLACE and ZERO refer only to the OPEN Banana.

Other references (except for GET) are made by moving the cursor

such that it is closer to some point on the Banana of interest

than it is to any point on any other Banana.

CEB$

ALL BANANAS MUST BE CONSTRUCTED IN CLOCKWISE ORDER

TPF$

SEC$.360  Bananas - Projections

CYB$

Projections via the PJ-command

BLP$

Data which fall within and on the boundries of a Banana are projected

onto the X-axis of a coordinate system which is rotated through an angle

DEGR with respect to the systen in which data channel-# (0,0)

falls at the origin and the first and second indices of the histogram array

define the X- and Y-axis, respectively.

Channel-# XP in the projected histogram is calculated from channel-#

X,Y in the 2-D histogram by an expression of the following form:

DBL$

CY1$

XP=A+COS(DEGR)*X+SIN(DEGR)*Y

where,

A=0.0                               For DEGR =   0 -  90

A=-COS(DEGR)*XMAX                   For DEGR =  90 - 180

A=-COS(DEGR)*XMAX-SIN(DEGR)*YMAX    For DEGR = 180 - 270

A=-SIN(DEGR)*YMAX                   For DEGR = 270 - 360

SIN$

BLP$

XMAX and YMAX are the "dimensions" of the 2-D histogram.

The effect of this transformation is to make all channel numbers in the

projected histogram positive.

ODP$

NOTE: The "length" of the projected histogram may be as large as

SQRT(XMAX**2+YMAX**2).

CYB$

Projections via the P-command

BLP$

Each time DAMM is executed it will create a new version of the file

DAMQ8Q.SPK for the storage of projections.

The file is only created if projrctions are actually made.

BLP$

When you project a Banana, both X- and Y-projections are stored on

DAMQ8Q.SPK under the ID-numbers displayed. These 1-D histograms may

be displayed (or otherwise used) in the normal manner for a SPK-file.

Use the P-qualifier to display spectra from DAMQ8Q.SPK without

explicitally opening it. For example, to display ID numbers 1,3,5 from

DAMQ8Q.SPK, type:

CY1$

D P 1,3,5

CHP$U300  DAMM - FITTING - WTM

TPF$

SEC$.400  FITTING - Introduction & General Features

BLP$

You specify how fitting is to be carried out by supplying a number

of Fit Specification Data Sets which may be given in any order.

Many of these have default values (see SEC# U300.450).

After the fitting process is specified, one or more Fit Requests are

entered. Subsequently, some or all of the Fit Specifications may be changed and

more Fit Requests entered etc. etc.

CEN$

GENERAL FEATURES

ODP$

(1)....Fit specifications may be entered interactively or read

from a file or a combination of the two methods may be used.

ODP$

(2)....Peak and background intensities are determined in a weighted linear

least-squares fit while peak positions, widths, and asymmetry

parameters are determined by a non-linear least-squares search

(either Gradient search or Gauss method -

See SEC#s U300.490 and U300.500).

ODP$

(3)....Peak positions may be typed in or selected interactively

or found automatically.

ODP$

(4)....Spectra are fitted one section at a time and

can be no more than 2048 channels in length.

ODP$

(5)....In the gradient search mode (FIT command),

each section may contain a total of 30 peaks and background terms.

That is, the number of linear coefficients to be determined

in the linear least squares fit (# of peaks plus # of background

terms) may not exceed 30.

In the Gauss mode, only 5 peaks are allowed and asymmetry is not

supported.

ODP$

(6)....Initial values of peak positions, widths and asymmetry parameters

must be specified by the user.

Different values of width and asymmetry may be assigned to each peak

or all peaks may be assigned the same values.

ODP$

(7)....The FWHM for peaks in a section may vary independently, conditionally,

or be held fixed.

All peaks in a section may be forced to have the same width

or fixed relative widths.

ODP$

(8)....Peak positions may be adjusted or held fixed.

ODP$

(9)....Peaks may be gaussian or asymmetric (see SEC# U300.480 & U300.540

ODP$

(10)...The background may be specified (by up to 50 X,Y-points) or be

determined in the fit.

If determined in the fit, the background takes the form,

Y = A + B*X + C*X*X + D*X*X*X + . . . .

with the number of terms in the power series specified by the user.

ODP$

(11)...The output includes the Fit Specification Data, peak positions,

widths, energies, areas and uncertainties (in percent) as well as a printer

plot of the fit on a 0.5 to 5 cycle plot.

TPF$

SEC$.410  Commands for 1-key (cursor mode)

DBL$

CY1$

One-Key cursor commands (valid following a DS or DSX command)

Type: C  - To enter cursor-mode

P/UP   Add/Delete peak to Library (pos specified by cursor)

M/M    Move nearest displayed peak to cusor pos (FW, ASYM unchanged)

X/UX   Fix/Free  peak position (for displayed peak nearest to cursor)

W/UW   Fix/Free  peak width    (for displayed peak nearest to cursor)

L/UL   Fix/Free  Lo-Side ASYM  (for displayed peak nearest to cursor)

H/UH   Fix/Free  Hi-Side ASYM  (for displayed peak nearest to cursor)

O/UO   Turn peak  ON/OFF       (for displayed peak nearest to cursor)

B/UB   Add/Delete background point at cursor position

<--    Set Expand Region   Lo-Limit

-->    Set Expand Region   Hi-Limit

[      Set Fit Region      Lo-Limit

]      Set Fit Region      Hi-Limit

V      Make Expand- & Fit-region markers visible/invisible (toggles)

J      Junk (delete) all expand- & fit-region markers

/ or ? Display chan#, cursor Y-value, chan contents

S      Disp sum, cent & fwhm of Fit-Reg - DAT([),DAT(]) defines BGD

A      Disp sum, cent & fwhm of Fit-Reg - CUR([),CUR(]) defines BGD

Q      Return from cursor control routine

E      Expand display (region defined by <--  -->)

See SEC# U300.110 or Type: \b\h mous\\ for use of Mouse Buttons.

SIN$

TPF$

SEC$.420  Setup Commands

CYB$

Commands for entry of peak, background & skip-regions ---------------------

CY1$

PZOT                   - Zero the Peak Library

PK     X,W,ASLO,ASHI   - List of complete peak specifications

BZOT                   - Delete Fixed Background array

BACK   X1,Y1 X2,Y2 ..  - X,Y-points for fixed background

SKIP                   - Without List turns SKIP OFF

SKIP   I1,I2 J1,J2 ..  - Up to 4 regions to omit from Fit

CYB$

Commands for defining FWHM, ASYM, WLIM, ALIM, NBC, WOOD, ECAL -------------

CY1$

FW     FWA,FWB,FWC     - Coefficients for standard width function

WLIM   FWLO,FWHI       - Variation limit factors for peak widths

ASYM   ASLO,ASHI       - Standard Lo-Side and Hi-Side asymmetries

ALIM   FALO,FAHI       - Variation limit factors for peak asymmetries

NBC    NBC             - Number of power series terms in variable BGD

WOOD   ON/OFF          - Turn Woods-Saxon BGD term ON/OFF (default OFF)

- ON creates an additional background component

- with a Woods-Saxon "jog" under each peak which

- is porportional to the peak intensity.

ECAL   ECO,ECA,ECB     - Coefficients for standard energy calibration

CYB$

Commands for control of non-linear parameter variation --------------------

CY1$

DPX    XSTEP,DXMAX     - Initial step size and limit for peak pos

DEL    DEL,DELFAC,NDEL - Initial step size, step size multiplier and

- number of DEL-values to use

VB                     - Use Variable Background (the default)

FB                     - Use Fixed Background if available

VX     KVAR            - Kind of variation for peak positions

VW     KVAR            - Kind of variation for peak widths

VALO   KVAR            - Kind of variation for Lo-Side asymmetries

VAHI   KVAR            - Kind of variation for Hi-Side asymmetries

KVAR = UIND     - says vary Unconditionally, Independently

= CIND     - says vary   Conditionally, Independently

= ULOC     - says vary Unconditionally, Locked

= CLOC     - says vary   Conditionally, Locked

= FIX      - says keep Fixed - this the default assignmemt

\b\Conditional\\ says hold Fixed if peak so specified.

\b\Unconditional\\ says vary regardless of peak specifications.

\b\Independent\\ says given parameter-types are varied independently.

ODP$

\b\Locked\\ says given parameter-types (width for example)

are varied together (multiplied by the same factor) in the non-linear search.

TPF$

SEC$.430  Display of Data, Fits and Printer-plots

CYB$

Commands for general display control --------------------------------------

CY1$

FIG    N           - Select screen configuration number-N

WIN    N           - Select window-N for subsequent displays

LIN/LOG            - Set graphic display to LIN (default) or LOG

DMM    YMIN,YMAX   - Set display-range (YMIN & YMAX)

DL     ILO,IHI     - Set display-range (channel# limits)

DS     ID          - Display spectrum# ID (range defined by DL)

DS     ID,ILO,IHI  - Display spectrum# ID (DL values replaced)

(MAX value of IHI-ILO = 4095)

DSX    ID          - Display Data defined by Expand Region

C                  - Enters 1-key cursor-mode

CYB$

Commands related to display of FITS ---------------------------------------

CY1$

MON/MOF            - Peak Markers ON/OFF for DF (default = ON)

DFI                - Set to display (DF ) DATA,FIT,BGD (default)

DPK                - Set to display (DF ) DATA,FIT,PEAKS,BGD

DPPB               - Set to display (DF ) DATA,FIT,(PEAKS+BGD),BGD

DF                 - Display Fit (channel-limits given by Fit-range)

DF     ILO,IHI     - Display Fit (channel-limits given by ILO,IHI)

DC     NPK         - Display Calculated peak #  NPK+ RESIDUAL

PRP    XLO,XHI     - Display peaks from Library in range XLO thru XHI

PRP                - Display all peaks from Library

PRB                - Display all fixed Bgd-points

FSTAT              - Display current fit-parameters

DR                 - List results of last Fit on VDT (terminal)

CYB$

Commands related to printer-plots of results ------------------------------

CY1$

KPPL   NONE        - Says do no printer plots

KPPL   FITS        - Says plot FITS only (the default)

KPPL   ALL         - Says plot FITS, COMPONENTS and RESIDUALS

PR                 - Print and Plot results of last Fit on printer

CYB$

Commands related to peak-finding ------------------------------------------

CY1$

FIND  BIAS,FWHM    - Enables  peak-finding (see SEC# U300.155)

FIND               - Enables  peak-finding with (BIAS=5, FWHM=5)

NOFI               - Disables peak-finding

BLP$

If FIND is enabled (see SEC# U300.155 for general details),

DAMM will do a peak find within the display region

each time a DS (or DSX) command is given. An attempt will then be

made to add the newly found peaks to the internal peak library.

If a newly found peak is closer than 0.5*FWHM channels to an existing

library peak, it will not be added. Finally, all library peaks will

be marked on the display in the usual manner. No distinction is made

between "found peaks" and "manually entered peaks".

TPF$

SEC$.440  FIT Execution Commands

CYB$

Commands for FIT execution ------------------------------------------------

CY1$

FIT    ID,ILO,IHI  - Fit Request - (non-linear gradient search)

GFIT   ID,ILO,IHI  - Fit request - (gaussian method)

RFIT   ID,ILO,IHI  - Resume FIT/GFIT start with Parms from last Fit

LFIT   ID,ILO,IHI  - Linear Fit - no non-linear search

- (Fit-range specified by ILO,IHI)

FIT    ID X        - Fit Range specified by cursors (Fit Region)

GFIT   ID X        - Fit Range specified by cursors (Fit Region)

RFIT   ID X        - Fit Range specified by cursors (Fit Region)

LFIT   ID X        - Fit Range specified by cursors (Fit Region)

Ctrl/C             - Terminates Fit-in-prograss

SEC$.450  FIT Parameters - Saving, Setting, Default

CYB$

Commands which save FIT parameters in memory library ----------------------

CY1$

SAV    I,J  - Save all Parms from peaks I thru J of last Fit in PK-LIB

SAX    I,J  - Save X-Parms   for  peaks I thru J of last Fit

SAW    I,J  - Save W-Parms   for  peaks I thru J of last Fit

SAL    I,J  - Save ASL-Parms for  peaks I thru J of last Fit

SAH    I,J  - Save ASH-Parms for  peaks I thru J of last Fit

(If I,J ommitted, indicated Parms from ALL peaks are saved)

CYB$

Commands which set FIT parameters -----------------------------------------

CY1$

SET- X1,X2          - Set STD WIDTH and ASYM for peaks in range X1-X2

values (defined by FWA, FWB, FWC, ASLO, ASHI)

SETW X1,X2          - Set WIDTH for peaks in range X1-X2 to STD value

SETW X1,X2,WA,WB,WC - Set WIDTH for peaks in range X1-X2 to value

defined by WA,WB,WC (FWA,FWB,FWC unchanged)

SETL X1,X2          - Set ASLO  for peaks in range X1-X2 to STD value

SETL X1,X2,ASLOT    - Set ASLO=ASLOT for peaks in range X1-X2

SETH X1,X2          - Set ASHI  for peaks in range X1-X2 to STD value

SETH X1,X2,ASHIT    - Set ASHI=ASHIT for peaks in range X1-X2

(If X1,X2,.. omitted, indicated Parms for ALL peaks are set)

CYB$

List of default FIT parameters --------------------------------------------

CY1$

DEL   = 0.05    FWLO = 0.5      VX   = CIND    NBC  = 2

DELFAC=0.25     FWHI = 2.0      VW   = CLOC    WOOD = OFF

NDEL  = 3       FALO = 0.5      VALO = FIX     KPPL = FITS

XSTEP = 0.2     FAHI = 2.0      VAHI = FIX

DXMAX = 5.0     ASLO = 0.0

ASHI = 0.0

TPF$

SEC$.460  Commands to Control Relative Peak-intensities

BLP$

The following commands may be used to \b\fix\\ the intensity

of two or more peaks relative to each other within a section

being fitted:

CY1$

RELI  X,R   Sets relative intensity of library-peak nearest chan-X to be R

RELI  ZOT   Deletes all relative intensity entries

RELI  OFF   Disables relative intensity control but saves previous entries

RELI  ON    Enables  relative intensity control (default)

BLP$

\b\NOTE:\\ RELI specifications must be entered \b\AFTER\\ the

associated peak library entries are completed.

RELI has no effect unless there two or more peaks with

specified relative intensities within the region being fitting.

\b\R \\may be in any

units however there is an 8 character limit on the number entered!

Peak-areas will be in the same ratios as the

relative intensities specified ONLY if width

& asymmetry parameters are the same for each peak.

SEC$.470  FIT Specification Details

CEB$

PK  Data Set - Complete Peak Specifications

BLP$

The PK Data Set accomodates a full specification of the

characteristics of each individual peak.

Up to 100 peaks may be included in the list.

Each peak is specified by the following parameters.

ODP$

X......Gives the initial peak position in channels.

ODP$

W......Specifies the initial peak FWHM in channels.

If not entered, FWHM is set to the value specified by FWA, FWB & FWC.

ODP$

ASLO...Specifies the Lo-Side asymmetry parameter.

ODP$

ASHI...Specifies the Hi-Side asymmetry parameter.

CEB$

Other Specifications

ODP$

ECO,ECA,ECB...Defines the spectrum energy calibration (not required for

fitting) through the relation;

CY1$

E = ECO + ECA*(CHAN #) + ECB*(CHAN #)**2

ODP$

FWA,FWB,FWC...Defines the peak WIDTH as a function of channel number

through the relation;

CY1$

FWHM(CHANNELS)=FWA+FWB*SQRT(CHAN #)+FWC*(CHAN #)

ODP$

ASLO,ASHI...Are the initial values of the Lo-Side and Hi-Side

asymmetry parameters.

If this specification is used, the initial values will be the same for all

peaks.

ODP$

FWLO...Is the minimum fraction of the initially specified value by which any

peak width may be reduced.

ODP$

FWHI...Is the maximum fraction of the initially specified value by which any

peak width may be increased.

CEN$

(continued on next page)

TPF$

SEC$.470  FIT Specification Details (continued)

ODP$

FALO...Is the minimum fraction of the initially specified value by which any

peak asymmetry parameter may be reduced.

ODP$

FAHI...Is the maximum fraction of the initially specified value by which any

peak asymmetry parameter may be increased.

ODP$

DEL....Specifies the fraction by which the peak width and the peak

asymmetry parameters are to be changed in each step of the non- linear

search.

For example,

CEN$

(NEW WIDTH) = (OLD WIDTH)*(1.0+-DEL)

ODP$

DELFAC-Is a factor by which the current value of DEL is multiplied

in order to obtain a new (smaller) value.

Typically one starts with a fairly large value of DEL (say 0.02 to 0.05) and

subsequently makes one or more reductions in order to achieve a greater

convergence speed.

ODP$

NDEL...Is the number of DEL-values to be used

ODP$

XSTEP..Is the maximum amount (in channels) that a peak may be moved

in any one step in the non-linear search for the best fit.

XSTEP is reduced at the same time and by the same factor (DELFAC) that

DEL is reduced.

ODP$

DXMAX..Is the maximum number of channels (either + or -) that any peak is

allowed to be moved from its original position.

ODP$

SKIP...Defines up to four regions within the Fit Range which are

to be ignored in doing the fit.

CY1$

KPPL=..NONE says do no printer plots.

ODP$

KPPL=..FITS says plot the FIT (experimental and calculated spectrum on the

same graph).

ODP$

KPPL=..ALL says plot the FIT (as in KPPL...=FITS) and in addition, plot

each component (calculated peak) together with the corresponding

"residual component".

What do you mean by residual component, you ask.

When plotting the Ith peak we calculate the Ith residual component

by subtracting any background (specified or calculated) as well as all

calculated peaks other than the Ith from the experimental spectrum.

ODP$

NBC....Denotes the number of background components to be included

in the power series.

NBC=2 Says use the form Y=A+B*X and NBC=4 says use Y=A + B*X

+ C*X*X + D*X*X*X.

The number of peaks in a section plus NBC must not exceed 30.

CEN$

(continued on next page)

TPF$

SEC$.470  FIT Specification Details (continued)

ODP$

WOOD...ON/OFF says turn Woods-Saxon background term ON/OFF. The default

is OFF. If WOOD is ON, an additional background component is included

which has a Woods-Saxon type "jog" under each peak which is

porportional to the peak intensity. The jog form is given by:

CY1$

Y    =  1.0/(1.0+EXP(ARG))    ;where

ARG  =  4.714*(X0-X)/FWHM     ;and

X0=peak-position, X=channel-of-interest, and FWHM=peak-FWHM.

FIP$

The use of such a background form could be helpful in the analysis of

weak peaks which are located on the low-energy side of strong peaks.

You will have to be the judge.

SEC$.480  Discussion of Peak Shapes

CY1$

The most general peak shape allowed is given by

CEN$

YL=EXP(-(X-XO)**2/(A**2*(1+ASLO*(XO-X)/A)

YH=EXP(-(X-XO)**2/(A**2*(1+ASHI*(X-XO)/A)

BLP$

Where A is the gaussian Half-Width at 1/e max and YL and YH

describe the curve on the Lo- and Hi-Sides, respectively.

If all asymmetry parameters are held to zero, the shape is gaussian.

The ASLO/ASHI parameters broaden the Lo/Hi sides of the peak and result

in an expodential fall-off (like EXP(-(XO-X)/(A*ASLO)) for example)

As you move far away from the peak maximum (i.e. channel XO).

To get some idea of what size asymmetry parameters to use see Fig 1.

SEC$.490  Gradient-search Method (FIT request)

BLP$

Each time the program encounters a Fit Request, it searches the

complete Library and includes in the Fit all peaks which are ON and

whose positions lie within the Range of Fit (i.e. between ILO and IHI).

CEN$

GENERAL PROCEDURE FOR THE NON-LINEAR SEARCH

ODP$

(1)....The initial values of all parameters which are to vary in a non-linear

way are set to the initial values specified by the user.

ODP$

(2)....Each individual parameter is changed (both increased and decreased)

by an amount determined by DEL or XSTEP in order to establish a "direction"

(increase or decrease) for each parameter.

ODP$

(3)....All parameters are changed in the direction determined in step (2)

and in step sizes determined by DEL and XSTEP until the Quality of Fit is

no longer improved.

ODP$

(4)....Steps (2) and (3) are repeated until no improvment in the Fit can be made

ODP$

(5)....DEL and XSTEP are multiplied by DELFAC and steps (2) and (3) are repeated

until no improvment in the Fit can be made.

ODP$

(6)....Step (5) is repeated (NDEL-1) times.

TPF$

SEC$.500  Gaussian Method (GFIT request)

BLP$

The GFIT (Gauss-method) fit request initiates an alternate non-linear

procedure. Commands are:

CY1$

GFIT ID,ILO,IHI

or

GFIT ID X

BLP$

This command initiates a nonlinear least-squares search by Gauss'

method as modified by Marquardt.  (See, for example, P.R. Bevington's

book, "Data Reduction and Error Analysis for the Physical Sciences",

p. 235 ff.  The routines used in GFIT are not Bevington's, but

are those of M.J. Saltmarsh from the SEL 840-A program PKFT.)

BLP$

The search continues until chi-squared per degree of freedom

(QFN) has changed by less than 0.0001 or until 25 iterations have

occurred.  The search may be resumed by the RFIT command.  The

iteration number and QFN appear on the right side of the screen.

BLP$

The printer output from GFIT includes absolute error estimates

for the peak positions, widths, and areas which are derived from

the correlation matrix of the fit.  The percentage error in the

area is printed in the column labeled PCE.

BLP$

The DAMM commands VW ULOC or VW CLOC (the default option)

ae interpreted to mean that the widths of the peaks are to be the

same and vary together.  Widths may be varied independently by

VW UIND or VW CIND.  Individual widths or positions may be frozen

or released by the standard cursor commands of DAMM.  If one

or more widths are to be kept fixed while others are varied,

the command VW CIND or VW CLOC should be given; if UIND or ULOC

is given, the instruction to fix is ignored.

BLP$

If GFIT is chosen, the program attempts to estimate the width

of the tallest peak for its initial guess of width.  If unsuccessful

it reverts to the standard DAMM procedure of using whatever was

stored from the last previous FW, SETW commands (or the default

option, which is FW = 5).

BLP$

At present The GFIT request is limited to fitting a sum of up to 5

Gaussian peaks with a linear background.  The parameters ASLO and

ASHI for asymmetric peaks are ignored.

TPF$

SEC$.510  Estimated Uncertainties in Peak Areas

BLP$

Estimated uncertainties should always be viewed with

considerable skepticism, especially when non-linerar as

well as linear fitting processes are involved, as it is here.

The uncertainties in the peak areas estimated by both the

FIT and GFIT procedures are rather "standard" and involve

CHISQ (of the overall fit) as well the diagonal element of

the inverse matrix corresponding to the peak intensity in question.

This inverse matrix is found in the standard linear

least-squares fitting process. See a book like "Bevington"

or Cziffra et. al. UCRL-8523, 1958 for a real discussion of this

subject. I have used:

CY1$

D(J) = SQRT(AINV(J,J)*QFN)     where;

D(J)      = the estimated uncertainty in the J-th fit parameter B(J)

(there is a B(J) for each peak-area(J))

QFN       = CHISQ/(#data-points  -  #adjustable-parameters)

AINV(J,J) = the J-th diagonal element of the inverse matrix found in the

linear least-squares fitting process.

PCE(J)    = 100*D(J)/B(J)  =  percent uncertainty in J-th peak-area.

SEC$.520  Reading Fit Results from damm.log

BLP$

The table of fit-results recorded on damm.log, as a result of

a \b\PR\\ command, includes flags of the form \b\LAB$\\ to

facillitate the location and decoding of relevant data by other

programs. Formats associated with the different line-labels are

listed below.

CY1$

Label  Format

TIT$   (1X,15X,2I6,6X,20A4)

DEL$   (1X,2F10.0,I10,6F10.0)

SKP$   (1X,8I10)

CAL$   (1X,8F10.0,I10)

VAR$   (1X,6(6X,A4),I10)

FIT$   (1X,5F10.0,3F8.0,4F7.0,I5,I7)

GFI$   (1X,5F10.0,3F8.0,4F7.0,I5,I7)

BGD$   (1X,2F10.2)

QFN    (1X,6X,F10.0,11X,F10.0)

BLP$

Alternatively, one may make use of the routines contained in the

internally documented demonstration program \b\samred\\. The source

of this program is in \b\/usr/users/milner/Ddamm/samred.f\\.

SEC$.530  Common Problems

ODP$

(1)....If you define the standard FWHM (via command: FW FWA,FWB,FWC)

or standard asymmetry parameters (via command: ASYM ASLO,ASHI),

this does not re-define such parameters for previously defined peaks.

You must use SETW to do this (see SEC# U300.450).

TPF$

SEC$.540  Peak Shape vs Asymmetry (Log plot)

CY1$

YL=EXP(-(X-XO)**2/(A**2*(1+ASLO*(XO-X)/A)

YH=EXP(-(X-XO)**2/(A**2))

Y = 100  ------------------------------------------------------------------

SYMBOL  ASLO                            0

0   0.0                          11 00

2   0.2                         1     0

4   0.4                        1

6   0.6                       10       0

8   0.8                      14

1   1.0                     140         0

162

FWHM = 12                    1640           0

18 2

1864

20             0

1864

186

1   420               0

1 86

8  4

1  6  20                 0

1 8  4

1 8 6  2

1 8 6 4  0                   0

1

8 6 4 2

Y = 10  ------------------- 1 8 -------------------------------------------

1    6     0                     0

1  8    4 2

6

1  8    4

1  8  6    2

0                       0

1  8  6  4

1

1   8  6     2

8      4

1               0                         0

1   8   6  4

2

1   8   6

1           4

1    8   6

8           2  0                           0

1        6   4

1    8

6

1    8        4   2

1         6

8                0                             0

1    8         4

Y = 1   -------------------------------------------------------------------

CEN$

Figure 1

CHP$U300  DAMM - CUSTOMIZING - WTM

TPF$

SEC$.550  Screen Configurations

BLP$

The sizes and locations of display windows are controlled by one

or more of the commands - FIG, FIGF or FIGI.

CY1$

FIG ID - Sets the number and size of display windows to that specified

by configuration number ID. A given configuration ID number

may specify up to 20 display windows. The default configuration

library contains ID-numbers 1 thru 16. Type: H FIG for a

display indicating the default sceen configurations or just

Type: FIG 1, FIG 2 ... FIG 16 and see what you get.

FIGF FILENAME - Requests that a new configuration library be read from

a file named FILENAME. The file \b\/usr/hhirf/fig.dat\\ (or the file

\b\/home/upak/fig.dat\\) which contains the default configuration is

listed on the next next two pages. The **** in col-1 of the table

denote comment lines and are ignored in processing.

CY1$

****DEFAULT SCREEN-CONFIGURATION TABLE

****--------------------------------------------------

****FIG-ID   X0(PIX)   Y0(PIX)    W(PIX)    H(PIX)

1        30        84       540       480

****

2         0        39       480       480

2       495        39       480       480

****

3        30       369       540       300

3        30        39       540       300

3       585        39       390       390

****

4         0       369       480       300

4         0        39       480       300

4       495       369       480       300

4       495        39       480       300

****

5        30       489       480       195

5        30       264       480       195

5        30        39       480       195

5       525        39       435       435

****

6         0       489       480       195

6         0       264       480       195

6         0        39       480       195

6       495       489       480       195

6       495       264       480       195

6       495        39       480       195

****

7        30       534       480       135

7        30       369       480       135

7        30       204       480       135

7        30        39       480       135

7       525        39       435       435

(continued on next page)

TPF$

CY1$

****DEFAULT SCREEN-CONFIGURATION TABLE (continued)

****--------------------------------------------------

****FIG-ID   X0(PIX)   Y0(PIX)    W(PIX)    H(PIX)

8         0       534       480       135

8         0       369       480       135

8         0       204       480       135

8         0        39       480       135

8       495       534       480       135

8       495       369       480       135

8       495       204       480       135

8       495        39       480       135

****

9        30       579       480       105

9        30       444       480       105

9        30       309       480       105

9        30       174       480       105

9        30        39       480       105

9       525        39       435       435

****

10         0       579       480       105

10         0       444       480       105

10         0       309       480       105

10         0       174       480       105

10         0        39       480       105

10       495       579       480       105

10       495       444       480       105

10       495       309       480       105

10       495       174       480       105

10       495        39       480       105

****

11         0        39       960       525

****

12         0       354       960       270

12         0        54       960       270

****

13         0       489       960       195

13         0       264       960       195

13         0        39       960       195

****

14         0       549       960       135

14         0       384       960       135

14         0       219       960       135

14         0        54       960       135

****

15       240        39       750       750

****

16         0        39       645       645

---------------------------------------------------------------------------

FIG-ID  = Configuration ID number to be associated with window.

X0(PIX) = X-coordinate of upper left corner of window in pixels.

Y0(PIX) = Y-coordinate of upper left corner of window in pixels.

W(PIX)  = Width  of window in pixels.

H(PIX)  = Height of window in pixels.

---------------------------------------------------------------------------

FIGI   Restores the configuration library to the default state.

TPF$

SEC$.560  Graphic Screen Color Mapping

BLP$

The colors (or grey scale) is controlled by means of a color map

which has 40 entries that specify (red, green, blue) intensities

in the range 0-65535.

The default table contained in \b\/usr/hhirf/cmap.dat\\ or in

\b\/home/upak/cmap.dat\\ is listed below:

CY1$

RED  GREEN   BLUE   ;ENTRY# - NORMAL USE ------------------------------

0      0      0  ;01 - BLACK

65535  65535  65535  ;02 - 1-D DISPLAY - COL(1) - FIT DATA

65535      0      0  ;03 - 1-D DISPLAY - COL(2)

0  65535      0  ;04 - 1-D DISPLAY - COL(3) - FIT CALC

0      0  65535  ;05 - 1-D DISPLAY - COL(4)

65535  65535      0  ;06 - 1-D DISPLAY - COL(5) - FIT BACK

65535      0  65535  ;07 - 1-D DISPLAY - COL(6)

0  65535  65535  ;08 - 1-D DISPLAY - COL(7)

65535  65535      0  ;09 - 1-D DISPLAY - COL(8)

65535  65535      0  ;10 - NOT USED FOR NOW

0      0  32767  ;11 - 2-D COLOR DISPLAY

0      0  65535  ;12 - 2-D COLOR DISPLAY

32767      0  32767  ;13 - 2-D COLOR DISPLAY

65535      0  65535  ;14 - 2-D COLOR DISPLAY

32767      0      0  ;15 - 2-D COLOR DISPLAY

65535      0      0  ;16 - 2-D COLOR DISPLAY

32767  32767      0  ;17 - 2-D COLOR DISPLAY

65535  65535      0  ;18 - 2-D COLOR DISPLAY

32767  32767  32757  ;19 - 2-D COLOR DISPLAY

65535  65535  65535  ;20 - 2-D COLOR DISPLAY

15000  15000  15000  ;21 - 2-D GREY-SCALE DISPLAY

20600  20600  20600  ;22 - 2-D GREY-SCALE DISPLAY

26200  26200  26200  ;23 - 2-D GREY-SCALE DISPLAY

31800  31800  31800  ;24 - 2-D GREY-SCALE DISPLAY

37400  37400  37400  ;25 - 2-D GREY-SCALE DISPLAY

43000  43000  43000  ;26 - 2-D GREY-SCALE DISPLAY

48600  48600  48600  ;27 - 2-D GREY-SCALE DISPLAY

54200  54200  54200  ;28 - 2-D GREY-SCALE DISPLAY

59800  59800  59800  ;29 - 2-D GREY-SCALE DISPLAY

65535  65535  65535  ;30 - 2-D GREY-SCALE DISPLAY

32767  32767  32767  ;31 - NOT USED FOR NOW

32767  65535  65535  ;32 - NOT USED FOR NOW

65535      0      0  ;33 - NOT USED FOR NOW

65535  65535  65535  ;34 - GCOR(1)

65535      0      0  ;35 - GCOR(2), SAM PK MARK,  FIT VAR MARK

0  65535      0  ;36 - GCOR(3), 1-D PK LAB, 2-D BAN & EX-MARK

0      0  65535  ;37 - GCOR(4), 1-D REG MARK, FIT VAR MARK

65535  65535      0  ;38 - GCOR(5), CURSOR

65535      0  65535  ;39 - GCOR(6)

0  65535  65535  ;40 - GCOR(7)

BLP$

Different color mapping is accomplished by the CMAP command as

shown below:

CY1$

CMAP FILENAME  ;Processes a file FILENAME of the structure shown above

;and maps as  specified  therein.  The new mapping only

;takes place subsequent to the next FIG command.

TPF$

SEC$.570  Graphic Screen Black & White Mapping

BLP$

When using black & white monitors, the table \b\/usr/hhirf/bmap.dat\\,

(or \b\/home/upak/bmap.dat\\) listed below, may be more

useful. If you are using the REVV mode, then table

\b\/usr/hhirf/bmapr.dat or /home/upak/bmapr.dat\\, not listed here,

should be used as a template.

You will probably need to make other adjustments in order to achieve

semi-satisfactory results. Note: that table entries are labeled with

their uses.

CY1$

RED  GREEN   BLUE   ;ENTRY# - NORMAL USE ------------------------------

0      0      0  ;01 - BLACK

65535  65535  65535  ;02 - 1-D DISPLAY - COL(1) - FIT DATA

65535  65535  65535  ;03 - 1-D DISPLAY - COL(2)

65535  65535  65535  ;04 - 1-D DISPLAY - COL(3) - FIT CALC

65535  65535  65535  ;05 - 1-D DISPLAY - COL(4)

65535  65535  65535  ;06 - 1-D DISPLAY - COL(5) - FIT BACK

65535  65535  65535  ;07 - 1-D DISPLAY - COL(6)

65535  65535  65535  ;08 - 1-D DISPLAY - COL(7)

65535  65535  65535  ;09 - 1-D DISPLAY - COL(8)

65535  65535  65535  ;10 - NOT USED FOR NOW

0      0  32767  ;11 - 2-D COLOR DISPLAY

0      0  65535  ;12 - 2-D COLOR DISPLAY

32767      0  32767  ;13 - 2-D COLOR DISPLAY

65535      0  65535  ;14 - 2-D COLOR DISPLAY

32767      0      0  ;15 - 2-D COLOR DISPLAY

65535      0      0  ;16 - 2-D COLOR DISPLAY

32767  32767      0  ;17 - 2-D COLOR DISPLAY

65535  65535      0  ;18 - 2-D COLOR DISPLAY

32767  32767  32757  ;19 - 2-D COLOR DISPLAY

65535  65535  65535  ;20 - 2-D COLOR DISPLAY

15000  15000  15000  ;21 - 2-D GREY-SCALE DISPLAY

20600  20600  20600  ;22 - 2-D GREY-SCALE DISPLAY

26200  26200  26200  ;23 - 2-D GREY-SCALE DISPLAY

31800  31800  31800  ;24 - 2-D GREY-SCALE DISPLAY

37400  37400  37400  ;25 - 2-D GREY-SCALE DISPLAY

43000  43000  43000  ;26 - 2-D GREY-SCALE DISPLAY

48600  48600  48600  ;27 - 2-D GREY-SCALE DISPLAY

54200  54200  54200  ;28 - 2-D GREY-SCALE DISPLAY

59800  59800  59800  ;29 - 2-D GREY-SCALE DISPLAY

65535  65535  65535  ;30 - 2-D GREY-SCALE DISPLAY

32767  32767  32767  ;31 - NOT USED FOR NOW

32767  65535  65535  ;32 - NOT USED FOR NOW

65535      0      0  ;33 - NOT USED FOR NOW

65535  65535  65535  ;34 - GCOR(1)

45000  45000  45000  ;35 - GCOR(2), FIT PK MARK,  FIT VAR MARK

65535  65535  65535  ;36 - GCOR(3), 1-D PK LAB, 2-D BAN & EX-MARK

65535  65535  65535  ;37 - GCOR(4), 1-D REG MARK, FIT VAR MARK

65535  65535  65535  ;38 - GCOR(5), CURSOR

65535  65535  65535  ;39 - GCOR(6)

65535  65535  65535  ;40 - GCOR(7)

QIT$