RPD$075066001

CHP$U640  STOPE (Stopping Power Program) - WTM

TPF$

SP1$

CEN$

\BU\A Stopping Power Program With Graphical Display\\

CYB$

Sec Page Contents

010   2  Introduction

020   3  List of Commands

030   4  Definition of Output Quantities

040   5  References

SP1$

CEN$

\bu\Foreword\\

BLP$

Program \b\stope\\ estimates the stopping power for any ion in any

target element by the three different methods given below.

ODP$

o......Using the formulas and coefficients given by J.F. Ziegler

in Ref 1.

ODP$

o......Using the method and parameters of Northcliffe and

Schilling from Ref 2.

ODP$

o......Using D. Ward's effective charge (Ref 3) and Ziegler's proton

stopping power (Ref 1).

ODP$

o......Shell correction factors for Northcliffe's values are computed

using the Ward's tables and formulas given in Ref 3 and 4.

ODP$

o......Computation results are presented in both tabular and graphical

form.

SP1$

CEN$

\bu\How to Get Started\\

CYB$

---------------------------------------------------------------------------

Type:  stope              ;To start if \b\/usr/hhirf/\\ is defined in your

;.login, .cshrc or login.com files, otherwise:

Type:  /usr/hhirf/stope   ;To start on HRIBF Alpha platforms

---------------------------------------------------------------------------

TPF$

SEC$.010  Introduction

BLP$

Program \b\stope\\ calculates tables of stopping power vs ion energy for any

ion in any elemental target (ZT = 1-92). It's main feature is

ease of use. For stopping powers, energy loss and ranges in compounds

and mixtures, see program \b\stopx\\ by T. C. Awes - SEC# U630.

CEB$

\bu\A Brief Description of Ziegler's Method Used Here\\

BLP$

Program \b\stope\\ uses the formulas and coefficients given by J. F. Ziegler

in - Stopping and Ranges of Ions in Matter, Vols 3 & 5, Pergamon Press,

1980. However, Ziegler uses somewhat more elaborate procedures in

calculating his final "curves" than are used here.

The nuclear stopping power (col labeled - SNUC) is calculated from

Formula 16 given in Vol 5.

CYB$

For Protons ===============================================================

CY1$

EPN=EKEV/AP                ;Energy per nucleon

SE=PSTOP(ZT,EPN)           ;Electronic stopping power

;(in units of eV/(10**15 atoms/cmsq))

CYB$

For He or Li ==============================================================

CY1$

EPN=EKEV/AP                ;Energy per nucleon

EFCR=ZEFCR(AP,ZP,ZT,EKEV)  ;Fractional effective charge ratio to that for

;protons at same velocity in target element

ZEF=EFCR*ZP                ;Fractional effective charge of ion

SE=ZEF*ZEF*PSTOP(ZT,EPN)   ;Electronic stopping power of ion in target

;(in units of eV/(10**15 atoms/cmsq))

CYB$

For Heavy Ions ============================================================

CY1$

EPN=EKEV/AP                ;Energy per nucleon

EFCR=HICRAT(AP,ZP,ZT,EKEV) ;Fractional effective charge ratio to that for

;protons at same velocity in target element

ZEF=EFCR*ZP                ;Fractional effective charge of ion

SE=ZEF*ZEF*PSTOP(ZT,EPN)   ;Electronic stopping power of ion in target

;(in units of eV/(10**15 atoms/cmsq))

CY1$

Where:

ODP$

\b\PSTOP..\\is a routine which calculates proton stopping power in any

target element (ZT = 1-92) at energy per nucleon = \b\EPN\\. It uses

a table of coefficients (12 per target element) and formulas from Ref 1.

ODP$

\b\ZEFCR..\\is a special routine for \b\He & Li\\ which uses formulas

from Ref 1 to compute the fractional effective charge ratio of the

ion to that for protons at the same velocity in the same target element.

ODP$

ODP$

\b\HICRAT.\\is a routine for for heavy ions which uses formulas

from Ref 1 to compute the fractional effective charge ratio of the

ion to that for protons at the same velocity in the same target element.

TPF$

SP1$

SEC$.020  List of Commands

CYB$

Command        ;Meaning

-------        ;---------------------------------------------

h              ;Displays this list again

soli           ;Specifies SOLID  target - default

gas            ;Specifies GAS    target

doc            ;Displays/logs definitions & references

lon            ;Enables  output to stope.log- dflt

lof            ;Disables output to stope.log

lmon           ;Turns Lo-E modification ON - dflt

lmof           ;Turns Lo-E modification OFF

cmd  filename  ;Processes commands from filename

proj ion       ;ion = name like he4, ar40, u238, etc

targ ele       ;ele = name like 58Ni or H2o, CH2, Mylar

eion ea eb ec  ;Ion energy (MeV) ea to eb in steps of ec

;(if ea,eb,ec omitted - does preset list)

dedx           ;Generates DE/DX table using proj, targ, eion

end            ;Ends program

TPF$

SP1$

SEC$.030  Definition of Output Quantities

CY1$

All stopping powers are in units of MeV*CMSQ/MG

NUCL   = Nuclear stopping power - from Ziegler - Ref 1

WARZIG = S(ele) from Ward's effective charge & Ziegler's proton electronic

stopping power Ref 1,3

ZIEGL  = S(ele) from Ziegler - Ref 1

NORTH  = S(ele) from Northcliff & Schilling corrected for target mass

Ref 2

CNORTH = S(ele) shell-corrected NORTH - see SHELF - Ref 2,4

SHELF  = Shell correction factor = S(ele,He4)WARD/S(ele,He4)NORTH

Ref 2,4

WARHE4 = S(ele,He4) from Ward's table for MeV/Amu listed in Ref 4

Entries flagged by * computed from linear velocity dependance in

range of 0.0 to 0.10 MeV/Amu

T-WARZIG = WARZIG+NUCL

T-ZIEGL  = ZIEGL +NUCL

T-CNORTH = CNORTH+NUCL

Display-solid-line   = ZIEGL  (electronic)

Display-open-circle  = WARZIG (electronic)

Display-close-circle = NORTH  (electronic)

SP1$

SEC$.040  References

CY1$

1)  J. F. Ziegler, The Stopping and Ranges of Ions in Matter, Vols 3 & 5,

Pergamon Press, 1980.

2)  L. C. Northcliff & R. F. Schilling, Nuc Data Tables, Vol 7, 1970.

(Range .0125 to 12.0 MeV/Amu

3)  D. Ward, et al, Stopping Powers for Heavy Ions, AECL-5313,

Chalk River, 1976.

4)  D. Ward, et al, Compilation of Realistic Stopping Powers for 4He Ions

in Selected Materials, AECL-4914, Chalk River, 1975

QIT$