RPD$075066

CHP$U310  LEMOR - Unix/Linux Version - WTM

TPF$

SP1$

CEB$

LEMOR (List-tape Examine, Modify, Output - Revised)

CYB$

Sec Page Contents

010   2  Introduction - General

012   3  Introduction - Definition of L001 List-Data Format

014   4  Introduction - Definition of L002 List-Data Format

016   5  Introduction - Definition of L003 List-Data Format

018   6  Introduction - Definition of List Data Files (LDFs)

020   7  Commands General

030   7  Command  File Operations

040   7  Commands to  Assign Tapes and Files

050   8  Commands for Tape & File Control

060   8  Commands to  Examine a Tape or LDF

070   9  Commands to  Display/Log Special Records from a Tape or LDF

080  10  Commands for Simple  Copy from (Tape of LDF) to (Tape or LDF)

090  11  Commands for Modify--Copy from (Tape or LDF) to (Tape or LDF)

100  12  Commands to  Copy Simulations from Disk to (Tape or LDF)

110  13  Commands to  Copy PAC-files from (Tape or LDF) to Disk

120  13  Commands to  Copy ASCII Files (Tape-to-Disk or Disk-to-Tape)

130  14  Discussion of Certain Commands

140  15  Discussion of Byte-Swapping

150  16  Discussion of Tape Conversion

160  17  Discussion of Prescan (Modify-Copy)

170  19  Discussion of Banana Gating (Free-Form-Gating)

180  20  How to Create a Customized Prescan Program

190  21  Lemor Routines Which aid in Customization

200  21  User  Routines Which aid in Customization

210  22  User  Routine -  USERCMP  - User Command Processor

220  22  User  Routine -  USERMOC  - User Processing of Raw Data

230  23  User  Routine -  USERMOC  - Example with Banana Gating

240  24  Lemor Routines which Unpack Events

250  26  Lemor Routines which Output Events

260  27  Lemor Routine  for Testing  1-D Gates

270  27  Lemor Routines for Testing  2-D Gates (Bananas)

300  28  Logical Units and COMMON Blocks

310  29  Processing of User Defined Files - UDFs - Linux Version Only!

CYB$

===========================================================================

Type:  lemor              ;To start with /usr/hhirf/ defined in your .login

;Otherwise:

Type:  /usr/hhirf/lemor   ;To start on HHIRF DECstation or Alpha

===========================================================================

TPF$

SP2$

SEC$.010  Introduction - General

BLP$

\b\lemor\\ is a tape processing utility which can be customized

(via user supplied routines) to produce a

prescan program. The stock version of \b\lemor\\ can do a number of

useful things: Some of these are:

CY1$

\b\Refer to\\

\b\--------\\

o....L001, L002 & L003 formatted data tapes may be copied to        \b\SEC#018\\

list-data-files on disk (called  LDFs) for subsequent          \b\SEC#080\\

processing with scanur histogramming programs.

o....Any  tape  or LDF containing records no longer than 32768      \b\SEC#060\\

bytes, may be examined (records read and displayed in various

formats).

o....Any tape or LDF may be copied to another tape or LDF           \b\SEC#080\\

o....Input data  records may be byte-swapped for compatibility      \b\SEC#140\\

with other platforms.

o....The text records from L002 or L003 formatted tapes or LDFs     \b\SEC#110\\

(which normally contain the Data Acquisition PAC program) can

be restored to disk.

o....In the modify-copy mode, the stock lemor can copy (tapes       \b\SEC#150\\

or LDFs) in data formats (L001, L002, L003) to another tape

or LDF in the same or different format.

o....The  usual  tape  control  functions  (forward  and  backward  \b\SEC#050\\

spacing of records and files, rewind etc.) are provided.

o....Simulated list-data files may be copied from disk to tape.     \b\SEC#100\\

CYB$

===========================================================================

See SEC#220 & 230 for discussion of the user-supplied customizing routine

USERMOC

===========================================================================

TPF$

SP1$

SEC$.012  Introduction - Definition of L001 List-Data Format

CYB$

L001 formatted list-data (often generated in external labs) consists of:

ODP$

o......16-bit data words.

ODP$

o......Fixed length data records which are no shorter than 2048 bytes and

no longer than 32768 bytes.

ODP$

o......A fixed number of parameters per event.

ODP$

o......A fixed (integer) number of events per record (no event splitting

across record boundries).

ODP$

o......A number of data-record header-words may be specified which are

to be ignored during processing.

ODP$

o......Header records of 256 bytes in length and structured as defined

in SEC#014 may be utilized for display, searches, etc.

ODP$

o......All other records not of the specified data-record length will be

ignored during processing.

TPF$

SEC$.014  Introduction - Definition of L002 List-Data Format

BLP$

The \b\L002 data stream\\ (from the old Concurrent system) consists of

Header records, Event Handler

Source Code (EVS) records, Data records and Scaler records - as

illustrated below:

CY1$

Header    record   ;256 bytes

EVS-file  record   ;1600 bytes

.

Data      record   ;8192 bytes

Data      record   ;8192 bytes

.          .

Scaler    record   ;6780 bytes

End-of-File        ;File mark

Header    record

Data      record

.          .

.          .

End-of-File        ;Double file-mark

End-of-File        ;ends all data on tape

BLP$

\b\L002 events\\ consist of parameter ID flags followed by one or more

data words corresponding to sequential parameter IDs  - as illustrated

below:

CY1$

8000-hex + ID     ;Parameter ID

Data              ;for Parameter ID

Data              ;for Parameter ID+1

.

8000-hex + JD     ;Parameter JD

Data              ;for Parameter JD

Data              ;for Parameter JD+1

.

FFFF-hex          ;End-of-event

CYB$

o......Events \b\may be split\\ across record boundries.

o......Data words cannot have the hi-order bit set.

o......All parameter IDs and data words are 16-bit.

CY1$

===========================================================================

\bu\Structure of the 256-byte header record\\

\bu\Header Word Number\\

16-BIT   32-BIT   # BYTES  CONTENTS                        TYPE

01 - 04  01 - 02         8  'HHIRF   '                      ASCII

05 - 08  03 - 04         8  'L002    '                      ASCII

09 - 16  05 - 08        16  'LIST DATA       '              ASCII

17 - 24  09 - 12        16  MO/DA/YR HR:MN                  ASCII

25 - 64  13 - 32        80  User Title                      ASCII

65 - 66       33         4  Header Number                   BINARY

67 - 70  34 - 35         8  Reserved (set to 0)             BINARY

71 - 72       36         4  # of Secondary Header Records   BINARY

37         4  Record Length (bytes)           BINARY

38         4  # Blocked Line Image Records    BINARY

39         4  Record Length (bytes)           BINARY

40         4  Parameters/Event (ref only)     BINARY

41         4  Data Record Length (bytes)      BINARY

42 - 64            Reserved (set to 0)             BINARY

TPF$

SP1$

SEC$.016  Introduction - Definition of L003 List-Data Format

SP1$

CYB$

The L003 data stream (currently in use) may consist of:

CY1$

Header    records - 256   bytes

PAC-file  records - 1600  bytes

Scaler    records - 32000 bytes

Dead-time records - 128   bytes

Data      records - 2048 to 32768 bytes (but not 32000 bytes)

CYB$

A typical data stream might look like:

CY1$

Header    record

PAC-file  record

PAC-file  record

Data      record

Data      record

.          .

.          .

Scaler    record

Data      record

.          .

.          .

Scaler    record

Dead-time record

End-of-File

Header    record

Data      record

.          .

.          .

End-of-File

End-of-File

BLP$

\b\L003 events\\ consist of pairs of 16-bit numbers, the first of

which defines the parameter ID and the second the data - as shown

below.

CY1$

8000-hex + ID     ;Parameter ID

Data              ;Parameter data

8000-hex + ID

Data

8000-hex + ID

Data

.

.

FFFF-hex          ;End-of-event word-1

FFFF-hex          ;End-of-event word-2

ODP$

o......Events are \b\not\\ split across record boundries.

ODP$

o......Unfilled records are padded with FFFF-hex.

TPF$

SEC$.018  Introduction - Definition of List Data Files (LDFs)

BLP$

The list data file (LDF) ia a structured disk file for containing

L001, L002 or L003 data. It is processed as a direct access file whose

record length is 8194 32-bit words.

CY1$

\bu\The General Record Structure is:\\

32-bit word  1       TYPE  - Record type (DIR, HEAD, PAC, SCAL, DEAD, DATA)

2       NFW   - number of 32-bit words of data

3-8194  DATA  - 8192 full-words (32768 bytes) data plus pads

\bu\Directory record structure:\\

32-bit word  1       'DIR '  - type = Directory

2       8192    - full-word data size

3       8194    - full-word blocksize

4       NREC    - number of records written on file

5               - un-used

6       NHED    - number of header records written on file

7       HED-ID  - header ID number

8       RECN    - record number where header is written

9       HED-ID  - header ID number

10       RECN    - record number where header is written

.

\bu\Header record structure\\

32-bit word  1       'HEAD'  - type = header

2        64     - number of full-words of header

3-66            - header record - 256 bytes

\bu\PAC record structure\\

32-bit word  1       'PAC '  - type = PAC

2        NFW    - number of full-words of data

3-NFW+2         - PAC file records (up to 409 lines)

\bu\Scaler record structure\\

32-bit word  1       'SCAL'  - type = Scaler

2        NFW    - number of full-words of data

3-NFW+2         - Data for up to 400 scalers

\bu\Deadtime record structure\\

32-bit word  1       'DEAD'  - type = Deadtime

2        32     - 128 bytes of deadtime data

3-34            - Deadtime data

\bu\Data record structure\\

32-bit word  1       'DATA'  - type = Data

2        8192   - number of full-words of data

3-8194          - list data plus padding

TPF$

SP1$

SEC$.020  Commands General

CY1$

H             ;Displays directory to list of commands

LON           ;Enable  output to log-file \b\lemor.log\\ (default)

LOF           ;Disable output to log-file \b\lemor.log\\

STAT          ;Displays/logs open tapes, LDFs, options, etc.

STAT GATE     ;Displays/logs currently defined gates

STX           ;Display/log tape status (MB-used, MB-left, Errors/MB)

;(for all tape units which are open)

UCOM TEXT     ;Send TEXT to USERCMP

END           ;Terminates program

SEC$.030  Command File Operations

CY1$

CMD  FIL.CMD  ;Assign FIL.CMD as CMD-file (not read yet)

CCMD          ;Continue reading  CMDS from CMD-file

CLCM          ;Continue with last CMD from file (backspaces)

CCON          ;Continue reading  CMDS from VDT (Terminal)

MSG  TEXT     ;Display TEXT (44 bytes) on VDT

Ctrl/C        ;Interrupts command file processing

SEC$.040  Commands to Assign Tapes and Files

CY1$

IN   rxxx     ;Specifies tape (rmxx) for INPUT

OU   ryyy     ;Specifies tape (rmyy) for OUTPUT

;rxxx, ryyy Denote rmt0:,rmt1:, rst0,  etc

INF  file.ldf ;Opens LDF \b\file.ldf\\ for input

UDF  filename ;Specifies input to be a special User Defined File

;(see SEC# 310) \bu\Linux Version Only!\\

OUF  file.ldf ;Opens/creates LDF \b\file.ldf\\ for output

ELDF          ;Erases output LDF (i.e. deletes & recreates)

GAT  file.gat ;Open and read 1-D gates from file.gat

GATZ          ;Reset all previously defined gates to "impossible"

BAN  file.ban ;Open file.ban for banana testing

BANZ          ;Zero banana archive

TPF$

SEC$.050  Commands for Tape & File Control

CY1$

RDI  N        ;Read    N records from INPUT

RDO  N        ;Read    N records from OUTPUT

FRI  N        ;Forward N records on   INPUT

FRO  N        ;Forward N records on   OUTPUT

BRI  N        ;Backup  N records on   INPUT

BRO  N        ;Backup  N records on   OUTPUT

FFI  N        ;Forward N files   on   INPUT

FFO  N        ;Forward N files   on   OUTPUT

BFI  N        ;Backup  N files   on   INPUT

BFO  N        ;Backup  N files   on   OUTPUT

RWI           ;Rewind                 INPUT

RWO           ;Rewind                 OUTPUT

BTI           ;Go to BOTTOM of INPUT  (to DBL EOF, Backup 1 F)

BTO           ;Go to BOTTOM of OUTPUT (to DBL EOF, Backup 1 F)

CLI           ;Close                  INPUT

CLO           ;Close                  OUTPUT

ULI           ;Unload and Close       INPUT  tape

ULO           ;Unload and Close       OUTPUT tape

SEC$.060  Commands to  Examine a Tape or LDF

BLP$

Open an input tape (or LDF) or an output tape (or LDF),

read one or more records and display (or log) data from the last

record read using one of the following commands:

CY1$

CLID IDH,IDL  ;Specify VME clock parameter IDs (hi & lo parts respectively)

;1st VME-clock entry, VMET, of each buffer displayed via RDI

;format is full decimal value displayed as xxx,xxx,xxx

;\bu\Linux Version Only!\\

CLID          ;Disables VMET search & display - \bu\Linux Version Only!\\

RDI  N        ;Read    N records from INPUT  (displays REC#, #bytes, VMET)

RDO  N        ;Read    N records from OUTPUT (displays REC#, #bytes, VMET)

PEV  IA,IB    ;Print 16-bit word IA thru IB in EVENT   Format (integer)

DEV  IA,IB    ;Disp  16-bit word IA thru IB in EVENT   Format (integer)

PEVZ IA,IB    ;Print 16-bit word IA thru IB in EVENT   Format (hex)

DEVZ IA,IB    ;Disp  16-bit word IA thru IB in EVENT   Format (hex)

PZ   IA,IB    ;Print 16-bit word IA thru IB in HEX     Format

DZ   IA,IB    ;Disp  16-bit word IA thru IB in HEX     Format

PA   IA,IB    ;Print 16-bit word IA thru IB in ASCII   Format

DA   IA,IB    ;Disp  16-bit word IA thru IB in ASCII   Format

PI   IA,IB    ;Print 16-bit word IA thru IB in INTEGER Format

DI   IA,IB    ;Disp  16-bit word IA thru IB in INTEGER Format

PIF  IA,IB    ;Print 32-bit word IA thru IB in INTEGER Format

DIF  IA,IB    ;Disp  32-bit word IA thru IB in INTEGER Format

TPF$

SEC$.070  Commands to  Display/Log Special Records from a Tape or LDF

CY1$

LISF filename ;Open output file for recording records

LISI          ;Display/log HEAD, DEAD, PAC, & SCAL records from

;INPUT tape or LDF

LISI SCAL     ;Display/log SCAL (scaler) records only

LISI TYPA TYPB .. ;Display/log record types TYPA, TYPB, etc. where,

;legal types are HEAD, DEAD, PAC, SCAL

LISO          ;Same function as LISI but for OUTPUT tape or LDF

===========================================================================

If output file is undefined (via LISF command), the specified records

will be recorded on lemor.log if LON.

===========================================================================

TPF$

SP1$

SEC$.080  Commands for Simple  Copy from (Tape of LDF) or (LDF or tape)

BLP$

\b\Tape-to-tape\\ copies are double buffered for speed. Also, the

maximum input record length is auto-detected unless you override

the auto-detect feature via the \b\RECI\\ command.

Output record lengths are the same as the number of bytes actually read from

the input.

BLP$

\b\Tape-to-disk\\ copies are tape-buffered for speed. Also, the

maximum input record length is auto-detected unless you override

the auto-detect feature via the \b\RECI\\ command.

Output record lengths are fixed at 32776 bytes (8 bytes of ID information

and 32768 bytes of data) to enable direct access to the file.

BLP$

\b\Disk-to-Tape\\ copies are un-buffered.

Output data record lengths are fixed at 32768 bytes.

BLP$

\b\Disk-to-disk\\ copies are un-buffered.

CY1$

===========================================================================

IN   rxxx     ;Specifies input  tape (rxxx denotes rmt0, rmt1, etc)

INF  file.ldf ;Specifies the input LDF \b\file.ldf\\

UDF  filename ;Specifies input to be a special User Defined File

;(see SEC# 310) \bu\Linux Version Only!\\

OU   ryyy     ;Specifies output tape (rmyy denotes rmt0, rmt1, etc)

OUF  file.ldf ;Specifies the output LDF \b\file.ldf\\

STX           ;Display/log tape status (MB-used, MB-left, Errors/MB)

;(for all tape units which are open)

ELDF          ;Erases output LDF (i.e. deletes & recreates)

STAT          ;Displays/logs open tapes, LDFs, options, etc.

RECI          ;Says max input record length be auto-detected (default)

RECI RECL     ;Sets max input record length to be \b\RECL\\

SWAB          ;Request    byte-swap of input buffers (SEC#140)

SWOF          ;Request no byte-swap (default)

SHON          ;Says byte-swap headers once more than data

SHOF          ;Says byte-swap headers & data the same way (default)

COPY N        ;Copy N files   from INPUT to OUTPUT

CREC N        ;Copy N records from INPUT to OUTPUT

CC            ;Continue COPY - saves file- or record-count

Ctrl/C        ;Interrupts copy process

EOF           ;Write EOF on OUTPUT (not normally needed)

TPF$

SEC$.090  Commands for Modify--Copy from (Tape or LDF) or (LDF to tape)

BLP$

Tape-to-tape modify-copies are double buffered for speed. Also, the

maximum input record length is auto-detected unless you override

the auto-detect feature via the \b\RECI\\ command.

Output record lengths are 32768 bytes by default but can be set to

smaller values via the \b\RECO\\ command.

CY1$

IN   rxxx       ;Specifies input  tape (rxxx denotes rmt0, rmt1, etc)

INF  file.ldf   ;Opens LDF \b\file.ldf\\ for input

OU   ryyy       ;Specifies output tape (rmyy denotes rmt0, rmt1, etc)

OUF  file.ldf   ;Opens/creates LDF \b\file.ldf\\ for output

STX             ;Display/log tape status (MB-used, MB-left, Errors/MB)

;(for all tape units which are open)

ELDF            ;Erases output LDF (i.e. deletes & recreates)

STAT            ;Displays/logs open tapes, LDFs & record pointers

FMTI L001 NS NP ;Specify non-standard           Input (see SEC#012)

FMTI L002       ;Specify old     HHIRF standard Input  format

FMTI L003       ;Specify current HIRBF standard Input  format (default)

FMTO L001 NS NP ;Specify non-standard           Output (see SEC#012)

FMTO L002       ;Specify old     HHIRF standard Output format

FMTO L003       ;Specify current HIRBF standard Output format (default)

RECI            ;Says max input record length be auto-detected (default)

RECI RECL       ;Sets max input record length to be \b\RECL\\

RECO            ;Sets output data record length to 32768 bytes (default)

RECO RECL       ;Sets output data record length to \b\RECL\\  bytes

;(Allowed range is 2048 to 32768 bytes)

UPON NPRAW      ;Turn User-processing ON (see SEC#220)

;NPRAW = Max # of raw parameters (for user only)

UPON            ;Turn User-processing ON (NPRAW=0) (default)

UPOF            ;Disable User-processing (enable CHIL processing)

MILF file.mil   ;Read & process CHIL generated \b\mil-file\\

UCOM TEXT       ;Send TEXT to USERCMP

SWAB            ;Request    byte-swap of input buffers (SEC#140)

SWOF            ;Request no byte-swap (default)

SHON            ;Says byte-swap headers once more than data

SHOF            ;Says byte-swap headers & data the same way (default)

INIT            ;Resets Modify-Copy Input & Output buffers

ZBUC            ;Zero total Input & Output buffer counters

MOC  N,M        ;Modify-Copy (N-files/M-recs - 1st to occur)

MOCE N,M        ;Modify-Copy (END on request complete)

Ctrl/C          ;Interrupts Modify-Copy process

TPF$

SEC$.100  Commands to  Copy Simulations from Disk to (Tape or LDF)

BLP$

\b\lemor\\

accomodates the examining and copying of disk-files containing

event-list data. This feature is intended to be an aid to those

doing simulations. The following commands are available.

CY1$

RECI RECL     ;Sets input  data record length to be RECL

RECI          ;Sets input  data record length to be default (32768)

RECO RECL     ;Sets output data record length to RECL  bytes

RECO          ;Sets output data record length to be default (32768)

INEV filname  ;Specify input file for exam (RDI, DEV) & copy

OU   ryyy     ;Specifies output tape (rmyy denotes rmt0, rmt1, etc)

OUF  file.ldf ;Opens/creates LDF file.ldf for output

STAT          ;Displays/logs open tapes, evel-files & record pointers

SWAB          ;Request    byte-swap of input buffers (SEC#140)

SWOF          ;Request no byte-swap (default)

HTIT  TITLE   ;TITLE contains title for next tape header

HNUM  HN      ;HN specifies next tape header number to use

HOUT          ;Outputs tape header and increments HN

COPY  1       ;Copies one file input-to-output

Ctrl/C        ;Interrupts copy process

BLP$

In addition, an example program found in

\b\/usr/users/milner/Develx/evelx.f\\

contains routines for opening an event-file (EVELOPEN) and for

writing data to it (EVELOUT). The "simulator" might (or might not)

wish to include these routines in his simulation source code. Feel

free to copy \b\evelx.f\\ to your directory for examination etc.

The routines are internally documented.

The idea is this:

ODP$

o......You do your simulations and write the generated events to a file.

ODP$

o......Use \b\lemor\\ to examine this file (read, display, etc)

ODP$

o......You may also specify a tape header number and title,

output the header and finally copy the entire file to tape or LDF.

BLP$

An typical file-to-LDF copy session might look as follows:

CY1$

lemor>\b\inev  eventfile.dat           \\;open event-file for input

lemor>\b\ouf   eventfile.ldf           \\;open LDF for output

lemor>\b\htit  simulation-3 no-gates   \\;title for header

lemor>\b\hnum  3                       \\;next header number to use

lemor>\b\hout                          \\;output the header

lemor>\b\copy  1                       \\;copy 1-file input-to-output

BLP$

\b\NOTE:\\ You can also use the header setup and output feature (hnum,

htit & hout commands) to add headers while copying one tape to another.

There is no provision, however, to delete or modify existing headers.

TPF$

SP2$

SEC$.110  Commands to  Copy PAC-files from (Tape or LDF) to Disk

SP1$

CY1$

\bu\Commands to Copy a PAC-file From Input to Disk\\

CY1$

STEX filname  ;Store text records (PAC source) on \b\filename\\

SP2$

SEC$.120  Commands to  Copy ASCII Files (Tape-to-Disk or Disk-to-Tape)

CY1$

FCOP filename ;Copies \b\filename\\ to output-tape (previously opened)

;Variable length records from \b\filename\\ are de-tabbed and

;written as fixed length (80 byte) records on tape.

;(DEC's rules for Fortran tabs are used in de-tabbing)

TCOP filename ;Copies 1 file from input-tape to \b\filename\\ (created)

;Fixed length (80 byte) records from tape are

;written as variable length records on \b\filename\\.

TPF$

SP1$

SEC$.130  Discussion of Certain Commands

ODP$

FIND...N attempts to find HEADER # N by searching forward on the

INPUT tape (TITLES and HEADER numbers are displayed along the way):

If found, \b\lemor\\ backs up one record.

If  not found, \b\lemor\\ will read past the first

Double-EOF, and back up one File Mark.

ODP$

BTO....Advances OUTPUT past the first Double-EOF and backs

up one File Mark (i.e. positions properly for appending).

Header titles and numbers are displayed along the way.

ODP$

BTI....Does the same thing for the INPUT tape.

ODP$

DEV....PEV, DA, PA, DZ, PZ, DI, PI, DIF & PIF

are all commands which display on the terminal

or list on the printer some portion of the last record read from

tape (either the INPUT or OUTPUT). When you do a read (RIN or ROU),

\b\lemor\\ tells you how many bytes were read but you must specify the

portion of the buffer to be displayed in half-words (16-bit words).

FIP$

In EVENT FORMAT (DEV or PEV), \b\lemor\\ looks for a hex FFFF before

starting to accumulate the first EVENT to be displayed.

Therefore, the first EVENT in any record is usually not displayed

by DEV or PEV (you can see it via DZ or PZ, however).

ODP$

COPY...N says COPY N-files from INPUT to OUTPUT. All that is

required is that records be no longer than 32768 bytes. File-marks

are copied and the OUTPUT tape is always positioned between Double

File-Marks on normal completion of a COPY request.

The OUTPUT tape is positioned ahead of Double File-Marks on

abnormal completion (via SEND STOP, input error, etc) of a COPY

request. The COPY is terminated (normally) if a Double-EOF or an

End-Of-Medium is encountered on the INPUT.

ODP$

CREC...N says copy N-records from INPUT to OUTPUT. The same rules

apply as for COPY except that the OUTPUT tape is always left

positioned ahead of a Double File-Mark.

ODP$

CC.....Says continue previous COPY or CREC. It remembers how

many files or records have already been copied and continues the

count.

TPF$

SP1$

SEC$.140  Discussion of Byte-Swapping

BLP$

The command SWAB causes the following byte-swapping actions:

ODP$

o......For L002 or L003 input, swaps bytes for all input records

(appropriately) in both COPY and MOC modes.

ODP$

o......For L001 input, swaps bytes of all input records (assuming

16-bit integers) in the COPY mode.

ODP$

o......For L001 input, swaps bytes of data records only in MOC mode.

BLP$

Under some circumstances, you may need to byte-swap the data and

not the headers or vice versa. In such cases, the command SHON turns

on an additional byte-swap for the header only. SHOF turns it off

and is the default.

TPF$

SP1$

SEC$.150  Discussion of Tape Conversion

BLP$

The stock \b\lemor\\ may be used to convert certain non-standard

tapes to L003 format under the Modify-Copy (\b\MOC\\) mode

if the following conditions are met:

ODP$

o......The length of event-by-event data records to be processed

must be different from all other records which will be encountered.

ODP$

o......All data records must be written in 16-bit mode.

ODP$

o......If the high-order bit of a data word in the input stream is set

it will be masked off (lost) in the converted data stream.

ODP$

o......Each data record must contain a fixed (integer) number of

events. i.e. events may not be split across record boundries.

ODP$

o......Each event must contain a fixed number of parameters.

ODP$

o......Bytes may be swapped if requested.

ODP$

o......A specified number of data words (record header words etc)

may be skipped at the beginning of each data record.

CYB$

Execute the following commands:

lemor             ;Starts program lemor

IN   rmtx         ;Specify input  tape rmtx

ou   rmty         ;Specify output tape rmty  -  or

ouf  file.ldf     ;Specify output LDF (file.ldf)

L001 ns np        ;L001 says that input meets the requirements above

;ns = # of header words to skip before reading events

;np = # of parameters per event

swab              ;Only if bytes are to be swapped!!

reco nbyts        ;Specifies output record-length = nbyts bytes

;Default   output record-length = 32768 bytes

upon              ;Enable "User Processing" (the default)

moc nfiles        ;Says process nfiles files from input

TPF$

SP2$

SEC$.160  Discussion of Prescan (Modify-Copy)

BLP$

Pre-Scanning (as interpreted here) involves the processing of an

INPUT data stream (from Mag Tape) to produce an OUTPUT data

stream (to Tape or LDF). The processing may include

selection of certain events, selection of certain parameters,

modification of input parameters or creation of new parameters or

any combination of the preceding.

BLP$

A prescan task involves the use of \b\lemor\\ (or a

customized version thereof) to control the process combined with

user-supplied routines which aid in the selection of events and/or

parameters. Modification or creation of parameters will normally

require one or more user-supplied subroutines USERSUBS.

BLP$

In the MODIFY-COPY mode, records are read from the input tape,

events are expanded and passed (one at a time) to the USERSUBS.

BLP$

Input records which are not of the length specified by the command

\b\reci or auto-detected\\ are taken to be non-data records and

are simply copied to the output tape or LDF.

Thus, Headers are copied automatically.

If the record length and/or the Max # of Parms for the output tape

is specified differently from the input, the primary Header is

modified appropriately.

BLP$

If an End-of-Medium is encountered on an output tape and the

recording is continued on a new tape, only the primary header

(not the text blocks containing the PAC program etc) will

be reproduced on the continuation tape.

CEB$

End-of-File or End-of-Medium on Input Tape

BLP$

When an EOF or EOM is encountered on the input tape, \b\lemor\\ writes

any partially filled output buffer and two File Marks onto all

output tapes and backs up one File Mark.

To continue with the next input file, type the command MOC.

To continue with the next input tape, type RWI, mount next input

tape and type MOC.

CEB$

End-of-Medium on Output Tape

BLP$

When an EOM is encountered on the output tape, \b\lemor\\ backs up one

record, writes a File Mark, rewinds the output, instructs you to

mount a new tape, and pauses.

When you type: CONTINUE, \b\lemor\\ writes the primary header (not the text

blocks) and the last output record (the one it was writing when it

hit EOM) onto the new tape, and continues processing.

CYB$

See table on subsequent page for more information on termination of processing.

CEB$

(continued on next page)

TPF$

SP2$

SEC$.160  Discussion of Prescan (Modify-Copy) (continued)

BLP$

When running in the modify-copy (MOC) mode,

one or more of the following operations may be initiated which

can result in the modification of the data stream. Operations

are listed in the order that they will be carried out if requested.

The column headed CMD gives the run-time \b\lemor\\ command which requests

the associated operation.

CY1$

CMD    OPERATION-----------------------------------------------------------

ODP$

SWAB...Swap bytes in data buffers (also HHIRF headers) (see SEC#140).

ODP$

L001...Convert data buffers from L001 to L003 format (see SEC#???).

ODP$

UPON...Process buffers via user-supplied routine USERMOC (see SEC#220).

ODP$

INIT...Resets both the EVENT- and OUTPUT-buffers. Do this if you

have been doing some tests but are now ready to prescan for

real or any time you wish to make a clean start.

ODP$

MOC....N,M Starts the MODIFY-COPY process, where: N is the number

of files to process and M is the number of records to process.

The processing terminates on either N or M - the first to be

satisfied. The default values of N and M are 1 and 100,000,000

respectvely.

ODP$

MOCE...N,M Differs from the MOC-command described above only in that

upon completion of the request, both INPUT and OUTPUT tapes are

unloaded and the program is terminated.

FIP$

The following table summarizes the different ways in which processing

may be terminated and the state of the OUTPUT-tape, OUTPUT-buffer

and EVENT-buffer for each:

CY1$

====================================================================

TERMINATION BY---   RESULTS IN -------------------------------------

Requested # Files   Flush OUT-BUF, 2-EOF, 1-BKFIL, Scrub-EV

EOM on INPUT        Flush OUT-BUF, 2-EOF, 1-BKFIL, Scrub-EV

Requested # Recs    Flush OUT-BUF, 2-EOF, 2-BKFIL,  Save-EV

Ctrl/C                             2-EOF, 2-BKFIL,  Save-EV

Input Error         Flush OUT-BUF, 2-EOF, 2-BKFIL, Scrub-EV

MOCE                Flush OUT-BUF, 2-EOF, Unload Tapes, EXIT

====================================================================

FIP$

Where, Scrub-EV indicates that any partial-event which has not

been fully processed will be deleted from the EVENT-buffer and

Save-EV means that any partial-event will be retained for

subsequent MOC requests.

FIP$

NOTE: Processing is always terminated with two File-Marks being

written on all OUTPUT tapes.

CYB$

====================================================================

TPF$

SEC$.170  Discussion of Banana Gating (Free-Form-Gating)

BLP$

\b\2-D free-form gate\\ testing must be carried out by user-supplied

code executed by routine \b\USERMOC\\.

There are some intrinsic aids for 2-D gate testing

and ban-file processing. This is how it goes:

CYB$

Reading in ban-files-------------------------------------------------------

BLP$

One or more ban-files are opened and read in via the commands:

CYB$

ban file1.ban

ban file2.ban

.

BLP$

All entries (bananas) in each file are read in and stored in memory

in the order in which they occur in the file. Subsequent references to

individual bananas (stored in memory) may be via the stacking ordinal

(sequence number as read in) or via the ban-ID from the ban-file.

If more than one file is read in, one must make sure that all IDs are

unique or one must reference bananas by sequence number rather than

ID-number.

Non-unique IDs will generate a warning at read-in time but it is not

a fatal error.

Also, the ID numbers of bananas to be referenced by ID number must be

limited to the range of 1 to 8000. Any out-of-range IDs will generate

a warning at read-in time but, again, the error is not fatal.

CYB$

Zeroing the in-memory bananas ---------------------------------------------

BLP$

You may use the command \b\banz\\ to clear all in-memory bananas.

Subsequently, a new set may be read in as described above.

CYB$

Functions which test 2-D Gates --------------------------------------------

BLP$

After the appropriate ban-files have been read in, functions \b\BANTESTI\\

or \b\BANTESTN\\ may be used to test

X and Y parameters against individual bananas. (see SEC#270).

BLP$

The following list summarizes the features and limitations of this type

of free-form gating support.

ODP$

o......Multiple ban-files may be read in. All bananas in each file are

stored in memory.

ODP$

o......A ban-ID directory is built as files are read in. If more than one

banana has the same ID, only the last one read will be entered in the

directory. A warning will be displayed at read-in time but the error is not

fatal (you can always reference via sequence # via routine \b\BANTESTN\\).

ODP$

o......Up to 3000 bananas may be stored by the standard support routines.

ODP$

o......Up to 8000 banana IDs can be accomodated by the standard routines.

ID numbers must be in he range of 1 through 8000 in order to be entered

into the ID-directory. If any IDs are out of range, a warning will be

given at read-in time but the error is not fatal (you can always reference

via sequence# via routine \b\BANTESTN\\).

ODP$

o......Up to 1024000 banana channels (2000 512-channel bananas) may be

stored in memory.

TPF$

SP2$

SEC$.180  How to Create a Customized Prescan Program

CY1$

\bu\For Alphas Workstations\\

BLP$

Use the make file \b\/usr/hhirf/lemor.make\\

(listed below) as a template for creating

your own make file for generating a customized \b\prescan\\ program.

CY1$

FFLAGS= -fpe2

#

OBJS= /usr/hhirf/lemor.o\

/usr/users/milner/Dlemor/dummysubs.o

#

LIBS= /usr/hhirf/lemorlib.a\

/usr/hhirf/miliba.a\

/usr/hhirf/orphlib.a

#

LDF = -laio -lpthreads

#

lemor: $(OBJS) $(LIBS)

#

f77 -O4 $(FFLAGS) $(OBJS) $(LIBS) $(LDF) -o lemor

BLP$

Assume that your username is \b\userdoe\\ and your customizing routines

are in your directory \b\Dlemosubs\\ and there are two files to be

included, \b\subsa\\ and \b\subsb\\ and you wish to produce a program

\b\mylemo\\. Copy the template file into your directory and

create a make file \b\mylemo.make\\ by changing the template

file as/where indicated by the \b\bold\\ face type. The generic result

is shown below.

CY1$

FFLAGS= -fpe2

#

OBJS= /usr/hhirf/lemor.o\

/usr/users/\b\userdoe/Dlemosubs/subsa.o\\\

/usr/users/\b\userdoe/Dlemosubs/subsb.o\\

#

LIBS= /usr/hhirf/lemorlib.a\

/usr/hhirf/miliba.a\

/usr/hhirf/orphlib.a

#

LDF = -laio -lpthreads

#

\b\mylemo\\: $(OBJS) $(LIBS)

f77 -O4 $(FFLAGS) $(OBJS) $(LIBS) $(LDF) -o \b\mylemo\\

CY1$

===========================================================================

The procedures is the same for \b\DECstations\\ except that the \b\LDF\\

entry is not defined.

===========================================================================

TPF$

SP1$

SEC$.190  Lemor Routines Which aid Customization

CY1$

Routine   Function or use                                               See

UNPACKL   Unpacks L001, L002 or L003 buffers into single events     SEC#240

Form is ID-list, Data-list and expanded data-array

UNPACKAA  Unpacks L003 buffers into single events                   SEC#240

Form is ID-list and associated data-list

UNPACKBB  Unpacks L003 buffers into single events                   SEC#240

Form is ID-list and associated expanded data-array

GATTESTI  Tests X data against a 1-D gate identified by ID-number   SEC#260

BANTESTI  Tests X,Y data against a banana identified by ID-number   SEC#270

BANTESTN  Tests X,Y data against a banana identified by ordinal     SEC#270

EVLISOUT  Sends single events to the output data stream             SEC#250

EVEXPOUT  Sends single events to the output data stream             SEC#250

SP1$

SEC$.200  User-Supplied Routines Which aid Customization

CY1$

Routine   Function or use                                               See

USERCMP   Receives \b\UCOM\\ commands for the support of other           SEC#210

user-supplied routines

USERMOC   The user-supplied routine called by \b\lemor\\ when operating  SEC#220

in the modify-copy mode.                                  SEC#230

SP1$

SEC$.210  User Routine: USERCMP - Users Command Processor

BLP$

\b\lemor\\ supports customization by means of a user supplied command

processor as follows:

CY1$

The command:  \b\ucom text\\

Results in the following actions:

BLP$

The \b\ucom\\ is removed from the command line and the \b\text\\ field

is used as an argument in a call to routine \b\USERCMP\\ as follows:

CY1$

CALL USERCMP(text)

BLP$

The user supplied routine \b\USERCMP\\ may then interpret the 80-byte

character string \b\text\\ as desired and pass the results to other

routines through user defined \b\COMMON\\ or calls to other user supplied

routines.

TPF$

SP1$

SEC$.220  User Routine: USERMOC - User Processing of Raw Data

BLP$

\b\lemor\\ provides users with the means

(via a user-supplied routine \b\USERMOC\\) to do the following:

ODP$

o......Receive raw input data buffers and modify if desired.

ODP$

o......Call routine \b\UNPACKL\\ (see SEC#240) to return single events.

ODP$

o......Process (test, modify, etc) events and call an event

output routine (\b\EVLISOUT or EVEXPOUT)\\ (see SEC#250) to output

single events to tape or LDF.

BLP$

At run time, calls to \b\USERMOC\\ are enabled/disabled as follows:

CY1$

UPOF             ;Disable calling of USERMOC

UPON             ;Enable  calling of USERMOC (NPRAW=0) (default)

UPON NPRAW       ;Enable  calling of USERMOC with NPRAW defined

;NPRAW = Max # raw parms (for USERMOC only,  if needed)

BLP$

The default \b\USERMOC\\ listed below does no event selection, however it

may be used to convert one of the data formats (L001, L002, L003)

to another.

CY1$

===========================================================================

C$PROG USERMOC   - Default USERMOC (can convert data formats)

C

SUBROUTINE USERMOC(IBUF,NHW)

IMPLICIT NONE

C     ------------------------------------------------------------------

INTEGER*2    IBUF(*)

INTEGER*4    IDLST(2000),DALST(2000),EXPEV(2000)

INTEGER*4    NHW,MXID,NPAR,IERR

INTEGER*4    NWDS,IX,IY

CHARACTER*4  DONE

DATA         MXID/2000/

C     ------------------------------------------------------------------

C

100 CALL UNPACKL(

&           IBUF,   !I*2 - raw data buffer

&           NHW,    !I*4 - # of I*2 words in IBUF

&           MXID,   !I*4 - max-ID (dimension IDLST, DALST, EXPEV)

&           IDLST,  !I*4 - ID-list   for returned event

&           DALST,  !I*4 - Data-list for returned event

&           EXPEV,  !I*4 - expanded-event array

&           NPAR,   !I*4 - # of parameters in this event

&           DONE,   !C*4 - YES/NO - requests new IBUF

&           IERR)   !I*4 - 0 means OK, nonzero means error

C

C

IF(DONE.EQ.'YES ') GO TO 200

IF(IERR.NE.0)      GO TO 100

CALL EVLISOUT(IDLST,DALST,NPAR)

GO TO 100

200 RETURN

END

===========================================================================

TPF$

SP2$

SEC$.230  User Routine USERMOC - Example with Banana Gating

CY1$

The following \b\USERMOC\\:

(1)....Receives data buffers in default (L003) or specified data format.

(2)....Calls \b\UNPACKL\\ to unpack and return single events,

(3)....Calls \b\BANTESTI\\ to test parameters 5 & 7 against banana 1,

(4)....Calls \b\EVLISOUT\\ to output events into a tape or LDF.

===========================================================================

CY1$

C$PROG USERMOC   - Example USERMOC which tests one banana-gate

C

SUBROUTINE USERMOC(IBUF,NHW)

C

IMPLICIT NONE

C

C     ---------------------------------------------------------------------

INTEGER*2    IBUF(*)

INTEGER*4    IDlST(2000),DALST(2000),EXPEV(2000)

INTEGER*4    NHW,MXID,NPAR,IERR

INTEGER*4    NWDS,IG,IX,IY

CHARACTER*4  DONE

LOGICAL      BANTESTI

DATA         MXID/2000/

C     ---------------------------------------------------------------------

C

IG=1               !Banana ID number to test

IX=5               !X-parameter to test

IY=7               !Y-parameter to tset

C

100 CALL UNPACKL(

&           IBUF,   !I*2 - raw data buffer

&           NHW,    !I*4 - # of I*2 words in IBUF

&           MXID,   !I*4 - max-ID (dimension of IDLST, DALST, EXPEV)

&           IDLST,  !I*4 - ID-list   for returned event

&           DALST,  !I*4 - Data-list for returned event

&           EXPEV,  !I*4 - expanded-event array

&           NPAR,   !I*4 - # of parameters in this event

&           DONE,   !C*4 - YES/NO - requests new IBUF

&           IERR)   !I*4 - 0 means OK, nonzero means error

C

C

IF(DONE.EQ.'YES ')     GO TO 200     !Test for end of buffer

IF(IERR.NE.0)          GO TO 100     !Test for error

IF(BANTESTI(IG,                      !Test EXPEV(IX),EXPEV(IY)

&            EXPEV(IX),               !against banana-IG

&            EXPEV(IY)) GO TO 110     !Test for BAN-gate satisfied

GO TO 100                            !If no,  skip it

C

110 CALL EVLISOUT(IDLST,DALST,NPAR)      !If yes, send to output

GO TO 100                            !Go back for next enent

200 RETURN                               !Return for next buffer

END

TPF$

SP1$

SEC$.240  Lemor Routines which Unpack Events

CY1$

\bu\UNPACKL - Unpacks L001, L002 & L003 Buffers\\

BLP$

The data format is interpreted as (L001, L002 or L003) depending

on that specified by the user at run time. The default format is

\b\L003\\ since that is the one we are currently using in data

acquisition.

CY1$

100 CALL UNPACKL(

&           IBUF,   !I*2 - raw data buffer

&           NHW,    !I*4 - # of I*2 words in IBUF

&           MXID,   !I*4 - max-ID (dimension of IDLST, DALST, EXPEV)

&           IDLST,  !I*4 - ID-list   for returned event

&           DALST,  !I*4 - Data-list for returned event

&           EXPEV,  !I*4 - expanded-event array

&           NPAR,   !I*4 - # of parameters in this event

&           DONE,   !C*4 - YES/NO - requests new IBUF

&           IERR)   !I*4 - 0 means OK, nonzero means error

BLP$

\bu\Important Note: \\In interest of speed,

\b\UNPACKL\\ resets (to -1) only those elements of \b\EXPEV\\ which

were set by the previous call (all elements are initially set to -1).

If the user sets other elements

in this array, it is her/his/its responsibility to reset these elements

prior to subsequent calls to \b\UNPACKL\\.

BLP$

\bu\Important Note: \\In the interest of speed,

\b\UNPACKL uses IDLST\\ to reset elements

in \b\EXPEV\\ which were set in the previous unpack operation.

Therefore, the user should \b\NOT\\ modify \b\IDLST\\.

BLP$

\bu\Important note:\\ Routine \b\UNPACKBB\\ resets elements in the

expanded array \b\EVBUF\\ to 0 (not -1).

TPF$

SP1$

SEC$.240  Lemor Routines which Unpack Events (continued)

CY1$

\bu\UNPACKAA - Unpacks L003 Buffers to Single Events\\

BLP$

The L003 format used in the ORPHAS data acquisition system consists of pairs

of 16-bit words.  The first word is the parameter ID and the second word is

the parameter data.  The end-of-event is marked by a pair for which both

the ID and the data are 'FFFF'X.  An event data buffer always has an

integral number of events.

BLP$

UNPACKAA unpacks single events from a buffer.  Each call returns a single

event in the EVBUF array.  When all events in the buffer have been processed,

the alternate FORTRAN return is taken.

BLP$

UNPACKAA returns data in a two dimensional array.  The first element is the

parameter ID and second is the parameter data.

CY1$

CALL UNPACKAA(IBUF,NHW,EVBUF,NPARAM,EVSIZE,IERR,IEND)

\b\call:\\   INTEGER*2  IBUF       - raw event data buffer

INTEGER*4  NHW        - number of INT*2 words in IBUF

INTEGER*4  EVSIZE     - Second dimension of array EVDAT

\b\return:\\ INTEGER*4  EVBUF(2,*) - Event data. EVBUF(1,*) is the parameter ID

INTEGER*4  NPARAM     - Number of parameters in this event.

INTEGER*4  IERR       - 0 says OK. Nonzero says too many parameters

INTEGER*4  IEND       - 0 says more data - nonzero says end-of-buffer

\b\NOTE: IEND has been implemented because gnu fortran doesn't support the

alternate RETURN used in previous versions\\

SP2$

CY1$

\bu\UNPACKBB - Unpacks L003 Buffers to Single Events\\

BLP$

UNPACKBB performs the same function as UNPACKAA.  The parameter ID is

used as an index to the EVBUF array to get the parameter data.  The EVBUF

is zero for parameters which were not present in the event.

CY1$

CALL UNPACKBB(IBUF,NHW,IDBUF,EVBUF,MAXID,NPARAM,IERR,IEND)

\b\call:\\   INTEGER*2  IBUF      - raw event data buffer

INTEGER*4  NHW       - number of INT*2 words in IBUF

INTEGER*4  MAXID     - Max ID. IDBUF and EVBUF must be dimensioned

at least as great as MAXID

\b\return:\\ INTEGER*4  IDBUF()   - List of IDs in this event

INTEGER*4  EVBUF(id) - Event data for parameter id

INTEGER*4  NPARAM    - Number of parameters in this event.

INTEGER*4  IERR      - 0 says OK. Nonzero says too many parameters.

INTEGER*4  IEND       - 0 says more data - nonzero says end-of-buffer

- Also nonzero if L003 format error occurs

\b\NOTE: IEND has been implemented because gnu fortran doesn't support the

alternate RETURN used in previous versions\\

TPF$

SP1$

SEC$.250  Lemor Routines which Output Events

BLP$

Two event output routines are available in "user" mode, namely;

\b\EVLISOUT & EVEXPOUT\\. Both routines can generate output streams

in L001, L002 or L003 format (whichever is specified or defaulted).

The calling sequences are:

CYB$

CALL EVLISOUT(IDLST,  !I*4 array  - ID-list (from UNPACKL)

&              DALST,  !I*4 array  - data associated with ID-list

&              NPAR)   !I*4 scaler - number of IDs

CALL EVEXPOUT(EXPEV,  !I*4 array  - expanded event (from UNPACKL)

&              LO,     !I*4 array  - min parameter# to output

&              HI)     !I*4 scaler - max parameter# to output

BLP$

The default \b\lemor\\ contains the following \b\USERMOC\\ routine

and uses routine \b\EVLISOUT\\ to output events.

CY1$

C$PROG USERMOC   - Default user modify-copy routine (USERMOC)

C

SUBROUTINE USERMOC(IBUF,NHW)

IMPLICIT NONE

C     ------------------------------------------------------------------

INTEGER*2    IBUF(*)

INTEGER*4    IDLST(2000),DALST(2000),EXPEV(2000)

INTEGER*4    NHW,MXID,NPAR,IERR

INTEGER*4    NWDS,IX,IY

CHARACTER*4  DONE

DATA         MXID/2000/

C     ------------------------------------------------------------------

C

100 CALL UNPACKL(

&           IBUF,   !I*2 - raw data buffer

&           NHW,    !I*4 - # of I*2 words in IBUF

&           MXID,   !I*4 - max-ID (dimension IDLST, DALST, EXPEV)

&           IDLST,  !I*4 - ID-list   for returned event

&           DALST,  !I*4 - Data-list for returned event

&           EXPEV,  !I*4 - expanded-event array

&           NPAR,   !I*4 - # of parameters in this event

&           DONE,   !C*4 - YES/NO - requests new IBUF

&           IERR)   !I*4 - 0 means OK, nonzero means error

C

C

IF(DONE.EQ.'YES ') GO TO 200

IF(IERR.NE.0) GO TO 100

CALL EVLISOUT(IDLST,DALST,NPAR)

GO TO 100

200 RETURN

END

TPF$

SEC$.260  Lemor Routine for Testing 1-D Gates

BLP$

Simple 1-D gating is supported in user-mode as follows:

ODP$

o......One or more 1-D gates are specified via the syntax:

CYB$

GATE ID LO HI

FIP$

Where, \b\ID\\ is the gate ID# and \b\LO & HI\\ are the gate

limits. The allowed range for \b\ID\\ is 1 to 1000. All

quantities are \b\INTEGER*4\\.

ODP$

o......\b\LOGICAL FUNCTION GATTESTI(ID,IP)\\ is used to test

the parameter \b\IP\\ against gate \b\ID\\. (Note: gates are inclusive)

CYB$

GATTESTI is .TRUE.  if gate is satisfied

GATTESTI is .FALSE. otherwise

ODP$

o......Gates may be redefined - no warning is given.

ODP$

o......There is no check on gate limits.

ODP$

o......Gates must be on the same basis (scale) as the parameter being

tested.

ODP$

o......Gates may be specified interactively, or read from a file (see

SEC#020 and SEC#040 for relavant commands).

SEC$.270  Lemor Routine for Testing 2-D Gated (bananas)

BLP$

After the appropriate ban-files have been read in,

logocal function \b\BANTESTI\\ may be used by routine \b\USERMOC\\ to test

X and Y parameters against individual bananas.

CYB$

LOGICAL FUNCTION BANTESTI(IG,IX,IY)

C

INTEGER*4 IG,IX,IY

BLP$

\b\BANTESTI\\ references

bananas via ID-numbers IG. If the the IX,IY point is contained within the

specified banana, the value of the function is .TRUE., otherwise it .FALSE.

In NG or IG do not exist, .FALSE. is returned. There are no error messages.

SP1$

BLP$

After the appropriate ban-files have been read in,

logical function \b\BANTESTN\\ may be used by routine \b\USERMOC\\ to test

X and Y parameters against individual bananas.

CYB$

LOGICAL FUNCTION BANTESTN(NG,IX,IY)

C

INTEGER*4 NG,IX,IY

BLP$

\b\BANTESTN\\ references bananas via sequence numbers NG.

If the the IX,IY point is contained within the

specified banana, the value of the function is .TRUE., otherwise it .FALSE.

In NG or IG do not exist, .FALSE. is returned. There are no error messages.

TPF$

SP2$

SEC$.300  Logical Units and COMMON Blocks

CYB$

\b\lemor\\ may use some or all of the following logical unit assignments:

===========================================================================

LU#     USE

5     VDT INPUT

6     VDT OUTPUT

7     LIST FILE/DEV

8     HELP-FILE, MIL-FILE, TEX-FILE, ETC. CLOSED AFTER USE

10     CMD-FILE

11     INPUT    TAPE/FILE

12     OUTPUT-1 TAPE/FILE

13     OUTPUT-2 TAPE

14     OUTPUT-3 TAPE

15     INPUT    LDF-FILE

16     OUTPUT   LDF-FILE

17     OUTPUT   RECLOG-FILE

===========================================================================

\b\lemor\\ uses the following labeled COMMON blocks:

===========================================================================

COMMON/FFGA/

COMMON/FFGB/

COMMON/FFGC/

COMMON/FFGD/

COMMON/FFGE/

COMMON/LLL/

COMMON/LM01/ thru COMMON/LM50/

COMMON/ML01/

COMMON/ML02/

===========================================================================

BLP$

The user should not attempt to use any of these logical units or COMMON

block labels.

CHP$U310  Processing UDFs - Linux Version Only!

TPF$

SP1$

SEC$.310  Processing of User Defined Files - UDFs - Linux Version Only!

BLP$

Support is provided for the processing of non-scandard event data files.

Such files might be generated via simulations or from some other source.

I will call such files User Defined Files or just \b\UDFs\\.

CY1$

The commands:

CY1$

\b\UDF filename\\      ;Opens the \b\UDF\\ for sequential access reading

;and sets flags for \b\UDF\\ processing

\b\UDF filename RECL\\ ;Opens the \b\UDF\\ for direct access reading with

;record length = \b\RECL\\

;and sets flags for \b\UDF\\ processing

BLP$

A number of special routines are brought into play. Depending on specific

requirements, one or more of these routines must be customized by the user.

There are three routines that may need to be customized. These are:

ODP$

\b\UDFOPEN\\ which opens the \b\UDF\\ for the type of access required for

reading (sequential, direct, etc). The default version opens the file as

a formatted (ASCII) file to be read sequentially.

ODP$

\b\UDFHAN\\ processes the positioning commands \b\REW, FR, & BR\\

which provides for REWIND, BACK-RECORD and FORWARD-RECORD operations.

ODP$

\b\UDFEVENT\\ reads the \b\UDF\\ and returns one event per call in the form

of INTEGER*4 ID and DATA arrays. These event arrays are then used

to build standard L003 buffers. This routine will most

certainly need to be customized, however, it is probably the only one

which does.

CY1$

The archive for default routines are in \b\/usr/hhirf/lemorlib.a\\

The source  for default routines are in \b\/tera/milner/DDgnu/Dlemorlib/\\

BLP$

In order to use the \b\UDF\\ support, the user must understand the structure

of the \b\UDF\\ and be able to:

ODP$

o......Modify \b\UDFOPEN\\ to be compatible with the reading requirements,

ODP$

o......Modify \b\UDFEVENT\\ so as to read the \b\UDF\\ and return one

event per call.

BLP$

The template (default) \b\UDF\\ support routines are internally documented and

listed on subsequent pages.

CHP$U310  Routine UDFOPEN - Linux Version Only!

TPF$

SP1$

CY1$

C$PROG UDFOPEN   - Opens Input UDF-Files for LEMOR

C

C     ******************************************************************

C     BY W.T. MILNER AT HRIBF - LAST MODIFIED 03/31/2005

C     ******************************************************************

C

SUBROUTINE UDFOPEN

C

IMPLICIT NONE

C

C     ------------------------------------------------------------------

COMMON/LLL/ MSSG(28),NAMPROG(2),LOGUT,LOGUP,LISFLG,MSGF

INTEGER*4   MSSG,NAMPROG,LOGUT,LOGUP

CHARACTER*4 LISFLG,MSGF

CHARACTER*112 CMSSG

EQUIVALENCE (CMSSG,MSSG)

C     ------------------------------------------------------------------

COMMON/ML02/ IWDRAW(20)

INTEGER*4    IWDRAW

C     ------------------------------------------------------------------

COMMON/LM20/ LUINF,LUOUF,INFOP,OUFOP

INTEGER*4    LUINF,LUOUF

CHARACTER*4              INFOP,OUFOP

C     ------------------------------------------------------------------

COMMON/LM23/ INDIR(8192),OUDIR(8192),INTYP,OUTYP,INRECI,OURECI

INTEGER*4    INDIR,      OUDIR,                  INRECI,OURECI

CHARACTER*4                          INTYP,OUTYP

C     ------------------------------------------------------------------

COMMON/LM33/ UDFNAM(20),UDFRECL,UDFNPAR,UDFRECI

INTEGER*4    UDFNAM,    UDFRECL,UDFNPAR,UDFRECI

C     ------------------------------------------------------------------

COMMON/LM34/ NCEOF,LAUTO

INTEGER*4    NCEOF

CHARACTER*4        LAUTO

C     ------------------------------------------------------------------

CHARACTER*80 CNAMF

EQUIVALENCE (CNAMF,UDFNAM)

INTEGER*4    RECLVALU,IOS,IERR

C     ------------------------------------------------------------------

SAVE

C     ------------------------------------------------------------------

C     Process: UDF filename <RECL>     ;i.e. get UDFNAM & UDFRECL

C     ------------------------------------------------------------------

C

CALL UDFNAME(IWDRAW,UDFNAM,UDFRECL,IERR)

C

IF(IERR.NE.0) GO TO 1000

CEB$

(continued on next page)

TPF$

SP1$

CY1$

C

C     ------------------------------------------------------------------

C     Open User-Defined-File for input

C     ------------------------------------------------------------------

C

CLOSE(UNIT=LUINF)               !Close input file if open

INTYP='    '                    !Reset input type

NCEOF=0                         !Reset contiguous EOF counter

C

IF(UDFRECL.EQ.0) THEN           !Test for RECL specified

OPEN(UNIT      = LUINF,         !Otherwise,

&     FILE      = CNAMF,         !Open UDF for sequential access

&     STATUS    = 'OLD',

&     IOSTAT    = IOS)

GO TO 100                       !Go test status

ENDIF

C

OPEN(UNIT      = LUINF,         !Otherwise,

&     FILE      = CNAMF,         !Open UDF for DIRECT access

&     ACCESS    = 'DIRECT',

&     RECL      = RECLVALU(UDFRECL),!with RECORD LENGTH = UDFRECL

&     STATUS    = 'OLD',

&     IOSTAT    = IOS)

C

100 IF(IOS.NE.0) THEN               !Test OPEN status for good

CALL IOFERR(IOS)

GO TO 1000

ENDIF

C

INTYP='UDF '                    !Set input type to UDF

C

UDFRECI=1                       !Next rec# to be read

C

UDFNPAR=0

C

RETURN

C

C     ------------------------------------------------------------------

C     Send error messages

C     ------------------------------------------------------------------

C

1000 WRITE(CMSSG,1005)

1005 FORMAT('UDFOPEN ERROR')

CALL MESSLOG(LOGUT,LOGUP)

RETURN

END

CHP$U310  Routine UDFHAN - Linux Version Only!

TPF$

SP1$

CY1$

C$PROG UDFHAN    - Manages UDF-file position pointers for LEMOR

C

C     ******************************************************************

C     BY W.T. MILNER AT HRIBF - LAST MODIFIED 11/22/2004

C     ******************************************************************

C

SUBROUTINE UDFHAN(IERR)

C

IMPLICIT NONE

C

C     ------------------------------------------------------------------

COMMON/LLL/ MSSG(28),NAMPROG(2),LOGUT,LOGUP,LISFLG,MSGF

INTEGER*4   MSSG,NAMPROG,LOGUT,LOGUP

CHARACTER*4 LISFLG,MSGF

CHARACTER*112 CMSSG

EQUIVALENCE (CMSSG,MSSG)

C     ------------------------------------------------------------------

COMMON/ML01/ IWD(20),LWD(2,40),ITYP(40),NF,NTER

INTEGER*4    IWD,    LWD,      ITYP,    NF,NTER

C     ------------------------------------------------------------------

COMMON/LM20/ LUINF,LUOUF,INFOP,OUFOP

INTEGER*4    LUINF,LUOUF

CHARACTER*4              INFOP,OUFOP

C     ------------------------------------------------------------------

COMMON/LM23/ INDIR(8192),OUDIR(8192),INTYP,OUTYP,INRECI,OURECI

INTEGER*4    INDIR,      OUDIR,                  INRECI,OURECI

CHARACTER*4                          INTYP,OUTYP

C     ------------------------------------------------------------------

COMMON/LM33/ UDFNAM(20),UDFRECL,UDFNPAR,UDFRECI

INTEGER*4    UDFNAM,    UDFRECL,UDFNPAR,UDFRECI

C     ------------------------------------------------------------------

COMMON/LM34/ NCEOF,LAUTO

INTEGER*4    NCEOF

CHARACTER*4        LAUTO

C     ------------------------------------------------------------------

INTEGER*4    IERR,KIND,NUM,NV,N

REAL*4       XV

CHARACTER*4  KMD

EQUIVALENCE (KMD,LWD(1,1))

C     ------------------------------------------------------------------

SAVE

C     ------------------------------------------------------------------

C     LUINF    =  Logical unit# for UDF-file

C

C     INTYP    = '    ', TAPE, FILE = input type

C

C     UDFRECI  =  Current rec# pointer for input file if DIRECT access

C                 Otherwise just keeps track of position but not used

C     ------------------------------------------------------------------

C

IERR=0

C

CALL MILV(LWD(1,2),NV,XV,KIND,IERR)

IF(IERR.NE.0) GO TO 1000

NUM=NV

IF(NUM.LE.0) NUM=1

C

IF(INTYP.NE.'UDF ') GO TO 1000

CEB$

(continued on next page)

TPF$

SP1$

CY1$

C

IF(KMD.EQ.'RWI ') GO TO 100

IF(KMD.EQ.'BRI ') GO TO 150

IF(KMD.EQ.'FRI ') GO TO 200

C

GO TO 1000

C

C     ------------------------------------------------------------------

C     Process REW  -  Rewind input file

C     ------------------------------------------------------------------

C

100 REWIND LUINF               !For SEQUENTIAL access only

NCEOF=0                    !Reset contiguous EOF counter

UDFRECI=1                  !Next REC# to be read for DIRECT access

UDFNPAR=0

RETURN

C

C     ------------------------------------------------------------------

C     Process BR   -  Backup record/s on input file

C     ------------------------------------------------------------------

C

150 DO 160 N=1,NUM

BACKSPACE LUINF            !For SEQUENTIAL access only

UDFRECI=UDFRECI-1

IF(UDFRECI.LT.1) THEN

UDFRECI=1

UDFNPAR=0

RETURN

ENDIF

160 CONTINUE

RETURN

C

C     ------------------------------------------------------------------

C     Process FR   -  Forward record/s on input file

C     ------------------------------------------------------------------

C

200 DO 210 N=1,NUM

READ(LUINF,205)KIND         !For SEQUENTIAL access only

205 FORMAT(A4)

UDFRECI=UDFRECI+1

UDFNPAR=0

210 CONTINUE

RETURN

C

C     ------------------------------------------------------------------

C     Send error messages

C     ------------------------------------------------------------------

C

1000 IERR=1

RETURN

END

CHP$U310  Routine UDFEVENT - Linux Version Only!

TPF$

SP1$

CY1$

C$PROG UDFEVENT  - Reads & returns one event from example UDF-file

C

C     ******************************************************************

C     BY W.T. MILNER AT HRIBF - LAST MODIFIED 11/28/2004

C     ******************************************************************

C

SUBROUTINE UDFEVENT(MXID,PID,DAT,NP,IERR)

C

IMPLICIT NONE

C

C     ------------------------------------------------------------------

COMMON/LM23/ INDIR(8192),OUDIR(8192),INTYP,OUTYP,INRECI,OURECI

INTEGER*4    INDIR,      OUDIR,                  INRECI,OURECI

CHARACTER*4                          INTYP,OUTYP

C     ------------------------------------------------------------------

COMMON/LM20/ LUINF,LUOUF,INFOP,OUFOP

INTEGER*4    LUINF,LUOUF

CHARACTER*4              INFOP,OUFOP

C     ------------------------------------------------------------------

COMMON/LM33/ UDFNAM(20),UDFRECL,UDFNPAR,UDFRECI

INTEGER*4    UDFNAM,    UDFRECL,UDFNPAR,UDFRECI

C     ------------------------------------------------------------------

INTEGER*4    MXID,PID(*),DAT(*),BUF(2,8),NP,LU,IERR,I

C

INTEGER*4    EOFTST,IOS

C     ------------------------------------------------------------------

SAVE

C     ------------------------------------------------------------------

C

NP=0

IERR=0

C

100 UDFRECI=UDFRECI+1

C

IF(UDFRECL.EQ.0) THEN

READ(LUINF,110,ERR=200,END=300)BUF      !Read ASCII input line

110 FORMAT(16I5)

ENDIF

C

IF(UDFRECL.GT.0) THEN

READ(LUINF,REC=UDFRECI,IOSTAT=IOS)BUF   !Read binary input line

IF(IOS.NE.0) GO TO 400

ENDIF

C

C

IF(BUF(1,1).EQ.0.AND.BUF(2,1).EQ.0) RETURN  !If 1st 2 wds 0,

C                                                 !then its end-event

C

140 DO 150 I=1,8                            !Loop on buffer values

IF(BUF(1,I).EQ.0) GO TO 150             !If parm-ID 0, skip it

NP=NP+1                                 !Inc cntr

PID(NP)=BUF(1,I)                        !Save parm-ID

DAT(NP)=BUF(2,I)                        !Save data

150 CONTINUE

GO TO 100                               !Go read next record

CEB$

(continued on next page)

TPF$

SP1$

CY1$

C

200 WRITE(6,205)                            !Error return

205 FORMAT('Error reading UDF file')

IERR=5

RETURN

C

300 WRITE(6,305)                            !EOF return

305 FORMAT('EOF reading UDF file')

IERR=999

RETURN

C

400 IF(EOFTST(IOS).NE.0) THEN

WRITE(6,305)

IERR=999

RETURN

ENDIF

C

WRITE(6,405)IOS

405 FORMAT('Error reading UDF, IOS =',I5)

IERR=5

RETURN

END

QIT$