RPD$075066

CHP$U930 - FEX (unix version) - WTM

TPF$

SP2$

CEB$

File Examination Utility

BLP$

FEX was written to make it easier for me to examine output

from program PAC (written onto POB-files). Essentially, it

allows you to display (or log) any range of 16- or 32-bit

words on the file in various formats. Arbitrary ASCII lines

may also be displayed or logged. Assuming that the path to the \b\fex\\

executable has been apprpriately defined:

CYB$

Type:  fex        ;To get started

BLP$

The following commands are supported:

CYB$

Commands for general setup ................................................

CY1$

RECL N    - Record length in bytes for subsequent OPEN - \b\See NOTE-1\\

(not normally needed - default = 512 bytes)

In  file  - Specify input file

LON       - Turn command logging to fex.log ON

LOF       - Turn command logging to fex.log OFF (dflt)

CMD file  - Process commands from file.cmd

WO        - Wait for typed [RETURN] to continue

HOFF N    - Specify 16-bit word offset on input file (default = 0)

FOFF N    - Specify 32-bit word offset on input file (default = 0)

- (Display & Log ranges are relative to offset)

- (does not apply to DL and PL commands)

END       - Ends program

CYB$

Byte swapping commands .....................................................

CY1$

CVIF SWAF - Says do full-word byte-swap of input file (in place)

CVIF SWAB - Says do half-word byte-swap of input file (in place)

NOTE:     - The file length should be an integer multiple of RECL

for this procedure to work to completion.

CYB$

Masking commands ...........................................................

CY1$

MSKF MSK  - Says do a full-word masking of input file (in place)

- MSK is entered in hexadecimal

NOTE:     - The file length should be an integer multiple of RECL

for this procedure to work to completion.

TPF$

SP2$

CYB$

Symbols and loops ..........................................................

CY1$

Commands related to LOOP execution and SYMBOL definition

SYM = EXPRESSION - Define symbol (SYM) up to 100 symbols supported

- symbols M, N, O, P, Q, R, S are reserved

- expression syntax is same as in CHIL

- no imbedded blanks are allowed in expressions

- symbols may contain up to 4 characters (5-8 ignored)

DSYM             - Displays list of currently defined sumbols & values

LOOP N           - Starts LOOP (executed N-times) N=SYM or CONST

CMD  ....        - Nesting supported

CMD  ....        - # lines between 1st LOOP & matching ENDL = 100

ENDL             - Defines end-of-loop

- KILL (entered before ENDL) kills LOOP

- Ctrl/C - aborts loop-in-progress

- opening of CMD-file within a LOOP not allowed

CYB$

Commands for screen display ................................................

CY1$

DZ  IA,IB<,IS> - Display 16-bit word IA-IB (HEX   fmt) IS per line

DI  IA,IB<,IS> - Display 16-bit word IA-IB (INT*2 fmt) IS per line

DZF IA,IB<,IS> - Display 32-bit word IA-IB (HEX   fmt) IS per line

DIF IA,IB<,IS> - Display 32-bit word IA-IB (INT*4 fmt) IS per line

- (default is 10 per line)

DA  IA,IB      - Display 16-bit word IA-IB (ASCII fmt)

DL  IA,IB      - Display lines       IA-IB (ASCII fmt)

DEV IA,IB      - Display 16-bit word IA-IB (EVENT fmt)

CYB$

Commands which log output onto fex.log .....................................

CY1$

PZ  IA,IB<,IS> - LOG     16-bit word IA-IB (HEX   fmt) IS per line

PI  IA,IB<,IS> - LOG     16-bit word IA-IB (INT*2 fmt) IS per line

PZF IA,IB<,IS> - LOG     32-bit word IA-IB (HEX   fmt) IS per line

PIF IA,IB<,IS> - LOG     32-bit word IA-IB (INT*4 fmt) IS per line

PA  IA,IB      - LOG     16-bit word IA-IB (ASCII fmt)

PL  IA,IB      - LOG     lines       IA-IB (ASCII fmt)

PEV IA,IB      - LOG     16-bit word IA-IB (EVENT fmt)

SP2$

DBL$

ODP$

\bu\NOTE-1\\ Files are opened for direct access (except for DL and PL

commands) with fixed record length = RECL.

This means that you may get an error when trying to display or

log words from a short file or near the end of any file whose length

is not a multiple RECL bytes. In these cases RECL may need to be

adjusted.

SIN$

QIT$