RPD$075066001

CHP$U510  ASAP (UNIX version) - WTM

TPF$

SP2$

CEB$

Automatic Spectrum Analysis Program

DBL$

CY1$

SEC PAGE CONTENTS

010   1  HOW TO GET STARTED

020   2  GENERAL FEATURES

030   2  PROGRAM OPERATION

040   2  LOOP EXECUTION AND SYMBOL DEFINITION

045   3  SYMBOLS IN FILENAMES

050   3  FIT SPECIFICATIONS - COMMAND LIST

060   4  INPUT QUANTITIES - DEFINITIONS

070   4  BACKGROUND COMPUTATION

080   5  PEAK FINDING

090   5  SUM AREA (SAREA) CALCULATION

100   5  CENTROID CALCULATION

110   5  GAUSSIAN AREA (GAREA) CALCULATION

120   6  NON-LINEAR SEARCH FOR PEAK POSITIONS AND WIDTHS

130   6  LISTING OF RESULTS - DEFINITIONS & FORMATS

140   7  COMMENTS

SIN$

SEC$.010  HOW TO GET STARTED

BON$

CY1$

Type:  asap               ;if asap is defined in your .login or .cshrc

;files, otherwise:

Type:  /usr/hhirf/asap    ;For HHIRF DECstation and Alpha users

or

Type:  /home/upak/asap    ;For SPARCstation users

BOF$

TPF$

SP1$

SEC$.020  GENERAL FEATURES

ODP$

(1)....ASAP is intended to be used for the QUICK, EASY and maybe

DIRTY analysis of high-resolution gamma-ray spectra.

ODP$

(2)....Processes data from his- or spk-files.

ODP$

(3)....Finds peaks, estimates the background, and calculates AREAS by

two methods (a SUMMING method and GAUSSIAN fitting)

ODP$

(4)....Produces a table of PEAK locations, ENERGIES, AREAS,

estimated ERRORS, and PEAK-WIDTHS.

ODP$

(5)....Spectra are analyzed in "sections" chosen by the program.

ODP$

(6)....Sections may be up to 256 channels in length and contain

up to 16 peaks.

ODP$

(7)....Up to 250 peaks may be processed in one FIT request.

ODP$

(8)....Spectra with up to 16384 channels may be processed.

SEC$.030  PROGRAM OPERATION

BLP$

The user specifies how the fitting is to be carried out by supplying a number

of FIT specifications which are entered free-form (i.e. a command NAME

followed by zero or more data items - numbers or alphanumeric specifiers).

Command and data item delimiters are space and comma.

Commands may be given in any order.

Most of data items have DEFAULT values (see SEC# U510.050).

SEC# U510.050 gives a complete list of the FIT specifications available,

some of which are described in more detail in SEC# U510.060.

BLP$

After the fitting conditions are specified, one or more FIT requests are

entered. Subsequently, some or all of the fitting conditions may be changed and

more FIT requests entered etc., etc.

SEC$.040  LOOP EXECUTION AND SYMBOL DEFINITION

BLP$

Commands related to LOOP execution and SYMBOL definition.

CY1$

SYM = EXPRESSION - Define symbol (SYM) up to 100 symbols supported

- symbols M, N, O, P, Q, R, S are reserved

- expression syntax is same as in CHIL

- no imbedded blanks are allowed in expressions

- symbols may contain up to 4 characters (5-8 ignored)

DSYM             - Displays list of currently defined sumbols & values

LOOP N           - Starts LOOP (executed N-times) N=SYM or CONST

CMD  ....        - Nesting supported

CMD  ....        - # lines between 1st LOOP & matching ENDL = 100

ENDL             - Defines end-of-loop

- KILL (entered before ENDL) kills LOOP

- Ctrl/C - aborts loop-in-progress

- opening of CMD-file within a LOOP not allowed

TPF$

SP2$

SEC$.045  SYMBOLS IN FILENAMES

BLP$

One symbol (integer variable) may be incorporated in a filname

specification as the following examples illustrate:

CY1$

\bu\Example-1\\

syn=3

OU fil"sym".spk,new  ;Creates and opens fil3.spk

\bu\Example-2\\

I=0

LOOP 3

I=I+1

IN fil"I".spk        ;Opens (in succession)  fil1.spk, fil2.spk, fil3.spk

.

.

ENDLOOP

SEC$.050  FIT SPECIFICATIONS - COMMAND LIST

BLP$

A list (with definitions) of all ASAP commands is given below.

Note: that commands may be entered in either upper or lower case.

CY1$

CMD   DATA-------Definition (general commands) ------------

IN    fil.ext    Open data file (ext = .spk or .his)

(on VAX, his-file names must include Disk name)

CMD   fil        Open and process commands from fil.cmd

CMD   fil.ext    Open and process commands from fil.ext

TIT   Title      Enter a Title for log-file

END              Ends program

CMD   DATA-------Definition (associated with fitting) -----

KWD   KWD        See text (usually KWD=2*FWHM)     (DFLT KWD=8)

WCAL  FWA,FWB    Peak-width=FWA+FWB*SQRT(chan#)    (DFLT FWA,FWB=4,0)

BIAS  BIAS       Peak-detect threshold             (DFLT BIAS=3.0)

DEL   DX,WDNC    Step size for pos & width         (DFLT 0.0,0.05)

KFIT  KIND       Kind of FIT (GAUS OR SUMA)        (DFLT KIND=GAUS)

KBAK  KIND       Kind of BGD (FULL,FAST,ZERO)      (DFLT KIND=FULL)

BSTD  BSTD       See text (STD DEV for BGD)        (DFLT BSTD=2.0)

MPIS  N          Max # Peaks/section (MAX=16)      (DFLT MPIS=8)

VW    KIND       Width variation (NONE,LOCK,FREE)  (DFLT KIND=LOCK)

FIT   ID,LO,HI   FIT spect# ID   (chans LO thru HI)

FIT   ID         FIT spect# ID   (using previous    LO,HI specs)

FIT              FIT             (using previous ID,LO,HI specs)

Ctrl/C           Interrupts FIT in progress

TPF$

SP2$

SEC$.060  INPUT QUANTITIES - DEFINITIONS

ODP$

KWD....KWD is a basic width such that 2*KWD will include all of the

stronger peaks (see the following sections for other ways KWD is used)

ODP$

WCAL...FWA,FWB: ASAP computes FWHM from the relation FWHM=FWA+FWB*SQRT(CHAN#).

IF FWA = FWB = 0.0, ASAP estimates FWA and FWB by fitting the widths

calculated from SAREAS and PEAK HEIGHTS.

If ASAP computes FWA and FWB and if KWD was initially set to 0,

ASAP sets KWD = 2*FWHM at mid-range of the FIT request.

ODP$

BIAS...BIAS: BIAS is a PEAK HEIGHT selector (the number of standard

deviations above  BACKGROUND that a count must be

to be considered part of a PEAK)

Depending on the statistics, BIAS may need to be set to values

ranging from 2.5 to 10.0

ODP$

DEL....DX,WDNC:

DX is the fraction of a channel that PEAKS will be moved

at each step in the non-linear-search for best PEAK position

(DX= 0.0 says do no non-linear search on PEAK position).

A maximum of 10 attempts is made to find the best position.

WDNC is the step size (fraction of the initial value of FWHM)

used in the non-linear search for best peak width/s.

Adjustment of FWHM is limited to 0.3 to 3.0 times the starting

value.

If WDNC = 0.0  FWHM will not be changed.

CYB$

Peak positioning is not usually required - start with DX = 0.0

ODP$

ECAL...A,B:

A is the spectrum intercept and

B is the spectrum GAIN in Kev/channel (or whatever).

(A and B are only used for identification purposes).

ODP$

BSTD...A BACKGROUND computation parameter which effects SAREA only

and is probably best left alone.

SEC$.070  BACKGROUND COMPUTATION (FOR SAREA ONLY)

BLP$

The BACKGROUND is calculated by least squares fitting the

spectrum in a number of overlapping sections.

10*KWD or 256 (whichever is smaller) channels are considered

in each FIT.

Each section is fitted three times- the first time to the form

BGD(I)=A+B*X(I) and the last two times to the form

BGD(I)=A+B*X(I)+C*X(I)*X(I).

after each fit any channel containing a count Y(I) is thrown out of

the next fit if (Y(I)-BGD(I)).GT.BSTD*SQRTF(BGD(I)).

(see the list of definitions)

This essentially throws away the PEAKS (at least that is what it is

supposed to do).

Only the center 1/2 of each section is retained except for the first

and last sections.

I.E. the first 1/2 of the second section overlaps the

last 1/2 of the first section etc.

You should probably use BSTD=2.0 (the DEFAULT value).

TPF$

SP1$

SEC$.080  PEAK FINDING

BLP$

Peak finding is accomplished using a routine due to J. D. Larson.

This routine (used by RIP and STP) computes a "spectrum derivative",

looks for inflection points, locates the largest count near this

inflection point and tests against the peak detect threshold

specified (BIAS in this program). The differentation process uses

KWD/2 as an estimate of the basic peak FWHM and is sensitive (but

not extremely so) to this value.

It does what it does and does it rather well, I think.

SEC$.090  SUM AREA (SAREA) CALCULATION

BLP$

The SUM AREAS (SAREA'S) are calculated

by summing Y(I)-BGD(I) from I1 to I2 where

I1=(IP-KWD+1)

and

I2=(IP+KWD).

(see the list of definitions)

IP is the rough position of the peak.

I.E. Y(IP) is the maximum peak count.

If peaks are closer together than 2*KWD, the difference is split

to get the summing range.

If IDIFLO is the separation between IP and the next

peak below it and IDIFHI is the separation between IP and the next

peak above, the sum is done from I1 to I2 where,

CY1$

I1=(IP-MINF(KWD,IDIFLO/2)+1)

and

I2=(IP+MINF(KWD,IDIFHI/2)).

BLP$

Where MINF(A,B) means the smaller of A and B.

The quoted ERROR is given by,

CY1$

ERR=100*SQRTF(SUM(Y(I))+SUM(BGD(I)))/AREA

SEC$.100  CENTROID CALCULATION

BLP$

The CENTROID or best peak position may be gotten by summing

as follows from I1 to I2.

CY1$

CENTROID=SUM(I*(Y(I)-BGD(I))**2)/SUM((Y(I)-BGD(I))**2   I=I1,I2

BLP$

Notice that this is not really the CENTROID, since the square of the

net count is used rather than the net count, but this method

seems to give a better peak

position under adverse conditions.

SEC$.110  GAUSSIAN AREA (GAREA) CALCULATION

BLP$

Peaks which are well separated are fitted by the linear least squares

method to a single gaussian plus a linear background

over the range J1 to J2 where,

J1=IP-2*KWD

and

J2=IP+2*KWD.

If peaks are closer together than 2*KWD the range is extended to include

up to 16 peaks, however the DEFAULT maximum number is 8.

If peaks are still in the way, the range is reduced somewhat to exclude

them. The errors in the peak areas are calculated in the conventional

way for this type of fitting but is too involved to discuss here.

TPF$

SP2$

SEC$.120  NON-LINEAR SEARCH FOR PEAK POSITIONS AND WIDTHS

BLP$

If DX is entered (greater than 0),

an attempt is made to find the best positions for all peaks

in the section being fitted by positioning the peaks one

at a time, starting with the largest and working down to the

smallest, in steps of DX

(see the list of definitions)

channels per step.

If DX=0.0, no positioning will be done.

If no best position is found the attempts will be terminated

after 10 trys.

After positioning is completed an attempt may be made to find

the best peak widths.

Peak widths may be

held fixed (VW=NONE), varied together in a given FIT section

(VW=LOCK) or allowed to vary freely (VW=FREE).

Adjustment of peak widths is limited to 0.3 to 3.0 times

initial values.

If WDNC = 0.0 no width search will be done.

SEC$.130  LISTING OF RESULTS - DEFINITIONS & FORMATS

BLP$

Most of the output is self explanatory, with a few exceptions:

ODP$

(1)....XG of the table denotes the best position found in the positioning

operation.

ODP$

(2)....AFWHM denotes the adjusted FWHM.

ODP$

(3)....The output col labeled 100(S-G)..../G compares GAUSSIAN AREAS

(GAREA) and SUM AREAS (SAREA) and

flags (with ***) if the agreement is worse than the estimated

ERRORS predict.

ODP$

(4)....If you don't use GAUSSIAN analysis the GAREA col will contain the

approximate PEAK HEIGHT. (this is just an accident).

BLP$

In order to facilitate the reading of \b\asap.log\\ by a another

program, each type of output data line is specifically flagged for

identification. A listing of output data, associated flags, and record

formats are given below.

CY1$

\bu\Definition of output quantities flagged with FIL$\\

MO/DA/YR HR:MN:SC - Month, day, year, hour, minute, second of run

filename          - Name of file from whence spectrum was read

Format is:  (1H ,4X,2(I2,1H/),I2,2X,2(I2,1H:)I2,4X,20A4,9X,'FIL$')

If line is read on FORMAT(30A4), FIL$ flag is in word-30

\bu\Definition of output quantities flagged with TIT$\\

Title       Entered by user at ASAP run time

Format is:  (1H ,4X,19A4,35X,'TIT$')

If line is read on FORMAT(30A4), TIT$ flag is in word-30

TPF$

SP1$

SEC$.130  LISTING OF RESULTS (continued)

CY1$

\bu\Definition of output quantities flagged with DAT$\\

ID          Spetrum or histogram ID number

ILO         First channel # of peak search range (LO from FIT cmd)

IHI         Last  channel # of peak search range (HI from FIT cmd)

KWD         A "width parameter" (see HANDBOOK)

BIAS        Peak-detect threshold (in standard deviation units)

KEV/CH      Energy calibration (the B from ECAL command)

E0          Energy calibration (the A from ECAL command)

FW          Initial width parameter (the A from WCAL command)

FWB         Initial width parameter (the B from WCAL command)

WDNC        Peak width    step size (in fractional units)

DX          Peak position step size (in channel   units)

VW          Width variation flag (NONE, LOCK, FREE)

BSTD        Background control parameter for Sum-Area (see HANDBOOK)

Format is:  (1H ,4I6,7F8.4,4X,A4,F8.4,I8,11X,'DAT$')

If line is read on FORMAT(30A4), DAT$ flag is in word-30

\bu\Definition of output quantities flagged with SAP$\\

SAREA       Sum-area (see SEC# U510.090 for computation details)

ERR         Estimated error (via counting statistics with BGD subtraction)

GAREA       Gaussian-area from fitting process

ERR         Estimated error from quality-of-fit, etc.

AFWHM       Adjusted full-width at half-maximum from non-linear search

XG          Gaussian peak position from non-linear search (found in fit)

CENTROID    Peak centroid (see SEC# U510.100)

EGAM        Peak energy from CENTROID, E0 and KEV/CH

100(S-G)/S  100*(SAREA-GAREA)/SAREA

**          Says SAREA & GAREA differ by more than "uncertainties predict"

ID          Spectrum or histogram ID

Format is:  (1H ,9F10,1X,A2,3X,I8,11X,'SAP$')

If line is read on FORMAT(30A4), SAP$ flag is in word-30

SEC$.140  COMMENTS

ODP$

(1)....I don't know how well this program will work for your data.

Only you can determine that.

It's not going to work very well if the whole region of interest

is just one mass of overlapping peaks.

ODP$

(2)....Performance will depend on the fitting conditions that you

specify, of course. The DEFAULTS are set to "reasonable" values

for an "average" Ge(Li) spectrum - whatever that is.

If it is possible to supply good width coefficients (FWA & FWB)

and keep the widths of all peaks in a section locked together

(VW LOCK), results should be more satisfactory.

ODP$

(3)....In most cases it is best not to set the maximum number

of peaks per section to too large a value.

It makes the program slow and gives poor results for the smaller

peaks in the section. You will just have to find out what works

(or doesn't work) for your data.

QIT$