RPD$075066

CHP$U210  PACOR -  Physics Acq Compiler - Revised

TPF$

SP2$

CY1$

Sec Page Contents

010   1  Introduction

020   2  Hardware Description & Compiler Directives - Syntax

030   3  Hardware Description & Compiler Directives - Definitions

035   4  Module Type Table (MOTYP)                  - Definitions

040   5  Hardware Description & Compiler Directives - Discussion

050   7  Conditional (programmed) CAMAC Readout     - Syntax

060   7  Conditional (programmed) CAMAC Readout     - Discussion

070   8  Conditional (programmed) CAMAC Readout     - Example

080   9  PACOR Output Tables (for McConnell & Milner only)

SEC$.010  Introduction

BLP$

User-supplied front-end programming of the DECstation/VME-based data

acquisition system consists of two principal parts:

ODP$

(1)....A hardware description table in which the locations of all modules

to be read out are given along with associated device names (ADCs, TDCs,

etc.), parameter-IDs, function codes

for module clearing, etc.

ODP$

(2)....A conditional readout section in which certain CAMAC sub-addresses

may be read (or not) depending on pattern word bits. Given module types

of Fastbus or Ferabus may also be conditionally read.

BLP$

It is intended that the hardware description table should be more or less

self documenting with the device names serving as the primary label as well

being used in any subsequent conditional readout section to make connections

to hardware locations, etc.

CEB$

Data Stream Format

BLP$

At this first stage of development, the data stream format will be what

we will call \b\L003\\ - that is:

a sequence of (parameter-ID, data-word) pairs

terminated by two hex FFFFs.

CEB$

How to Get Started

CYB$

Type: /usr/hhirf/pacc prog    ;To compile prog.pac.

;Expanded listing & diagnostics

;are written to prog.lst

Type: /usr/hhirf/pacc prog L  ;To compile prog.pac and

;download to VME front-end.

TPF$

SEC$.020  Hardware Description & Compiler Directives - Syntax

CYB$

$ini  cnaf C,N,A,F,DATA      ;Executed at start-up time only    (200 max)

$dun  cnaf C,N,A,F,DATA      ;Executed at end-of-event or kill  (200 max)

$run  cnaf C,N,A,F,DATA      ;Executed at TRUN or STARTVME      (200 max)

---------------------------------------------------------------------------

$dla  uncondit  DT           ;Delay-time for  unconditional CAMAC   readout

$dla  condit    DT           ;Delay-time for    conditional CAMAC   readout

$dla  camac     DT           ;Delay-time for block-transfer CAMAC   readout

$dla  fastbus   DT           ;Delay-tine for                Fastbus readout

$dla  fera      DT           ;Delay-time for                Fera    readout

$dla  vme       DT           ;Delay-time for                VME     readout

;Valid DT range is 0 to 255 microseconds

---------------------------------------------------------------------------

$lat  c0C  n0N  f0F  a0A     NAME:0N              fc0F  ac0A

$lat  c0C  n0N  f0F  a0A     NAME:0N     id0N     fc0F  ac0A

---------------------------------------------------------------------------

$cam  c0C  n0N  f0F  a0A-0B  NAME:0N,0I  id0N,0I  fc0F  ac0A

$cam  c0C  n0N       a0A-0B  NAME:0N,0I  id0N,0I  fc0F  ac0A  mt=MOTYP

$fer  c0C  n0N       a0A-0B  NAME:0N,0I  id0N,0I              mt=MOTYP

$fas  c0C  n0N       a0A-0B  NAME:0N,0I  id0N,0I              mt=MOTYP

$xia  c0C  n0N  v0V  g0G

$vme  adc0N          a0A-0B  NAME:0N,0I  id0N,0I              mt=MOTYP

$vme  tdc0N          a0A-0B  NAME:0N,0I  id0N,0I              mt=MOTYP

$did  FASI,FERI,CAMI         ;IDs for illegal Fastbus, Fera, CAMAC  readout

$cid  idhi idlo              ;ID#s for hi- & lo-order parts of 100HZ clock

---------------------------------------------------------------------------

$rgat GNAM(N) PNAM(I) LO,HI              ;define raw gate GNAM(N)

....                                     ;on parameter    PNAM(I)

....                                     ;gate limits  =  LO,HI

$cgat CNAM(N) = <N>GNAM(I) OP<N>GNAM(J)..;define constructed gate CNAM(N)

....                                     ;from raw gates GNAM(I),GNAM(J)...

....                                     ;<N> is optional and implies .not.

....                                     ;OP is either .and. or .or.

....                                     ;evaluation is left-to-right

---------------------------------------------------------------------------

$kil any   LW(N),MSK         ;Kill event if any latch & mask bits match

$kil none  LW(N),MSK         ;Kill eveny if no  latch & mask bits match

$kil true  GNAM(N)           ;Kill event if gate GNAM(N) is true

$kil false GNAM(N)           ;Kill event if gate GNAM(N) is false

$rif LW(N)  MSK   MOTYP      ;If any latch & mask bits match, read MOTYP

$rif true GNAM(N) MOTYP      ;If gate GNAM(N) is true,        read MOTYP

$cdn GNAM(N) ICOUNT          ;If gate GNAM(N) true, "countdown" via ICOUNT

---------------------------------------------------------------------------

$pat  PAT =  NAME(1),I1,I2  NAME(2),J1,J2  NAME(3),K1,K2 ....

---------------------------------------------------------------------------

$lat, $cam, $fer, $fas, $xia, $vme denote: Gated-latch, CAMAC, Fera,

Fastbus, XIA & VME  modules

---------------------------------------------------------------------------

TPF$

CYB$

\bu\IMPORTANT COMMENTS RELATED TO GATES\\

ODP$

(1)....All gates are inclusive.

ODP$

(2)....All parameters on which gates are set must be \b\CAMAC\\ parameters.

ODP$

(3)....Gates are only supported for \b\CAMAC\\ modules which allow

randon access to the data. No gate support is provided for modules which

generate only compressed data readout (i.e. sparce data readout) -

the \b\LECROY-2277\\ TDC is a prime example.

SEC$.030  Hardware Description & Compiler Directives - Definitions

BLP$

The following list of definitions apply to quantities specified in the

\b\$lat, $cam, $fas, $fer, $xia, and $vme\\ directives above.

CY1$

\b\c \\    ;Labels the crate number \b\(see Note-1)\\   - denoted by 0C

\b\n \\    ;Labels the slot  number                - denoted by 0N

\b\a \\    ;Labels sub-address range for read      - denoted by 0A-0B

\b\f \\    ;Labels function-code for read          - denoted by 0F

\b\fc\\    ;Labels function-code for clear         - denoted by 0F

\b\ac\\    ;Labels sub-address for clear           - denoted by 0A

\b\v \\    ;Labels the virtual station number      - denoted by 0V - $xia only

\b\g \\    ;Labels the group number (optional)     - denoted by 0G - $xia only

\b\adc\\   ;Labels CAEN ADC number                 - denoted by 0N - $vme only

\b\adc\\   ;Labels CAEN ADC number                 - denoted by 0N - $vme only

\b\mt\\    ;Labels Fera or Fastbus module-type     - denoted by MOTYP

;\b\(Required for all Fera & Fastbus modules - optional for CAMAC)\\

\b\NAME\\  ;User-supplied name for program reference (12 bytes max)

;\b\0N\\ denotes first index (corresponds to sub-addr 0A)

;\b\0I\\ denotes the   index increment

\b\id\\    ;Labels parameter-ID and parameter-ID-increment entry

;\b\0N\\ denotes parameter-ID corresponding to sub-address 0A

;\b\0I\\ denotes parameter-ID increment

---------------------------------------------------------------------------

\b\PAT\\   ;See discussion of \b\$pat\\ (SECT# 040)below

---------------------------------------------------------------------------

\bu\Note-1 CAMAC crate numbers are supported as indicated below:\\

Crate# 0-7           are supported for CAMAC data readout (via $lat & $cam)

Crate# 0-7 and 10-17 are supported for Fera  data readout (via $fer)

Crate# 0-7 and 10-17 are supported for writing (i.e. via $ini, $dun & $run)

---------------------------------------------------------------------------

TPF$

SP1$

SEC$.035  Module Type Table (MOTYP) - Definitions

CY1$

\b\MOTYP = LRS_4300\\     for LECROY-4300   (Fera    ADC) - Code-01

\b\MOTYP = GAN_812F\\     for GANELEC-812F  (Fera    TDC) - Code-02

\b\MOTYP = SILENA_4418\\  for SILENA-4418   (Fera    ADC) - Code-03

\b\MOTYP = BAKLASH\\      for Clover module (Fera    ADC) - Code-06 ***

\b\MOTYP = BAKLASH\\      for Clover module (Fera    ADC) - Code-07 ***

\b\MOTYP = BAKLASH\\      for Clover module (Fera    ADC) - Code-08 ***

\b\MOTYP = AD_413\\       for ORTEC AD_413  (Fera    ADC) - code-09

\b\MOTYP = LRS_1885\\     for LECROY-1885   (Fastbus ADC) - Code-11

\b\MOTYP = PHIL_10C6\\    for PHILLIPS-10C6 (Fastbus TDC) - Code-12

\b\MOTYP = LRS_1872\\     for LECROY-1872   (Fastbus TDC) - Code-13

\b\MOTYP = LRS_1875\\     for LECROY-1875   (Fastbus TDC) - Code-13

\b\MOTYP = LRS_1881\\     for LECROY-1881   (Fastbus ADC) - Code-14

\b\MOTYP = LRS_1877\\     for LECROY-1877   (Fastbus ADC) - Code-15

\b\MOTYP = PHIL_7164\\    for PHILLIPS-7164 (CAMAC   ADC) - Code-21

\b\MOTYP = PHIL_7186\\    for PHILLIPS-7186 (CAMAC   TDC) - Code-22

\b\MOTYP = LRS_2277\\     for LECROY-2277   (CAMAC   TDC) - Code-23

\b\MOTYP = SILENA_4418C\\ for SILENA-4418   (CAMAC   ADC) - Code-24

\b\MOTYP = LRS_4300C\\    for LECROY-4300C  (CAMAC   ADC) - Code-26

\b\MOTYP = LRS_3377C\\    for LECROY-3377C  (CAMAC   ADC) - Code-27

\b\MOTYP = XIA_TIME\\     for XIA TIME      (CAMAC   MOD) - Code-28

\b\MOTYP = AD_413C\\      for ORTEC AD_413C (CAMAC   ADC) - Code-29

\b\MOTYP = CAEN-775\\     for CAEN-775 TDC                - Code-41

\b\MOTYP = CAEN-785\\     for CAEN-785 ADC                - Code-42

---------------------------------------------------------------------------

CYB$

*** Data readout by the BAKLASH module types:

BLP$

Currently there are three module types: BAKLASH, BAKLASH2 and

BAKLASH3. Zero-suppression is done on each module individually.

The data from a clover module and corresponding output vs module type is:

CY1$

Word#  Parameter                 Module types which readout listed param

-----  -----------------------   ---------------------------------------

1  VSN (internal use only)

2  ??? (junk word)

3  Hit pattern               BAKLASH, BAKLASH2, BAKLASH3

4  Hires GE                  BAKLASH, BAKLASH2, BAKLASH3

5  Hires GE                  BAKLASH, BAKLASH2, BAKLASH3

6  Hires GE                  BAKLASH, BAKLASH2, BAKLASH3

7  Hires GE                  BAKLASH, BAKLASH2, BAKLASH3

8  GE time                   BAKLASH, BAKLASH2

9  GE time                   BAKLASH, BAKLASH2

10  GE time                   BAKLASH, BAKLASH2

11  GE time                   BAKLASH, BAKLASH2

12  side channel              BAKLASH, BAKLASH2

13  side channel              BAKLASH, BAKLASH2

14  side channel              BAKLASH, BAKLASH2

15  BGO                       BAKLASH

16  BGO                       BAKLASH

TPF$

SP1$

SEC$.040  Hardware Description & Compiler Directives - Discussion

ODP$

\b\$ini\\...defines CAMAC operations which are to be executed at

"init time" only (i.e. when the \b\INITVME\\ command is issued).

Only operations for which F.GT.7 are allowed.

ODP$

\b\$run\\...defines CAMAC operations which are to be executed each

time a \b\TRUN or STARTVME\\ command is executed. Only operations for

which F.GT.7 are allowed.

ODP$

\b\$dun\\...defines CAMAC operations which are to be executed at

the end of every event (whether normal or killed). Only

operations for which F.GT.7 are allowed.

ODP$

\b\$dla\\...specifies a time delay (in microseconds) to be imposed

prior to the readout of the specified "class" of modules or readout

type (conditional or unconditional).

ODP$

\b\$lat\\...specifies that a gated latch is to be read and saved.

That latch may be subsequently referenced (in pattern register

definitions or in conditional CAMAC readout) by the associated NAME.

The latch will be loaded into the data stream only if the \b\id\\

field is entered.

ODP$

\b\$cam\\...defines the sub-addresses and associated parameter NAMEs

and IDs to be read from one CAMAC module. A given \b\$cam\\ can

only reference one module but one module may be referenced by more

than one \b\$cam\\. If the \b\fc & ac\\ fields are entered, that

module will be cleared at the end of each event. Otherwise, an

external clear is expected.

FIP$

Any \b\$cam\\ entries which are not referenced in the conditional

CAMAC readout program will be read unconditionally for every

"un-killed" event.

ODP$

\b\$fer\\...defines Ferra modules, sub-addresses, associated parameter

NAMEs and IDs to be read. The module type \b\mt=MOTYP\\ must be given

and MOTYP must be either \b\LRS_4300 or GAN_812F\\.

All existing parameters specified for a given module type will be

loaded into the data stream unless a conditional readout is

specified via the \b\$rif\\ directive defined below.

ODP$

\b\$fas\\...defines Fastbus module sub-addresses, associated parameter

NAMEs and IDs to be read. The module type \b\mt=MOTYP\\ must be given

and MOTYP must be either \b\LRS_1885 or PHIL_10C6\\.

All existing parameters specified for a given module type will be

loaded into the data stream unless a conditional readout is

specified via the \b\$rif\\ directive defined below.

ODP$

\b\$xia\\...defines the crate#, slot#, virtual-station#, and

group# (c0C, n0N, v0V, g0G) for XIA modules which are to be read.

The group number is optional at this time.

ODP$

\b\$vme\\...defines the adc# (or tdc#), sub-addresses, associated

parameter NAMEs and IDs to be read. The module type must also be

given.

TPF$

SP2$

SEC$.040  Hardware Description & Compiler Directives - Discussion (cont)

ODP$

\b\$rif\\...defines the conditional readout of a given Fera, Fastbus

or CAMAC module type. Only one conditional entry is allowed for a

given module type. It says, if any bits in the specified latch word

LW(N) and mask MSK match,  read module type MOTYP.

FIP$

Note: LW must be a name defined in a previous \b\$lat\\ specification

and the index N must be a number (not a symbol) for now at least.

ODP$

\b\$kil\\...defines the conditional "killing" of events. The

\b\any\\-type says, kill the event if any bits in the specified

latch word LW(N) and mask MSK match. The \b\none\\-type says kill

the event if no bits match.

ODP$

\b\$did\\...specifies default parameter IDs \b\FASI & FERI\\ to be

used for any illegally reported slot# and sub-address from Fastbus

and Fera readout, respectively. This is for diagnostic purposes

only. For example, you could histogram parameters FASI & FERI and

look for non-zero spectra. If you do not enter \b\$did\\, PAC will

assign to FASI and FERI the values MAX-ID+1 and MAX-ID+2,

respectively, where, MAX-ID is the maximum parameter-ID specified

anywhere in the PAC program.

ODP$

\b\$cid\\...specifies two ID-numbers to be associated with the hi- and

lo-order parts of VME processor's internal 100HZ clock which are to be

entered into the data stream. If the \b\#cid\\ directive is not entered,

then no clock parameters will be recorded.

ODP$

\b\$pat\\...provides the user with a method of defining one or more

(up to 5)

indexed pattern word names (denoted PAT in the definition table)

each of whose elements refer to one bit of one latch word. The

idea is to make it easier to construct loops which involve multiple

latch words.

FIP$

In the definition table, \b\PAT\\ denotes a user-supplied name

associated  with a multi-word  bit-pattern composed of bits from

one or more latch-words which have been defined via the \b\$lat\\

directive. The example given in the table uses bits I1-I2 from

NAME(1), J1-J2 from NAME(2) .... In subsequent conditional programming

\b\PAT(I)\\ refers to 1 bit of one latch-word.

CEB$

COMMENTS

ODP$

(1)....At this time no scalar symbols (representing numbers)

are supported for any of the

$-type directives. Examble: for the $ini directive, C,N,A,F,DATA

must all be numbers. DATA must be hex and the DATA field must be

present for \b\ALL\\ CAMAC function codes whether used or not.

ODP$

(2)....Hex numbers are entered with a numeric first-digit and a

trailing "h" or "H". That is, in the usual way.

ODP$

(3)....A ! or ; introduces a comment field on a statement line.

ODP$

(4)....Any non-printing characters \b\(including TABs)\\ will cause

the compiler to \b\ABORT!\\

CY1$

TPF$

SP2$

SEC$.050  Conditional (Programmed) CAMAC Readout - Syntax

CY1$

In the following list:

LW(I)   denotes Ith latch-wd      defined via $lat      - always indexed

PAT(J)  denotes Jth pattern-bit   defined via $pat      - always indexed

ADC(J)  denotes Jth named-hwd dev defined via $cam      - always indexed

GNAM(J) denotes jth gatename defined via $rgat or $cgat - always indexed

MSK is a mask - number or scaler symbol

LAB is a statement label

IFU(PAT(J))GOTO LAB    ;If latch-bit implied by PAT(J) not set,  GOTO LAB

IFS(PAT(J))GOTO LAB    ;If latch-bit implied by PAT(J)  is set,  GOTO LAB

IFT(GNAM(J))GOTO LAB   ;If raw or calculated gate GNAM(J) true,  GOTO LAB

IFF(GNAM(J))GOTO LAB   ;If raw or calculated gate GNAM(J) false, GOTO LAB

IFA(LW(I),MSK)GOTO LAB ;If any bits set in MSK are set in LW(I), GOTO LAB

IFN(LW(I),MSK)GOTO LAB ;If no  bits set in MSK are set in LW(I), GOTO LAB

GOTO LAB               ;Unconditional GOTO LAB

READ ADC(J)            ;Read ADC(J) (C,N,A,F defined in $CAM statement)

CNAF C,N,A,F,DATA      ;Explicit CNAF (DATA field is optional)

LABL  CONTINUE         ;Labeled continue statement

SP2$

SEC$.060  Conditional (Programmed) CAMAC Readout - Discussion

CYB$

ODP$

(1)....Statement labels must start in col-1 and be no more than 4 characters.

ODP$

(2)....Statements must start in col-7 or greater (like FORTRAN).

ODP$

(3)....A ! or ; introduces a comment field on a statement line.

ODP$

(4)....Hex numbers are entered with a numeric first-digit and a

trailing "h" or "H". That is, in the usual way.

ODP$

(5)....Any non-printing characters \b\(including TABs)\\ will cause

the compiler to \b\ABORT!\\

TPF$

SP2$

SEC$.070  Conditional (Programmed) CAMAC Readout - Example

BLP$

The \b\$lat\\ directive specifies devices (normally gated latches)

which are to be read first and saved under the name specified

for later use in bit-tests or in constructing multi-word bit patterns.

Such data may also be $kil-tested for event crashing.

BLP$

If \b\fc0F & ac0A\\ are included in the \b\$lat & $cam\\

directives, clearing of the modules will be automatic. If you do

not enter this, you must supply explicit CNAFs or clear by some

other method.

BLP$

The \b\$lat & $cam\\ directives on the preceding page imply full readout

unless "conditional" readout is specified by a subsequent "conditional

readout" program section. Anything not included in the

conditional section is still readout.

A conditional readout program section might look as follows:

CYB$

Example--1 ----------------------------------------------------------------

CY1$

I=0                     ;Init pattern-word bit counter

J=20                    ;Init ADC, TDC, index value

LOOP 40                 ;Loop over 40 "detectors"

I=I+1                   ;Increment bit-counter

J=J+1                   ;Increment ADC, TDC index

IFU(PAT(I))GOTO LAB1    ;Tst pattern bit - read if set

READ ADC1(J)            ;Read ADC1

READ ADC2(J)            ;Read ADC2

READ TDC1(J)            ;Read TDC1

READ TDC2(J)            ;Read TDC2

LAB1  ENDLOOP                 ;End-of-loop

CYB$

Example-2 -----------------------------------------------------------------

CY1$

$ini cnaf 0,5,0,9,0H

$dun cnaf 0,5,0,16,222h

$lat c00 n02 f00 a00    GLAT:1              fc09 ac00

$cam c00 n05 f00 a00-07 CADC:1,1  id01,1    fc09 ac00   dt120

$fas c01 n02 a00-95     FADC:1,1  id101,1   mt=lrs_1885

$fas c01 n10 a00-95     FTDC:1,1  id201,1   mt=phil_10c6

$pat PAT = GLAT(1)1,16

$kil none GLAT(1) 0FFFFh

I=0

LOOP 4

I=I+1

IFU(PAT(I)) GOTO L1

READ CADC(I)

L1    ENDL

TPF$

SP1$

SEC$.080  PAC Output Tables - POB Structure

CY1$

******************  WD#  *****************************************DIRECTORY

IOF  #ENT  DELAY -  1-3  ;IOF locates PAC-SOURCE FILENAME

IOF  #ENT  DELAY -  4-6  ;IOF locates CAMAC CRATE TABLE

IOF  #ENT  DELAY -  7-9  ;IOF locates CNAF-INIT LIST

IOF  #ENT  DELAY - 10-12 ;IOF locates CAMAC   MODULE TABLE

IOF  #ENT  DELAY - 13-15 ;IOF locates FASTBUS MODULE TABLE

IOF  #ENT  DELAY - 16-18 ;IOF locates FERA    MODULE TABLE

IOF  #ENT  DELAY - 19-21 ;IOF locates GATED LATCH    TABLE

IOF  #ENT  DELAY - 22-24 ;IOF locates GATE PARAMETER READOUT TABLE

IOF  #ENT  DELAY - 25-27 ;IOF locates RAW         GATE SPEC  TABLE

IOF  #ENT  DELAY - 28-30 ;IOF locates CALCULATED  GATE SPEC  TABLE

IOF  #ENT  DELAY - 31-33 ;IOF locates COUNT-DOWN LIST

IOF  #ENT  DELAY - 34-36 ;IOF locates CONDITIIONAL  KILL TABLE

IOF  #ENT  DELAY - 37-39 ;IOF locates UNCONDITIONAL READOUT TABLE

IOF  #ENT  DELAY - 40-42 ;IOF locates CONDITIOINAL  READOUT PROGRAM

IOF  #ENT  DELAY - 43-45 ;IOF locates CNAF-LIST FOR CONDITIONAL READOUT

IOF  #ENT  DELAY - 46-48 ;IOF locates ID-LIST   FOR CONDITIONAL READOUT

IOF  #ENT  DELAY - 49-51 ;IOF locates CAMAC   COND MODULE-TYPE  READOUT

IOF  #ENT  DELAY - 52-54 ;IOF locates FASTBUS COND MODULE-TYPE  READOUT

IOF  #ENT  DELAY - 55-57 ;IOF locates FERA    COND MODULE-TYPE  READOUT

IOF  #ENT  DELAY - 58-60 ;IOF locates CAMAC     ID-TABLE

IOF  #ENT  DELAY - 61-63 ;IOF locates FASTBUS   ID-TABLE

IOF  #ENT  DELAY - 64-66 ;IOF locates FERA      ID-TABLE

IOF  #ENT  DELAY - 67-69 ;IOF locates WINDUP  CNAF LIST

IOF  #ENT  DELAY - 70-72 ;IOF locates RUN     CNAF LIST

IOF  #ENT  DELAY - 73-75 ;IOF locates XIA MODULE TABLE

IOF  #ENT  DELAY - 76-78 ;IOF locates CAEN COND MODULE-TYPE READOUT

IOF  #ENT  DELAY - 79-81 ;IOF locates CAEN ADC HARDWARE MAP

IOF  #ENT  DELAY - 82-84 ;IOF locates CAEN TDC HARDWARE MAP

IOF  #ENT  DELAY - 85-87 ;IOF locates CAEN ADC ID-TABLE

IOF  #ENT  DELAY - 88-90 ;IOF locates CAEN TDC ID-TABLE

IOF  #ENT  DELAY - 91-93 ;IOF locates 100HZ CLOCK-ID TABLE

TPF$

SEC$.080  PAC Output Tables - POB Structure (continued)

CY1$

*************************************************PAC FILENAME.......(01-03)

ASCII FILENAME (80 BYTES)

*************************************************CRATE-LIST.........(04-06)

CRATE#

*************************************************CNAF INIT-LIST.....(07-09)

CNAF DATA

*************************************************CAMAC MODULE TABLE (10-12)

CN0T           (Packed C,N,dummy,MODTYP)

*************************************************FAST  MODULE TABLE (13-15)

CN0T           (Packed C,N,dummy,MODTYP)

*************************************************FERA  MODULE TABLE (16-18)

CN0T           (Packed C,N,dummy,MODTYP)

*************************************************GATED-LATCH TABLE..(19-21)

CNAF

*************************************************GATE READ TABLE....(22-24)

C N A F MOTYP

*************************************************RAW GATE SPEC TABLE(25-27)

GPI LO HI PATNDX MSK

*************************************************CAL GATE SPEC TABLE(28-30)

See separate discription

*************************************************COUNT-DOWN-TABLE...(31-33)

PATNDX MSK COUNT

*************************************************COND KILL TABLE....(34-36)

PATNDX MSK KILTYP

*************************************************UN-COND CAMAC READ.(37-39)

CNAF   ID

*************************************************COND-CAMAC PROGRAM.(40-42)

PATNDX MSK T-IDX T-NUM T-NXT F-IDX F-NUM F-NXT

*************************************************COND-CAMAC CNAFs...(43-45)

CNAF

*************************************************COND-CAMAC ID-TABL.(46-48)

ID

*************************************************CAMAC   MOTY READ..(49-51)

PATNDX MSK MOTY

*************************************************FASTBUS MOTY READ..(52-54)

PATNDX MSK MOTY

*************************************************FERA    MOTY READ..(55-57)

PATNDX MSK MOTY

*************************************************CAMAC     ID-TABLE (58-60)

ID (32,32,8) = 8192

*************************************************FASTBUS   ID-TABLE (61-63)

ID (256,32)  = 8192

*************************************************FERA      ID-TABLE (64-66)

ID (32,576)  = 18432

*************************************************WINDUP/CLR TABLE...(67-69)

CNAF DATA

*************************************************RUN CNAF   TABLE...(70-72)

CNAF DATA

*************************************************XIA MODULE TABLE...(73-75)

CNVG           (Packed C,N,VSN,GR)

*************************************************CAEN    MOTY READ..(76-78)

PATNDX MSK MOTY

*************************************************CAEN ADC HWD MAP...(79-81)

ADC (10)

*************************************************CAEN TDC HWD MAP...(82-84)

TDC (10)

*************************************************CAEN ADC ID-TABLE..(85-87)

ID (34,10) = 340

*************************************************CAEN TDC ID-TABLE..(88-90)

ID (34,10) = 340

TPF$

SP2$

SEC$.080  PAC Output Tables - POB Structure (continued)

CY1$

*************************************************CLOCK ID-TABLE.....(91-93)

CLOCID(2)

===========================================================================

DBL$

CY1$

PATNDX = Pattern-word index

MSK    = mask - (do IAND(LATCH(PATNDX),MSK) and test for .TRUE. or .FALSE.

T-IDX  = CNAF/ID table indices     if test TRUE

T-NUM  = CNAF/ID #entries to use   if test TRUE

T-NXT  = next COND_PROG index      if test TRUE

F-IDX  = CNAF/ID table indices     if test FALSE

F-NUM  = CNAF/ID #entries to use   if test FALSE

F-NXT  = next COND-PROG index      if test FALSE

SIN$

TPF$

SP1$

SEC$.080  PAC Output Tables - POB Structure (continued)

CY1$

-------------------------------------------------------------------------

\B\GATE PARM READ TABLE\\

C  N  A  F  MOTYP

C  N  A  F  MOTYP

.

.

\B\RAW GATE SPECIFICATION TABLE\\

GPI LO HI PATNDX MSK  ;GPI    = gate parm index (index in read table)

;LO, HI = gate limits

;PATNDX = pattern word number (index)

;MSK    = defines bit# to set if gate true

.

.

\B\CALCULATED GATE SPECIFICATION TABLE\\

PATNDX                ;pattern array index (word#)

MSK                   ;MSK defines bit# to set

NOTCODE               ;\BU\ 0 \\ implies normal \BU\ 1 \\ implies .not.

IDX                   ;index in raw gate specification table

OPCODE                ;\BU\ 0 \\ implies .and. \BU\ 1 \\ implies .or.

NOTCODE

IDX                   ;index in raw gate specification table

ENDCODE = -1          ;\BU\ -1 \\ implies end of spec for this bit

PATNDX

MSK

NOTCODE

IDX                   ;index in raw gate specification table

ENDCODE = -1

PATNDX

MSK

NOTCODE

IDX

OPCODE

NOTCODE

IDX

OPCODE

NOTCODE

IDX

ENDCODE = -1

.

.

\B\COUNT DOWN LIST\\

PATNDX  MSK  COUNT    ;down-count if bit is set

.

.

QIT$