RPD$075066

CHP$U325 - hisadd - Multiple-CPU Processing Aid - WTM

TPF$

SP1$

CYB$

Sec Page Contents

010   1  Introduction and General Features

020   2  List of Commands

030   3  Example

SEC$.010  Inproduction and General Features

CYB$

Using Multiple CPUs to Process List Data into Standard ORPH Histigrams

BLP$

Program \b\hisadd\\ is written to support the use of multiple CPUs in

processing list data into standard ORPH histogram files. The following

method is assumed.

ODP$

o......The user prepares a scanning program in the usual way.

ODP$

o......Let the list-data input be from \b\listdata.ldf\\.

ODP$

o......Then run a number of identical scan processes from different

CPUs (\b\alphalnx0, alphalnx1, ... alphalnx9\\ for example).

ODP$

o......All scan processes input from different regions (specified files or

specified records within specified files, etc.)

of \b\listdata.ldf\\.

ODP$

o......All scan processes generate output to histogram files with different

names or at least different paths.

ODP$

o......Use \b\damm\\ to examine the different output histograms for

gain match problems or whatever.

ODP$

o......Finally, use \b\hisadd\\ to add all of the scan-generated his-files

to a final his-file \b\"total.his"\\.

CEB$

General Features

ODP$

o......Any scan process which produces the scandard .his & .drr structures

will work.

ODP$

o......The ability to look at different regions of the \b\LDF-file\\ is

automatically provided.

ODP$

o......The user is not required to modify his scan program in any way.

ODP$

o......The user is required to learn very little which is new.

CEB$

(see next page for a list of hisadd commands)

TPF$

SP2$

SEC$.020  List of hisadd commands

CYB$

lon/lof            - Turn Log-output (to LU7) ON/OFF (default = ON)

cmd  filnam        - Processes commands from filnam.cmd

in   inpfile.his   - Opens input  his-file - .his & .drr must already exist

- cannot be same as output

ou   outfile.his   - Opens output his-file - .his & .drr must already exist

- cannot be same as input but must have same structure

init outfile.his   - Creates & initializes   outfile.his & outfile.drr

- with same attributes as inpfile.his & inpfile.drr

- outfiles .his & .drr must NOT already exist

stat               - Displays files which are open

cli                - close input files

clo                - close output files

zero               - Zeros outfile.his

add                - Adds inpfile.his to outfile.his

- 1-D, 2-D, I*2 and I*4 supported

Ctrl/C             - Interrupts (aborts) an ADD-in-progress

- Interrupts (aborts) a ZERO-in progress

- Interrupts (aborts) execution of a command-file

WAIT  MS           - Suspends execution for MS milliseconds

(60,000 Milliseconds is the maximum allowed)

WO                 - Suspends execution until you  type [RETURN]

(it prompts you to: Type [RETURN] to continue

end                - Ends program hisadd

CEB$

(see next page for an example)

TPF$

SP1$

SEC$.030  Example

BLP$

Lets take a simple example. Assume that you wish to use 10 processors

(\b\alphalnx0 - alphalnx9\\) to process data from one \b\ldf-file\\.

In this example, each processor will process the data from one header of

the \b\ldf-file\\ and finally \b\hisadd\\ will be used to sum the results

of all scan processes into a final \b\his-file\\.

CY1$

\bu\Assume the Following:\\

ODP$

o......The scan process is called \b\myscan\\

ODP$

o......The data file to be processed in \b\/tera/myname/Dmydata/listdata.ldf\\

ODP$

o......All scan processes are run in directory \b\/tera/myname/Dscans/\\

ODP$

o......Program \b\hisadd\\ is run in directory \b\tera/myname/Dhistot/\\

ODP$

o......Each scan process will process one file (each starting from a different

header on the \b\ldf-file\\.

CY1$

\bu\Do the following:\\

ODP$

o......Prepare the scanning program \b\myscan\\ in the usual way.

ODP$

o......Open windows & login to \b\alphalnx0, alphalnx1, ... alphalnx9\\.

CY1$

On \b\alphalnx0\\ type:

CY1$

\b\cd /tera/myname/Dscans/\\

\b\myscan scan0\\

\b\file /tera/myname/Dmydata/listdata.ldf\\

\b\find 1\\

\b\goen 1\\

On \b\alphalnx1\\ type:

CY1$

\b\cd /tera/myname/Dscans/\\

\b\myscan scan1\\

\b\file /tera/myname/Dmydata/listdata.ldf\\

\b\find 2\\

\b\goen 1\\

.

.

.

On \b\alphalnx9\\ type:

CY1$

\b\cd /tera/myname/Dscans/\\

\b\myscan scan9\\

\b\file /tera/myname/Dmydata/listdata.ldf\\

\b\find 10\\

\b\goen 1\\

CEB$

(continued on next page)

TPF$

SEC$.030  Example (continued)

SP1$

BLP$

Prepare a command file \b\addum.cmd\\ for \b\hisadd\\ which looks like the

one shown below.

(Actually, you probably should prepare this file before you start the

scan processes to make sure that you get all of the paths & names

defined correctly).

CYB$

in /tera/myname/Dscans/scan0.his

init /tera/myname/Dhistot/scantot.his

add

in /tera/myname/Dscans/scan1.his

add

in /tera/myname/Dscans/scan2.his

add

in /tera/myname/Dscans/scan3.his

add

in /tera/myname/Dscans/scan4.his

add

in /tera/myname/Dscans/scan5.his

add

in /tera/myname/Dscans/scan6.his

add

in /tera/myname/Dscans/scan7.his

add

in /tera/myname/Dscans/scan8.his

add

in /tera/myname/Dscans/scan9.his

add

end

\bu\To rum hisadd, type:\\

\b\hisadd\\

\b\cmd addum\\

QIT$