RPD$075066

CHP$U100  README - Directory to HRIBF Software

TPF$

SP1$

DBL$

CYB$

Sec Page Contents

010   1  Introduction

020   2  Documentation List

030   6  Data Acquisition - Introduction

040   7  Data Acquisition - Block Diagram of System

050   8  Data Acquisition - List of Software Modules

060   9  Data Acquisition - List of Things to be Done

070  10  Data Acquisition - Brief Description of System Operation

080  12  Data Processing, Analysis and Display

090  13  HRIBF List Data Tape Structure

100  16  HRIBF List Data File Structure

110  17  HRIBF Histogram Tape and File Structures

120  19  UPAK - Software Xport

130  20  UPAK - Comments

SIN$

SEC$.010  Introduction

BLP$

The HRIBF facility provides a number of programs, software

packages and libraries which are intended to enable the user

to efficiently acquire, process and analyze experimental data.

These software tools are also available for export.

Documentation is avaliable in hardcopy form. See:

CY1$

\b\ORPHAS MANUAL\\   for data acquisition related software

\b\ORPH UBOOK 2000\\ for data processing and general purpose software

\b\ORPH XBOOK 2003\\ for the experts, the brave or the foolish

BLP$

Most documents are also available \b\online\\. You may print or

view on-screen the latest version of the document of your choice by using

\b\dodoc & viewdoc\\.

If you wish to use \b\viewdoc\\, then your \b\PATH\\ must include

\b\/usr/local/bin\\.

ODP$

\b\dodoc\\...Prints entire documents on the printer of your choice.

To get a list of the available documents, just type \b\dodoc\\.  Some examples

of using \b\dodoc & pdoc\\ to print the document \b\readme\\ follow:

CY1$

\b\dodoc readme\\         ;Prints one-sided on default Postscript printer

\b\dodoc readme ps04\\    ;Prints one-sided on printer ps04

\b\dodoc readme double\\  ;Prints two-sided on  ps01 in Rm T-210

\b\pdoc  readme\\         ;Prints one-sided on cp6000

\b\pdoc  readme double\\  ;Prints two-sided on cp6000

ODP$

\b\viewdoc\\...Displays the document on-screen. To get a list of the

available documents, just type \b\viewdoc\\.  With viewdoc you can select

pages to print or print the entire document.  For example, to view the

this document \b\readme\\, type

CY1$

\b\viewdoc readme\\

TPF$

SP1$

SEC$.020  Documentation List

SP1$

CY1$

\bu\Data Acquisition\\

SP1$

BON$

BLP$$

The following are available in the \u\ORPHAS Handbook\\ & via

\u\dodoc\\ & \u\viewdoc\\

SP1$

DBL$

CYB$

VMEsys-----VME front-end acquisition system - comprehensive

hhirf_adc--HHIRF ADC interface - comprehensive description

modu_setup-CAMAC module setup code - for many different module types

vmexxlib---Describes routines for read/write/setup of CAEN ADCs & TDCs

acqlib-----Describes data acquisition routine library for workstations

pacman-----Data acquisition manager - description, operation, etc

pacor------Physics Acquisition Compiler - Hi-level language for data acq

scad-------CAMAC scaler display (ASCII and soft-meter display)

scop-------CAMAC scaler display (for accelerator operator)

scat-------CAMAC scalers (how to copy to tape/file during data acq)

ddman------Digital Display Manager for beam profile displays

TPF$

SP1$

SEC$.020  Documentation List (continued)

SP1$

CY1$

\bu\Data processing, Display and Analysis\\

BON$

BLP$$

The following are available in the \u\ORPH UBOOK 2000\\ & via

\u\dodoc\\ & \u\viewdoc\\

CYB$

scanor-----Provides for user customized histogramming of list-data

lemor------Program to examine, copy, modify-copy list-data (tape or file)

damm-------Display, Analysis and Manipulation Module for histograms

chil-------Comprehensive Histogramming Language for tape/file scanning

tapes------Describes structure of HRIBF list-data tapes & files

swapo------Byte swaps HIS-, SPK- & LDF-files (in place)

asap-------Automatic Spectrum Analysis program

banco------Copies Ban-files to/from edit compatible form

bando------Provides for editing & modifying Ban-files

dvd--------Creation and use of DVDs & CDs in data archiving & processing

SP1$

CY1$

\bu\Control Systems\\

BON$

BLP$$

The following are available via \u\dodoc\\ & \u\viewdoc\\

CYB$

dcon-------DRS control system - user interface

stepit-----Stepping motor control for DRS target ladders

rcon-------RMS control system - user interface

hvln-------HV-control & LN-filling-control user interface for RMS

SP1$

CY1$

\bu\Libraries\\

BON$

BLP$$

The following are available via \u\dodoc\\ & \u\viewdoc\\

CYB$

lnflib-----Library of routines supporting LN-filling & HV-control on RMS

orphlib----General routine library for HRIBF user support

acqlib-----Describes data acquisition routine library for workstations

vmexxlib---Describes routines for read/write/setup of CAEN ADCs & TDCs

TPF$

SP1$

SEC$.020  Documentation List (continued)

SP1$

CY1$

\bu\Miscellaneous Utilities\\

BON$

BLP$$

The following are available in the \u\ORPH UBOOK 2000\\ & via

\u\dodoc\\ & \u\viewdoc\\

CYB$

baco-------Converts Concurrent "backup tapes" to "stu format"

charge-----Predicts equilibrium charge state distributions (gas & solid)

exab-------Exabyte exerciser

fex--------File examine program

fitu-------X,Y fitting via Spline, Linear & non-Linear LSQ (with display)

funkyfit---X,Y fitting via linear-least-squares method

formats----List data structure of HRIBF tapes and files (LDFs)

kineq------Kinematics & Q-values (relativistic & non-relavistic)

mep--------Describes energy calibration of HRIBF Tandem by time-of-flight

nuci-------Nuclear information program

stope------Stopping power calculations with    graphical display

stopi------Stopping power calculations with no graphical display

stopx------Stopping power and range calculations

stu--------Simple Transfer Utility for xfer of files between systems

txx--------Text formatting program

txxps------Converts output of program txx to postscript

upak-------Describes HRIBF UPAK, How to get it, tips on using it

upak.ftp---Describes available versions of UPAK and how to retrieve

TPF$

SP1$

SEC$.020  Documentation List (continued)

CY1$

\bu\For the Experts only\\

BON$

BLP$$

The following are available in the \u\ORPH XBOOK 2003\\

CYB$

acqsetup---Setup of workstation and VME for data acquisition

alphahard--Alpha Power Supply CPU Board

multivme---Describes setup & use of multiple VME crates for data acq

trigger----Describes VME event trigger module used at HRIBF

lnf--------LN2 filling system - workstation setup

lnflib-----LN2 filling system - routine library

lnfutil----LN2 filling system - asorted utilities

rmslib-----RMS control system - routine library

morerms----RMS control system - routine library (more)

moredrs----DRS conrtol system - routine library

rmsigc-----RMS/DRS ion-gauge readout & control routine library

varian-----Varian - interactive readout & control

txxps------TXX to postscript converter

poblis-----Displays/logs tables generated by program pacor

BOF$

SIN$

CHP$U100  README - HRIBF Data Acquisition

TPF$

SP1$

SEC$.030  Data Acquisition - Introduction

SP1$

DBL$

BLP$

This document is intended to introduce the novice user to the architecture

of the HRIBF data acquisition system and to serve as

a directory to other documents which will provide the details needed

for a successful operation of the system.

BLP$

This document is not to be viewed as a cookbook for preparation and

operation of the HRIBF data acquisition system. But it is hoped that this

document will provide the new user with a better "feel" for how the

system operates and make the system seem less mysterious and arbitrary.

SIN$

TPF$

SP2$

SEC$.040  Data Acquisition - Block Diagram of System

RND$04

SP2$

CYB$

The HRIBF Data Acquisition System consists of three major components:

ODP$

(1)-A microprocessor controlled VME system with interfaces to

CAMAC, Fera and Fastbus data acquisition hardware.

ODP$

(2)-A Linux workstation connected to the VME system via a private

ethernet and connected to other facilities via a 100 megabit/sec LAN.

ODP$

(3)-A large RAID file server connected to the Linux workstation via

the 100 megabit/sec LAN.

RND$07

SP2$

CYB$

VME                         LINUX              RAID

SYSTEM                      WORKSTATION        FILE SERVER

-----------------           ---------------    ---------------

|Microprocessor |           |             |    |             |

|               |           | modu_setup**|    | General     |

CAMAC     |Interface      |           |             |    | File        |

<---------->|Modules        |           | pacor**     |    | Storage     |

|               |           |             |    |             |

FERA      |Acquisition    |           | pacman      |    | Source code |

<---------->|Readout,       |           |             |    |             |

|Control &      |           | logger      |    | Executables |

FASTBUS   |Communication  |  PRIVATE  |             |    |             |

<---------->|Software       |<--------->| pftoipc     |    | LDFs        |

|               |  ETHERNET |             |    |             |

OTHER     |               |           | tape        |    | Histograms  |

<---------->|               |           |             |    |             |

|               |           | scanor      |    |             |

|               |           |             |    |             |

SYS BUSY  |               |           | damm        |    |             |

<-----------|               |           |             |    |             |

|               |           | scad        |    |             |

EVENT     |               |           |             |    |             |

----------->|Trigger Module |           | scop        |    |             |

TRIGGER   |               |           |             |    |             |

|               |           |             |    |             |

-----------------           ---------------    ---------------

|                   |

|                   |

|  100 megabit LAN  |

<-----------<----------------->-------->

** Used in initialization phase only

TPF$

SP1$

SEC$.050  Data Acquisition - List of Software Modules

CY1$

---------------------------------------------------------------------------

\b\VME\\            Directly controls all front-end operations

\b\Microprocessor\\ Reads/writes CAMAC, Fera, Fastbus & other modules

\b\Code\\           Detects event trigger

Executes data readout as specified by pac program

Forms data packets & sends to host

Manages all communication with host workstation

---------------------------------------------------------------------------

\b\modu_setup\\     Provides for the setup of certain CAMAC modules

which require initialization.

\b\See ORPHAS Tab-9 or Type: viewdoc modu_setup\\

---------------------------------------------------------------------------

\b\pacor\\          Compiles readout specification program (pac)

Displays diagnostic information

Loads specification table into VME microprocessor

\b\See ORPHAS Tab-2 or Type: viewdoc pacor\\

---------------------------------------------------------------------------

\b\pacman\\         Spawns other processes including \b\logger & tape\\

Provides user control of the data acqisition operation

All control commands are entered into the \b\pacman\\ window

Creates iconized labeled windows for \b\damm & scanor\\ for your

convenience but you don't have to use them

\b\See ORPHAS Tab-4 or Type: viewdoc pacmanII\\

---------------------------------------------------------------------------

\b\logger\\         Displays/logs internally generated messages

This is an "output process" only

---------------------------------------------------------------------------

\b\pftoipc\\        Runs in the background - no window

Receives real time data buffers from VME system

---------------------------------------------------------------------------

\b\tape\\           Runs in the background - no window

Writes list data to List Data Files (\b\LDF\\s)

It did write tape (hence the name) - but no more!

---------------------------------------------------------------------------

\b\scanor\\         Receives real time data buffers. User customized to:

Generate 1-D & 2-D histograms in shared memory & on disk

\b\See ORPH UBOOK 2000 Tab-4 or Type: viewdoc scanor\\

---------------------------------------------------------------------------

\b\damm\\           General display, manipulation & fitting program

Accesses histograms in shared memory from \b\scanor\\

Accesses histograms on disk from anywhere

\b\See ORPH UBOOK 2000 Tab-2 or Type: viewdoc damm\\

---------------------------------------------------------------------------

\b\scad\\           Generates numeric display of up to 240 CAMAC scalers and

Generates graphic display of up to 8 CAMAC scalers

\b\See ORPHAS Tab-8 or Type: viewdoc scad\\

---------------------------------------------------------------------------

\b\scop\\           Generates graphic display of up to 8 CAMAC scalers and

sends rate-data to EPICS (the accelerator control system)

\b\See ORPHAS Tab-8 or Type: viewdoc scop\\

===========================================================================

If you do not have \b\/usr/hhirf/\\ defined in your path, then you must type:

\b\/usr/hhirf/viewdoc\\ instead of just \b\viewdoc\\

===========================================================================

TPF$

SP2$

SEC$.060  Data Acquisition - List of Things to be Done

SP1$

CY1$

o......Review the general setup requirements

\b\(see ORPHAS Tab-5 or type: viewdoc VMEsys)\\

CY1$

o......Review any special module setup requirements

\b\(see ORPHAS Tab-9 or type: viewdoc modu_setup)\\

CY1$

o......Review the general features of the data acwuisition manager \b\pacman\\

\b\(see ORPHAS tab-4 or type: viewdoc pacmanII)\\

CY1$

o......Write a data acquisitation specification program (\b\pac\\ program)

\b\(see ORPHAS Tab-2 or type: viewdoc pacor)\\

CY1$

o......Produce a customized histogramming program (Customized \b\scanor\\)

\b\(see UBOOK 2000 Tab-4 or type: viewdoc scanor)\\

CY1$

o......Create an initialization file for numeric display of CAMAC scalers

\b\(see ORPHAS Tab-8 or type: viewdoc scad)\\

CY1$

o......Create an initialization file for graphic display of CAMAC scalers

\b\(see ORPHAS Tab-8 or type: viewdoc scad & viewdoc scop)\\

CY1$

o......Review the features of general purpose display program \b\damm\\

\b\(see UBOOK 2000 Tab-2 or type: viewdoc damm)\\

SP1$

CYB$

***************************************************************************

*                              IMPORTANT!!                                *

*                           BEFORE YOU START!!                            *

*                       YOU SHOULD DO THE FOLLOWING                       *

***************************************************************************

CY1$

Add the following lines to your \b\.login\\ file:

\b\alias pacman /usr/acq/wks/pacman\\

\b\set ignoreeof\\

BLP$

In your login window (from which you will run \b\pacman\\), enter the following

command:

CY1$

\b\setenv VME vmex\\  ;where x=1,2,3... (i.e. define the vme system that you

;will be using) to be \b\vme1, vme2, vme3, ....

CYB$

***************************************************************************

See pacmanII, Sec-30 for other suggested changes to .login, .cshrc & .twmrc

***************************************************************************

TPF$

SP1$

SEC$.070  Data Acquisition - Brief Description of System Operation

BLP$

The following list of operations are intended to give the novice a concept

of how the

system works and is not intended to present a detailed description of

"everything you have to do".

All of the following operations are to be carried out on the data acquisition

workstation. First, the user creates (writes or modifies):

CY1$

A customized histogramming program based on \b\scanor\\ - call it \b\myscanor\\

CY1$

A data acquisition specification program  - call it \b\mydacq.pac\\

Cy1$

A CAMAC scaler display specification file - call it \b\myscals.sca\\

CY1$

Compile and test \b\mydacq.pac\\ by typing:=====================================

CYB$

pacor mydacq

CY1$

Compile and load \b\mydacq\\ into the VME microprocessor by typing:=============

CY1$

\b\setenv VME vmex\\    ;Specify the VME system (vme1, vme2, vme3...)

\b\pacor mydacq L\\

CY1$

Start \b\pacman\\ (acquisition control program) by typing:======================

CY1$

\b\setenv EXPT RIBxxx\\ ;Identify your experiment (for \b\DOE & OMB\\)

\b\setenv VME vmex\\    ;Specify the VME system (vme1, vme2, vme3...)

\b\pacman\\             ;This program manages the data acquisition process

;from the workstation side. It also creates windows

;starts 3 other processes \b\logger, pftoipc & tape\\

\b\startvme\\           ;Starts data acquisition (enables histogramming)

\b\stopvme\\            ;Stops  data acquisition

\b\trun bon\\           ;Starts recording to \b\LDF - beam ON\\

\b\trun boff\\          ;Starts recording to \b\LDF - beam OFF\\

\b\tstop\\              ;Stops  recording

CY1$

Start \b\myscanor\\ (customized histogramming program) by typing:===============

CY1$

\b\myscanor myhis\\     ;Histograms will be stored on \b\myhis.his\\

\b\acq VMEx\\           ;Assign input to be from \b\VMEx\\ (x=1,2,3..)

\b\go\\                 ;Start processing input list data

CY1$

Start \b\damm\\ (display & analysis program) by typing:=========================

CY1$

\b\damm\\               ;Starts program \b\damm\\

\b\fig n\\              ;Define windows for display (n = 1 to 20)

\b\in myhis.his\\       ;Opens histogram file for input - Actually uses the

;shared memory segment generated by \b\myscanor\\

\b\d id\\               ;Displays histogram number \b\id\\

TPF$

SP1$

SEC$.070  A Brief Description of System Operation (continued)

CY1$

Start \b\scad\\ (CAMAC scaler display program) by typing:=======================

CY1$

\b\setenv VME vmex\\              ;Specify reading from vme1, vme2, vme3...

\b\scad\\                         ;It prompts you for a file name

Enter filename->\b\myscals.sca\\  ;Enter filename

\b\run\\                          ;Start it going

===========================================================================

SP1$

ODP$

o......The VME system

responds to event triggers inputted to a special Trigger Module and

subsequently reads out data as prescribed by the pre-loaded acquisition

code (\b\pac\\).

ODP$

o......While the readout is proceeding a "VME system busy" is

asserted for external use.

ODP$

o......The data read is formed into ethernet packets

and sent to the host workstation.

ODP$

o......A process, \b\pftoipc\\  running on the workstation collects packets

and generates

standard data buffers which are made available to other processes

(such as \b\scanor & tape\\) running on the "data acquisition platform".

ODP$

o......Process \b\tape\\ writes data buffers to disk files (\b\LDF\\s).

ODP$

o......The user customized histogramming process (based on \b\scanor\\)

generates

a number of 1-D and 2-D histograms as specified by the customizing

routines.

ODP$

o......Histograms are generated in a shared menory segment identified

by the association with a histogram file (\b\.his\\ file) on disk.

ODP$

o......Program \b\damm\\ is used to access this memory segment and display

the histograms being generated by the (\b\scanor\\ based) process.

ODP$

o......Program \b\scad\\ is used for a tabular display of up to 240

CAMAC scalers at a

display interval of 1 second or greater (default is 5 sec).

\b\scad\\ may also be used to display a list of up to 8

CAMAC scalers graphically at a rate of up to 20 displays/sec.

Each scaler rate is displayed as a software

generated meter with either linear or log scale.

ODP$

o......Program \b\scop\\ is used to display up to 8 scalers graphically

(as for \b\scad\\ above) and to send the rate data to the EPICS accelerator

control system for operator monitoring.

CHP$U100  README - Data Processing and Analysis

TPF$

SP1$

SEC$.080  Data Processing, Analysis and Display

SP1$

CYB$

For information about:

ODP$

\b\List-data-Copying\\ - see \b\lemor\\ (ORPH Ubook 2000, Sec# U320) for

tape-to-tape, tape-to-file, file-to-tape copy procedures.

ODP$

\b\List-Data-Preprocessing\\ - see  \b\lemor\\ (ORPH Ubook 2000, Sec# U310) for

a description of available preprocessing facilities.

ODP$

\b\Histogramming\\ - see \b\scanor\\ (ORPH Ubook 2000, Sec# U320) for

list-data histogramming from tapes and files.

ODP$

\b\Histogram-Display\\ - see \b\damm\\ (ORPH Ubook 2000, Sec# U300)

for a description of on-line (and off-line) 1-D & 2-D histogram display.

ODP$

\b\Histogram-Analysis\\ - see \b\damm\\ (ORPH Ubook 2000, Sec# U300)

for a description of histogram analysis (peak fitting, summing, gateing, etc).

ODP$

\b\Byte-Swapping\\ - see \b\swapo\\ (ORPH Ubook, Sec# U540) for how to

byte-swap list-data files and histogram files.

CY1$

(documentation is also available via \bu\dodoc\\ and \bu\viewdoc\\)

CHP$U100  README - HRIBF Tape and File Structures

TPF$

SP1$

SEC$.090  HHIRF List Data Tape Structure

BLP$

There are currently three list data structures supported by the standard

HRIBF processing software (i.e. SCANOR, LEMOR and customized versions

thereof). These are:

CYB$

L001 - Old externally   generated data

L002 - Old Perkin/Elmer generated data

L003 - Current HRIBF    generated data

BLP$

These structures may be processed from either 8mm tape or special list

data files (\b\LDFs\\). See \B\LEMOR SEC#310\\ for how to copy tapes

to LDFs, LDFs to tape, etc.

CY1$

\bu\HHIRF List Data - L001 Tape Structure\\

CYB$

L001 formatted list-data (often generated in external labs) consists of:

ODP$

o......16-bit data words.

ODP$

o......Fixed length data records which are no shorter than 2048 bytes and

no longer than 32768 bytes.

ODP$

o......A fixed number of parameters per event.

ODP$

o......A fixed (integer) number of events per record (no event splitting

across record boundries).

ODP$

o......A number of data-record header-words may be specified which are

to be ignored during processing.

ODP$

o......Header records of 256 bytes in length and structured as defined

in SEC#014 may be utilized for display, searches, etc.

ODP$

o......All other records not of the specified data-record length will be

ignored during processing.

TPF$

SP1$

SEC$.090  HHIRF List Data Tape Structure (continued)

CY1$

\bu\HHIRF List Data - L002 Tape Structure\\

BLP$

The \b\L002 data stream\\ (from the old Concurrent system) consists of

Header records, Event Handler

Source Code (EVS) records, Data records and Scaler records - as

illustrated below:

CY1$

Header    record   ;256 bytes

EVS-file  record   ;1600 bytes

.

Data      record   ;8192 bytes

Data      record   ;8192 bytes

.          .

Scaler    record   ;6780 bytes

End-of-File        ;File mark

Header    record

Data      record

.          .

.          .

End-of-File        ;Double file-mark

End-of-File        ;ends all data on tape

BLP$

\b\L002 events\\ consist of parameter ID flags followed by one or more

data words corresponding to sequential parameter IDs  - as illustrated

below:

CY1$

8000-hex + ID     ;Parameter ID

Data              ;for Parameter ID

Data              ;for Parameter ID+1

.

8000-hex + JD     ;Parameter JD

Data              ;for Parameter JD

Data              ;for Parameter JD+1

.

FFFF-hex          ;End-of-event

CYB$

o......Events \b\may be split\\ across record boundries.

o......Data words cannot have the hi-order bit set.

o......All parameter IDs and data words are 16-bit.

CY1$

===========================================================================

\bu\Structure of the 256-byte header record\\

\bu\Header Word Number\\

16-BIT   32-BIT   # BYTES  CONTENTS                        TYPE

01 - 04  01 - 02         8  'HHIRF   '                      ASCII

05 - 08  03 - 04         8  'L002    '                      ASCII

09 - 16  05 - 08        16  'LIST DATA       '              ASCII

17 - 24  09 - 12        16  MO/DA/YR HR:MN                  ASCII

25 - 64  13 - 32        80  User Title                      ASCII

65 - 66       33         4  Header Number                   BINARY

67 - 70  34 - 35         8  Reserved (set to 0)             BINARY

71 - 72       36         4  # of Secondary Header Records   BINARY

37         4  Record Length (bytes)           BINARY

38         4  # Blocked Line Image Records    BINARY

39         4  Record Length (bytes)           BINARY

40         4  Parameters/Event (ref only)     BINARY

41         4  Data Record Length (bytes)      BINARY

42 - 64            Reserved (set to 0)             BINARY

TPF$

SP1$

SEC$.090  HRIBF List Data Tape Structure (continued)

CY1$

\bu\HHIRF List Data - L003 Tape Structure\\

CYB$

The L003 data stream (currently in use) may consist of:

CY1$

Header    records - 256   bytes

PAC-file  records - 1600  bytes

Scaler    records - 32000 bytes

Dead-time records - 128   bytes

Data      records - 2048 to 32768 bytes (but not 32000 bytes)

CYB$

A typical data stream might look like:

CY1$

Header    record

PAC-file  record

PAC-file  record

Data      record

Data      record

.          .

.          .

Scaler    record

Data      record

.          .

.          .

Scaler    record

Dead-time record

End-of-File

Header    record

Data      record

.          .

.          .

End-of-File

End-of-File

BLP$

\b\L003 events\\ consist of pairs of 16-bit numbers, the first of

which defines the parameter ID and the second the data - as shown

below.

CY1$

8000-hex + ID     ;Parameter ID

Data              ;Parameter data

8000-hex + ID

Data

8000-hex + ID

Data

.

.

FFFF-hex          ;End-of-event word-1

FFFF-hex          ;End-of-event word-2

ODP$

o......Events are \b\not\\ split across record boundries.

ODP$

o......Unfilled records are padded with FFFF-hex.

TPF$

SP1$

SEC$.100  HHIRF List Data File Structure

BLP$

The list data file (LDF) ia a structured disk file for containing

L001, L002 or L003 data. It is processed as a direct access file whose

record length is 8194 32-bit words.

CY1$

\bu\The General Record Structure is:\\

32-bit word  1       TYPE  - Record type (DIR,HEAD,PAC,SCAL,DEAD,DATA,EOF)

2       NFW   - number of 32-bit words of data

3-8194  DATA  - 8192 full-words (32768 bytes) data plus pads

\bu\Directory record structure\\

32-bit word  1       'DIR '  - type = Directory

2       8192    - full-word data size

3       8194    - full-word blocksize

4       NREC    - number of records written on file

5               - un-used

6       NHED    - number of header records written on file

7       HED-ID  - header ID number

8       RECN    - record number where header is written

9       HED-ID  - header ID number

10       RECN    - record number where header is written

.

\bu\Header record structure\\

32-bit word  1       'HEAD'  - type = header

2        64     - number of full-words of header

3-66            - header record - 256 bytes

\bu\PAC record structure\\

32-bit word  1       'PAC '  - type = PAC

2        NFW    - number of full-words of data

3-NFW+2         - PAC file records (up to 409 lines)

\bu\Scaler record structure\\

32-bit word  1       'SCAL'  - type = Scaler

2        NFW    - number of full-words of data

3-NFW+2         - Data for up to 400 scalers

\bu\Deadtime record structure\\

32-bit word  1       'DEAD'  - type = Deadtime

2        32     - 128 bytes of deadtime data

3-34            - Deadtime data

\bu\Data record structure\\

32-bit word  1       'DATA'  - type = Data

2        8192   - number of full-words of data

3-8194          - list data plus padding

\bu\EOF record structure\\

32-bit word  1       'EOF '  - type = EOF (End-of-File)

2        8192   - number of full-words of data

3-8194  'EOF '  - padded with EOFs

TPF$

SP1$

SEC$.110  HRIBF Histogram Tape and File Structures

BLP$

Program STU produces histogram tapes

for transport to other computer systems. See SEC# U410

for STU operating instructions.

If you have a VAX and HHIRF VAXPAK, STU is probably what you should

use. If you have a Unix or Linux system, then transfer should be

via \b\tar or ftp\\.

BLP$

If STU is requested to output a HIS-file to tape,

it writes an 80 byte ASCII Title record,

the associated DIR-file (or DRR-file) and the HIS-file

into one TAPE file. The Title record consists of the input filename

(words 1-19) and a byte-order flag BOF (word 20).

BOF = DEC says tape byte order is DEC compatible, BOF = IBM says

byte order is IBM compatible. The structure of such a tape file is

shown below:

CY1$

REC#  LENGTH(BYTES)  CONTENTS-----------------------------------------

1          80     FILNAM (76 bytes) & Byte-order-flag (4 bytes)

2         128     1st  DIR-file record

3         128     2nd  DIR-file record

-          -              -

-          -              -

M         128     M-1  DIR-file record

-          -              -

N         128     Last DIR-file record (N = NHIS+2 +(NHIS+31)/32)

N+1       16384     1ST  16KB of HIS-file

N+2       16384     2nd  16kb of HIS-file

-          -              -

N+I       16384     Ith  16KB of HIS-file

-          -              -

End-of-File .....................................

1          80     FILNAM (76 bytes) & Byte-order-flag (4 bytes)

2         128     1st  DIR-file record

3         128     2nd  DIR-file record

-          -              -

-          -              -

M         128     M-1  DIR-file record

-          -              -

N         128     Last DIR-file record (N = NHIS+2 +(NHIS+31)/32)

N+1       16384     1ST  16KB of HIS-file

N+2       16384     2nd  16kb of HIS-file

-          -              -

N+I       16384     Ith  16KB of HIS-file

-          -              -

End-of-File .....................................

End-of-File .....................................

CEN$

(see next page for DIR-file definitions)

TPF$

SP1$

SEC$.110  HHIRF Histogram Tape and File Structures (continued)

CY1$

STRUCTURE OF DIR-FILE - FIRST RECORD (128 BYTES) ---------------------

JDIRF(1-3)   - 'HHIRFDIR0001'

JDIRF(4)     - # of histograms on HIS-file

JDIRF(5)     - # of half-words on HIS-file

JDIRF(7-12)  - YR,MO,DA,HR,MN,SC (date, time of CHIL run)

JDIRF(13-32) - Text (entered in CHIL via $TEX command)

STRUCTURE OF DIR-FILE - DIRECTORY ENTRY (128 BYTES) ------------------

IDIRH(1)     - Histogram dimensionality (max = 4)

IDIRH(2)     - Number of half-words per channel (1 or 2)

IDIRH(3-6)   - Histogram parm#'s (up to 4 parameters)

IDIRH(7-10)  - Length of raw    parameters (pwr of 2)

IDIRH(11-14) - Length of scaled parameters (pwr of 2)

IDIRH(15-18) - Min channel# list

IDIRH(19-22) - Max channel# list

IDIRF(12)    - Disk offset in half-words (1st word# minus 1)

IDIRF(13-15) - X-parm label

IDIRF(16-18) - Y-parm label

XDIRF(19-22) - Calibration constants (up to 4 floating numbers)

IDIRF(23-32) - Sub-title (40 bytes) (entered via CHIL $TIT cmd)

STRUCTURE OF DIR-FILE - ID-LIST (32 ID'S/RECORD) ---------------------

IDLST(1)     - ID number of 1st histogram defined

IDLST(2)     - ID number of 2nd histogram defined

-

BLP$

Note: JDIRF, IDIRF & IDLST are 32-bit integers: IDIRH is a 16-bit

integer: XDIRF is REAL*4. There is one directory entry for each

histogram on the file.

CYB$

HHIRF Histogram Tapes - SPK-file Structure

BLP$

STU tape SPK-files consist of an 80 byte Title record (same as that

for HIS-files, above) followed by as many 2048 byte binary records

as required to contain the complete SPK-file (i.e. an image of the

disk file). A file-mark separates successive tape files. The last

tape file is followed by a double file-mark.

BLP$

If you don't have a VAX (and HHIRF VAXPAK), this record structure

will be a bitch to read - so I am not going to try to tell you how

to do it.

CHP$U100  README - HRIBF UPAK

TPF$

SP1$

SEC$.120  UPAK - Software Xport

BLP$

Most of the general purpose data acquisition, processing, analysis and

support software is available from the anonymous ftp site given below:

CYB$

ftp://ftp.phy.ornl.gov/pub/upak

BLP$

Click on \b\README\\ and you will see a list of the currently available

packages. The list at the time of this writing is shown below.

CYb$

mcsq-acq.tar.Z  :data acquisition libs & source, DEC/Compaq fortran, C

upak-osf.tar.Z  :Obsosete upak for DEC OSF workstations, DEC fortran

upakor-osf.tar.gz :Version for Compak TRU64, Compaq fortran

upak-sun.tar.gz  :Obsolete version of upak for SunOS 4.1.4, Sun f77

upak-ultrix-mips.tar.Z  :Version of upak for DECstations running Ultrix

upakor-linux-alpha.tar.gz :Linux running on Alpha CPU's, Compaq fortran

upak-macosx.tar.gz  :MacOSX, using Fink g77 and XFree86

\bu\In the Linux subdirectory (Linux/):\\

README-upak.txt: Information on installing the UPAK data analysis software

README-hribf-daq.txt: Information on installing the HRIBF acquisition

upakor-linux-pgf77-intel.tar.gz  :Linux running on Intel Pentium, pgf77

DvDController.tgz: A graphical interface for archiving data to DVD

hribf-daq-linux-src.tgz: Source code for the data acquisition system

hribf-daq.linux.tgz: Linux 9.0 runtime of the acquisition software

upak-linux-g77-src.tgz: Source of the UPAK software

upak-linux-g77-intel.tgz: Linux 9.0, Intel Pentium, g77 compiler

CY1$

This software is distributed to our nuclear physics colleagues, whereis, asis.

*This is the latest release, G77 will likely become our standard release.

Questions?

Email varner@phy.ornl.gov

---------------------------------------------------------------------

BLP$

To obtain the files from the anonymous ftp site, use the ftp command

on your local system,

CY1$

csh> ftp ftp.phy.ornl.gov

Username: anonymous

Password: (Enter your Email address)

ftp> cd pub/upak

ftp> bin

ftp> get upak-osf.tar.Z

(messages)

ftp> quit

csh> uncompress upak-osf.tar

BLP$

The tar file contains a directory, milner/, which is the source code for

all the upak software, plus a bit.  The directory hhirf/ contains the

binary code for the programs, as well as the documentation in the

hhirf/doc directory.

TPF$

SP1$

SEC$.130  UPAK - Comments

CY1$

To:       UPAK users

From:     W. T. Milner

Subject:  Some Things About UPAK (Alpha version - revised)

BLP$

Program \b\scanor\\ (which replaces scan and scanu) and program

\b\lemor\\ (which replaces lemo) have a number of new features

including the processing of \b\list-data-files\\. See the

documentation on \b\scanor and lemor\\ for details.

BLP$

UPAK contains two main directories \b\/usr/users/milner/ &

/usr/hhirf/\\.

The \b\/hhirf/\\ directory contains standard executables, template

make-files, and some run-time help-files plus a few other things.

The \b\/milner/\\ directory contains a few example spk- and his-files,

a .login file, a few example data files, this document, and not

much else.

BLP$

Each program or routine library is contained in separate sub-directory

of the \b\/milner/\\ directory. All sub-directory names consist of an

upper case D followed by the program name (\b\/milner/Ddamm/\\

contains \b\damm, /milner/Dscanor/\\ contains \b\scanor\\, etc).

There are no sub- sub- directories. Typically, such a program

sub-directory (\b\/milner/Dprog/\\ for example)

will contain some or all of the following:

ODP$

(1)....Source files specific to \b\prog\\ only.

ODP$

(2)....\b\prog.make\\ - a make file (shows all required files and

libraries).

ODP$

(3)....\b\prog.doc\\ - a source document file with formatting directives.

To learn how to process \b\doc-files\\, Read \b\/usr/hhirf/doc/txx.tex\\.

ODP$

(4)....\b\prog.tex\\ - a formatted document file (ready for screen

viewing or printing on any device). The \b\tex-files\\ may only be found

in directory \b\/usr/hhirf/doc/\\.

ODP$

(5)....\b\prog.txx\\ - a formatted document file (with escape sequences

to enable \b\bold\\ and \bu\undrelined\\ printout on an LN03 compatible

printer.

ODP$

(6)....\b\prog.hep\\ - a run-time interactive help file (not used for all

programs). These files are also contained in the \b\/usr/hhirf/\\

directory.

ODP$

(7)....Possibly some undocumented test programs, data files, etc.

BLP$

Libraries sources for \b\(lemorlib, scanorlib, miliba, milibb,

and \b\orphlib\\ are all contained in separate directories,

\b\Dlemorlib, .... Dorphlib\\.

CEN$

General Comments

BLP$

A simple way to make /UPAK/ executables available is to include the

following statement in your \b\.cshrc or .login\\ files.

CYB$

setenv PATH "${PATH}:/usr/hhirf"

The UPAK tape is in TAR format.

See next page for a discussion of how to implement run-time help.

TPF$

SP1$

SEC$.070  UPAK - Comments (continued)

BLP$

A number of programs utilize files to provide for run-time

help. Some examples are shown below:

CY1$

\bu\Program\\       \bu\Associated Help File\\

damm          damm.hep

scanor        scanor.hep

lemor         lemor.hep

BLP$

In the initialization phase of all programs which use run-time

help files, an attempt is first made to open \b\prog.hep\\ where,

\b\prog\\ denotes the program name. If that attempt fails, an

attempt is made to open \b\/usr/hhirf/prog.hep\\.

The path part, \b\/usr/hhirf/\\, is "built in" to routine

\b\helpopen\\ in library \b\orphlib.a\\ and the \b\prog.hep\\

part is supplied by the program itself.

If this attempt also fails, an attempt is made to open a file named

\b\upak_help_path\\ in the users main (login) directory. If this

file exists, it should contain (one line) the path to all files of

the type \b\prog.hep\\ (including beginning and ending slashes) as

shown in the default example below.

CYB$

/usr/hhirf/        ;Contents of the default upak_help_path

;This path name can contain 60 characters maximum

BLP$

If the attempt to open \b\upak_help_path\\ fails or if the attempt to

open the specified path and file fails, then run-time help will not

be available.

CEB$

Making Run-time Help Available on Your System

BLP$

There are several ways to do this and the one that you choose will

probably depend upon the resources available to you.

ODP$

(1)....If you create a directory \b\/usr/hhirf/\\ and place all files

of type \b\prog.hep\\ in this directory, then things should work without

modification.

ODP$

(2)....If you place all files of type \b\prog.hep\\ in some directory,

\b\/home/myname/UPAK/\\ for example, and create in your login directory

the file named, \b\upak_help_path\\, which contains the path name,

\b\/home/myname/UPAK/\\, then run-time help will be available.

ODP$

(3)....If you change the default path name in routine \b\helpopen\\

to correspond to the actual path to files of the type \b\prog.hep\\

and remake \b\orphlib.a\\ and then remake \b\asap, damm, lemor, etc.\\,

then each user will not be required th have the file

\b\upak_help_path\\ in his/her/it's login directory.

BLP$

Method-3 is quite a bit more trouble since you will probably need to

change all the make-files, etc. and would have to repeat the process

each time you receive a new version of \b\UPAK\\. I would suggest

either methods (1) or (2).

TPF$

SP1$

SEC$.070  UPAK - Comments (continued)

SP2$

CEB$

IMPORTANT NOTE

BLP$

In order for \b\scanor and lemor\\ to work on the Alpha, you must have

the "Real Time Kernal" installed - this is not automatic but is

an optional feature of the installation. This is used by the

asychronous I/O, etc.

CEB$

COMMENT

BLP$

You can make the "input focus" and "window stacking"

work better (in my opinion) by adding the following to

your .Xdefaults file:

CYB$

Mwm*keyboardFocusPolicy:  pointer

Mwm*focusAutoRaise: False

BLP$

This causes the input focus to follow the pointer (doesn't

require clicking) and windows are not restacked unless you

click on the banner.

QIT$