RPD$075066

CHP$rcon - RMS Control Program

TPF$

DBL$

CYB$

Sec Page Contents

010   2  How to Get Started - How to End

020   2  Introduction and General Features

030   3  Commands - General & Histogram Related

040   3  Commands - For Status Information

050   4  Commands - For Teslameter Parameter Check, Set, etc

060   4  Commands - For rmon Display Management

070   4  Commands - For Ion Specification

080   5  Commands - For Setting Operational Mode and Knobs

090   5  Commands - For Setting Magnets & Electrostatic Dipoles

100   6  Commands - Related to Electrostatic Dipoles Only

110   7  Commands - Related to Valves

120   7  Commands - For Varian Guages

130   7  Commands - For CAMAC operations

140   8  Commands - For Monitor options

150   9  Commands - To Disable/Enable read/write of Certain Devices

160   9  Commands - For Fingers

165   9  Commands - For snapshots of RMS parameters to printer & files

170  10  Discussion of Magnet Related Commands

180  10  Discussion of Finger Related Commands

190  11  Discussion of Program RMON Operations

200  13  Discussion of Magnet Set, Ramp and Cycle Operations

210  14  Discussion of Magnet Tweaking Operations

220  15  Miscellaneous Notes and Comments

230  16  Magnet Power Requirements

240  17  Hardware specification file       - rmshwd.dat

250  21  Teslameter Parameter Data File    - teslapar.dat

260  23  RMS Calibration Constants         - rmscal.dat

270  24  RMS Mode Table & Discussion       - rmsmode.dat

280  26  Histogram Table (ID vs Label)

290  27  What the Hell do I do Now

SIN$

TPF$

SEC$010  How to Get Started - How to End

ODP$

(1)....Log onto rms0 as user \b\rms\\.

ODP$

(2)....Open two windows: One for \b\rcon\\ which will receive

all typed commands and one for \b\rmon\\ which performs monitor,

control, logging, and continuous display functions.

ODP$

(3)....\b\rcon\\ and \b\rmon\\ should be run from the same directory,

Drcon, where the associated data files are located.

CYB$

Type:  rcon      ;in the rcon window to start rcon

Type:  rmon      ;in the rmon window to start rmon

Type:  h         ;in rcon window for a listing of commands

;All subsequent commands are typed in the \b\rcon\\ window

Type:  kill      ;in the \b\rcon\\ window to terminate \b\rmon\\

Type:  end       ;in the \b\rcon\\ window to terminate \b\rcon\\

CYB$

Abnormal termination of rmon can lead to problems (see SECT# 290)

SEC$020  Introduction and General Features

CEN$

\bu\Introduction\\

BLP$

The function of this package is to control and/or monitor the devices

associated with the RMS.

The program actually consists of two processes;

\b\rcon and rmon\\.

ODP$

(1)....\b\rcon\\ receives all typed input commands, processes some

itself (related to magnets, status, etc),

and passes those related to elecrtostatic dipoles to

\b\rmon\\ via shared memory.

ODP$

(2)....\b\rmon\\ receives input commands from \b\rcon\\ and

performs monitor, control, logging and histogramming functions.

ODP$

(3)....\b\rmon\\ generates repeative displays of selected \b\rms\\

parameters.

ODP$

(4)....Both \b\rcon & rmon\\ produce log files - rcon.log and

rmon.log.

CEN$

\bu\General Features\\

ODP$

(1)....The voltage of all electrostatic dipoles can be set and monitored.

ODP$

(2)....The conditioning currents of electrostatic dipoles can be controled

and monitored.

ODP$

(3)....The current may be set (and monitored) for all magnetic elements

and the associated magnetic field monitored.

ODP$

(4)....Electrostatic voltages, conditioning currents, magnet currents,

magnetic fields, magnet interlock status and certain vacuum gauge readings

are displayed repeatively.

ODP$

(5)....The state of all vacuum gauges can be displayed and logged.

ODP$

(6)....The state of all valves can be displayed and logged.

ODP$

(7)....A full set of Finger positioning commands are provided.

ODP$

(8)....A histogram record of all voltages, currents, magnetic fields and

vacuum gauge readings can be produced. (See SECT# 280)

TPF$

SP1$

SEC$030  Commands - General & Histogram Related

CYB$

H               ;Display on-line help

CLDF            ;Clears "rmon already running" flag which will

;be set if rmon is terminated abnormally!

LON             ;Turns output to rcon.log ON

LOF             ;Turns output to rcon.log OFF (default)

BLON            ;Turn bell associated with rmon messages ON  (default)

BLOF            ;Turn bell associated with rmon messages OFF

INIT            ;Enables histogramming & sets start Date/Time

ZERO            ;Zeros histogramms and does an INIT

HUP             ;Copy  histograms to disk

STOR            ;Store histograms on \b\rmslog-YR-MO-DA_HR:MN:SEC.spk\\

;(automatic after 16384 minutes)

KILL            ;Kill rmon (sets E1 & E2 control DACs to zero)

CMD  filename   ;Process commands from filename

END             ;End program \b\rcon\\

SP2$

SP2$

SEC$040  Commands - For Status Information

DBL$

CYB$

STAT  MAGS      ;Disp/logs state of Alphas, Tesla-meters, Interlocks

STAT  NAME      ;Disp/logs status of NAMEd magnet

STAT  ION       ;Disp/logs field requirements for specified ion

STAT  MODE      ;Disp/logs MODE parameters (basic GEOS and KNOB data)

STAT  CAL       ;Disp/logs Magnet & Electric-Dipole calibration constants

STAT  ED        ;Disp/logs Electric Dipole status information

STAT  OPTS      ;Disp/logs rmon options - display, logging, tweaking, etc

STAT  VACS      ;Disp/logs all vacuum gauge readings

STAT  VALV      ;Disp/logs all valve positions, set-points, etc

STAT  HWD       ;Disp/logs hardware configuration (from rmshwd.dat)

STAT  FING      ;Disp/logs finger status information

SIN$

TPF$

SP1$

SEC$050  Commands - For Teslameter Paramater Check, Set, etc

CYB$

TES   MAG CMD <DAT> ;General Tesla command to magnet, MAG (Q1, D1, etc)

TSLA  CHEC  MAG     ;Checks Tesla parms vs  teslapar.dat for magnet MAG

TSLA  CHEC  ALL     ;Checks Tesla parms vs  teslapar.dat for all magnets

TSLA  SAVE  MAG     ;Saves Tesla parms on   teslapar.dat for magnet MAG

TSLA  SAVE  ALL     ;Saves Tesla parms on   teslapar.dat for all magnets

TSLA  LOAD  MAG     ;Loads Tesla parms from teslapar.dat for magnet MAG

TSLA  LOAD  ALL     ;Loads Tesla parms from teslapar.dat for all magnets

TSLA  NUFI          ;Creates a new teslapar.dat & saves all Tesla parms

------------------------------------------------------------------------

MAG = D1, D2, D3, Q1, Q2, Q3, Q4, Q5, Q6, Q7, S1, S2

------------------------------------------------------------------------

SP2$

SEC$060  Commands - For rmon Display Management

CYB$

DON   MAGS  ISEC  ;Disp magnet   info every ISEC (min,max,dflt = 10,60,10)

DON   EDS   ISEC  ;Disp E-dipole info every ISEC (min,max,dflt = 2, 60,10)

DON   VACS  ISEC  ;Disp vacuum   info every ISEC (min,max,dflt = 10,60,10)

DON   ALL   ISEC  ;Disp MAGS,EDS,VACS every ISEC (min,max,dflt = 10,60,10)

DOF   MAGS        ;Turn OFF magnet    display

DOF   EDS         ;Turn OFF E-dipole  display

DOF   VACS        ;Turn OFF vacuum    display

DOF   ALL         ;Turn OFF all       display

SSI               ;Set Screen to Initial state (i.e. clear rmon window)

SP2$

SEC$070  Commands - For Ion Specification

CYB$

ION   NAME            ;Specify ion to be transported

MEV   MEV             ;Specify energy in MeV

Q     Q               ;Specify charge state

AION  AMU             ;Specify ion mass in amu

AOQ   X               ;Specify AMU/Q & compute Q using previous AMU

BEAM  NAME            ;Specify beam ion    (for Q-trams information only)

BMEV  MEV             ;Specify beam energy (for Q-trans information only)

TPF$

SP1$

SEC$080  Commands - For Setting Operational Mode and Knobs

CYB$

MODE  INIT            ;Reads & process file rmsmode.dat

;This is done automatically at rcon start-up but you

;may edit the file and re-process it.

MODE  modefile        ;Reads and processes file modefile which replaces

;data contained in rmsmode.dat

;(See SECT# 270 for discussion)

KNOB  ID  FAC         ;For KNOB ID set Scale-factor FAC

;(See SECT# 270 for disciussion)

KNOB  INIT            ;Sets all KNOB Scale-factors to zero (default)

SP1$

SEC$090  Commands - For Setting Magnets & Electrostatic Dipoles

CYB$

ALP  MAG CMD <DAT>    ;General Alpha command to magnet MAG (Q1, D1, etc)

TES  MAG CMD <DAT>    ;General Tesla command to magnet MAG (Q1, D1, etc)

TURN  NAME  ON        ;Turn ON  named element

TURN  NAME  OFF       ;Turn OFF named element

SETI  E1              ;Set  E1 to voltage required   for AMU, MeV, Q, X

SETI  E2              ;Set  E2 to voltage required   for AMU, MeV, Q, X

RAMI  E1              ;Ramp E1 to voltage required   for AMU, MeV, Q, X

RAMI  E2              ;Ramp E2 to voltage required   for AMU, MeV, Q, X

SETI  MAGS            ;Set   all   magnetic elements for AMU, MeV, Q, X

RAMI  MAGS            ;Ramp  all   magnetic elements for AMU, MeV, Q, X

CYCI  MAGS            ;Cycle all   magnetic elements for AMU, MeV, Q, X

CYCZ  MAGS            ;Cycle all   magnetic elements to  zero

SETI  NAME            ;Set   named magnetic element  for AMU, MeV, Q, X

RAMI  NAME            ;Ramp  named magnetic element  for AMU, MeV, Q, X

CYCI  NAME            ;Cycle named magnetic element  for AMU, MeV, Q, X

SETM  NAME  A  AMPS   ;Set   named magnetic element to A amps

SETM  NAME  G  GAUSS  ;Set   named magnetic element to G Gauss

RAMM  NAME  A  AMPS   ;Ramp  named magnetic element to A amps

RAMM  NAME  G  GAUSS  ;Ramp  named magnetic element to G Gauss

CYCM  NAME  A  AMPS   ;Cycle named magnetic element to A amps

CYCM  NAME  G  GAUSS  ;Cycle named magnetic element to G Gauss

AJI   NAME            ;Adjust named magnet  for AMU, MeV, Q, via tweaking

AJI   MAGS            ;Adjust all   magnets for AMU, MeV, Q, via tweaking

AJM   NAME  G         ;Adjust named magnet  for G-gauss      via tweaking

;Note1: Tweaking will be enabled for adjusted magnet

;Note2: Ramps first, if change is more than 15%

---------------------------------------------------------------------------

NAMEs are: Q1, Q2, Q3, Q4, Q5, Q6, Q7, S1, S2, D1, D2, D3, E1, E2

TPF$

SP2$

SEC$100  Commands - Related to Electrostatic Dipoles Only

CYB$

ECON               ;Turns electric-dipole control/monitor ON

ECOF               ;Turns electric-dipole control/monitor OFF (default)

MMON  Ex           ;Turns current-mode monitor  ON (default)

MMOF  Ex           ;Turns current-mode monitor OFF

;(no ramp-down in Current-mode)

DWEL  Ex  SEC      ;Dwell-time in sec (for MMOF operation) default = 20

SET+  Ex  KV       ;Set  positive supply    to   +KV

SET-  Ex  KV       ;Set  negative supply    to   -KV

SET   Ex  KV       ;Set  both     supplies (to +- KV)

SET   Ex  KV+ KV-  ;Set  POS supply to KV+ and NEG supply to KV-

;(Max allowed for set is 100.0 KV)

ZAP   Ex           ;Set POS and NEG supply to zero

RAMI  Ex           ;Ramp Ex to voltage required   for AMU, MeV, Q, X

RAM+  Ex  KV       ;Ramp positive supply   to KV kilovolts

RAM-  Ex  KV       ;Ramp negative supply   to KV kilovolts

RAM   Ex  KV       ;Ramp both     supplies to KV kilovolts

RAM   Ex  KV+ KV-  ;Ramp POS supply to KV+ and NEG supply to KV-

KVPM  Ex  DKV      ;Set increasing ramp-rate to DKV KV/min (default = 2.0)

KVPS  Ex  DKV      ;Set decreasing ramp-rate to DKV KV/sec (default = 2.0)

MXPR  Ex  TORR     ;Set max vessel pressure (units of torr for operation

MXCC  Ex  UA       ;Set max Conditioning Current (microamps) for operation

CCL   Ex  UA       ;Set +&- supply condit-current limit to UA (hwd limited)

CCL+  Ex  UA       ;Set    +supply condit-current limit to UA (hwd limited)

CCL-  Ex  UA       ;Set    -supply condit-current limit to UA (hwd limited)

CYB$

---------------------------------------------------------------------------

Note:  Ex  denotes electric dipole  E1  or  E2

---------------------------------------------------------------------------

SP1$

CYB$

Parameter Values - Defaults and Limits

------------------------------------------------

Parameter  Max-value   Min-value   Default-value

DWEL             500           0              20

KVPM            60.0        0.10            2.00

KVPS            20.0        0.10            2.00

MXCC            50.0        0.50            8.00

CCL+            50.0        0.50           10.00

CCL-            50.0        0.50           10.00

MXPR          3.0E-6           -          2.0E-6

------------------------------------------------

TPF$

SP1$

SEC$110  Commands - Related to Valves

CYB$

VSPP  VSPNAM  TORR      ;Sets set-point-pressure for valve-setpoint-name

;VSPNAM to pressure = TORR

VSPG  VSPNAM  VG        ;Sets set-point-gauge    for valve-setpoint-name

;VSPNAM to gauge VG

;VSPNAM = V1U, V1D .. VP3U, VP3D, etc

OPEN  VLVN              ;Opens  valve VLVN (only applies to V1 and V6)

CLOSE VLVN              ;Closes valve VLVN (only applies to V1 and V6)

SP1$

SEC$120  Commands - For Varian Guages

CYB$

VG  STAT  C          ;Display Controler status

VG  PRES  C G        ;Display Pressure

VG  EMIS  C G F      ;Read/set Emission - F = ON, ON2, OFF, STAT, READ

VG  DEGA  C G F      ;Degas             - F = ON, OFF, STAT

VG  SENS  C G F      ;Sensitivity       - F = READ

VG  GASC  C G F      ;Gas Correction    - F = READ

VG  SETP  C G F V P  ;Set-point  (not implemented)

VG  AUTO  C G F T P  ;Auto       (not implemented)

VG  HARD  C          ;Display hardware configuration

VG  TORR  C          ;Sets pressure units to torr

VG  SLOW  C          ;Sets TC update rate to SLOW

VG  FAST  C          ;Sets TC update rate to FAST

VG  LOCK  C          ;Locks           local keyboard

VG  UNLO  C          ;Unlocks         local keyboard

VG  PART  C          ;Partial lock of local keyboard

VG  DEFA  C          ;Resets Controler & sets default paremeters

CYB$

---------------------------------------------------------------------------

Where: C=Controler

G=Gauge

F=Function

P=Pressure(torr)

V=Value and

T=Thermocouple#

---------------------------------------------------------------------------

DADC                 ;Read & display ADCs

DREG                 ;Read & display REG

SP3$

SEC$130  Commands - For CAMAC operations

SP1$

CYB$

CAM  CNA    C N A        ;Specify C, N, A of CAMAC module to be written

;or read, subsequently

CAM  WRIT   DATA         ;Write DATA to previously defined CAMAC module

CAM  READ                ;Read/display 1 data word from defined module

CAM  CNAF C N A F <DATA> ;Execute general CAMAC CNAF

TPF$

SP1$

SEC$140  Commands - For Monitor options

CYB$

LGON  MAGS  ISEC  ;Log magnet   info every ISEC (min,max,dflt = 60,any,300)

LGON  EDS   ISEC  ;Log E-dipole info every ISEC (min,max,dflt = 60,any,300)

LGON  VACS  ISEC  ;Log vacuum   info every ISEC (min,max,dflt = 60,any,300)

LGON  ALL   ISEC  ;Log MAGS,EDS,VACS every ISEC (min,max,dflt = 60,any,300)

LGOF  MAGS        ;Turn OFF magnet    logging

LGOF  EDS         ;Turn OFF E-dipole  logging

LGOF  VACS        ;Turn OFF vacuum    logging

LGOF  ALL         ;Turn OFF all       logging

DON   MAGS  ISEC  ;Disp magnet   info every ISEC (min,max,dflt = 10,60,10)

DON   EDS   ISEC  ;Disp E-dipole info every ISEC (min,max,dflt = 2, 60,10)

DON   VACS  ISEC  ;Disp vacuum   info every ISEC (min,max,dflt = 10,60,10)

DON   ALL   ISEC  ;Disp MAGS,EDS,VACS every ISEC (min,max,dflt = 10,60,10)

DOF   MAGS        ;Turn OFF magnet    display

DOF   EDS         ;Turn OFF E-dipole  display

DOF   VACS        ;Turn OFF vacuum    display

DOF   ALL         ;Turn OFF all       display

HON   MAGS        ;Histogram magnet   info at 60 sec intervals

HON   EDS         ;Histogram E-dipole info at 60 sec intervals

HON   VACS        ;Histogram vacuum   info at 60 sec intervals

HON   ALL         ;Histogram MAGS, EDS, VACS

HOF   MAGS        ;Turn OFF magnet    histogramming

HOF   EDS         ;Turn OFF E-dipole  histogramming

HOF   VACS        ;Turn OFF vacuum    histogramming

HOF   ALL         ;Turn OFF all       histogramming

ECON              ;Turn ON  E-dipole monitor/control via rmon

ECOF              ;Turn OFF E-dipole monitor/control via rmon

MCON              ;Turn ON  magnet   monitor/control via rmon

MCOF              ;Turn OFF magnet   monitor/control via rmon

MTON  NAME        ;Turn ON  tweaking of NAMEd magnet  via rmon

MTON  ALL         ;Turn ON  tweaking of ALL   magnets via rmon

MTOF  NAME        ;Turn OFF tweaking of NAMEd magnet  via rmon

MTOF  ALL         ;Turn OFF tweaking of ALL   magnets via rmon (dflt)

TPF$

SP1$

SEC$150  Commands - To Disable/Enable Read/Write of Certain Devices

CYB$

RWON  MAGS  ALL   ;Enables  read/write of all Alpha supplies (dflt)

RWOF  MAGS  ALL   ;Disables read/write of all Alpha supplies

RWON  MAG   NAME  ;Enables  read/write of Alpha supply for NAMEd magnet

RWOF  MAG   NAME  ;Disables read/write of Alpha supply for NAMEd magnet

RWON  TESL  ALL   ;Enables  read/write of all tesla-meters (dflt)

RWOF  TESL  ALL   ;Disables read/write of all tesla-meters

RWON  TESL  NAME  ;Enables  read/write of tesla-meter for NAMEd magnet

RWOF  TESL  NAME  ;Disables read/write of tesla-meter for NAMEd magnet

RWON  VACS  ALL   ;Enables  read/write of all   vacuum-gauges (dflt)

RWOF  VACS  ALL   ;Disables read/write of all   vacuum-gauges

RWON  VAC   NAME  ;Enables  read/write of NAMEd vacuum-gauge

RWOF  VAC   NAME  ;Disables read/write of NAMEd vacuum-gauge

---------------------------------------------------------------------------

Magnet names are: Q1, Q2, Q3, Q4, Q5, Q6, Q7, S1, S2, D1, D2, D3, E1, E2

---------------------------------------------------------------------------

Vacuum gauge names are: VG11, VG12, ... VG51, ..., etc

SP2$

SEC$160  Commands - For Fingers

CYB$

FING  INIT               ;Initialize & calibrate all fingers

FING  PARK  FN           ;Park & calibrate finger-FN

FING  SET   FN XP <UNIT> ;Set   finger-FN to location XP UNITS

FING  SHIF  FN DX <UNIT> ;Shift finger-FN by          DX UNITS (+ or -)

;UNIT = CM, IN or STEPS (default = CM)

FING  OFF                ;Turn OFF power to all motors

STAT  FING               ;Displays/Logs finger status information

SP2$

SEC$165  Commands - for snapshots of RMS paramerers to printer & files

CYB$

NAMP  PRNAME             ;Specify Printer name such as cp6000, ps01, etc

NAMP                     ;Says use default printer (this is the default)

SNAP                     ;Send SNAPSHOT to default (or specified) printer

;and to a file named rms-yr-mo-da-Hr:mn:sc.snap

TPF$

SP1$

SEC$170  Discussion of Magnet Related Commands

ODP$

\b\ALP..MAG..CMD..<DAT>\\ directs to magnet, MAG, any Alpha

command, CMD, with associated data, DAT. Where, MAG denotes Q1, D1, etc.

The command reply is displayed and logged (if LON).

ODP$

\b\TES..MAG..CMD..<DAT>\\ directs to magnet, MAG, any Tesla

command, CMD, with associated data, DAT. Where, MAG denotes Q1, D1, etc.

The command reply is displayed and logged (if LON).

SP3$

SEC$180  Discussion of Finger Related Commands

ODP$

\b\FING..INIT\\ initializes and calibrates all fingers. The fingers

are moved (one at a time) to the outer limit and subsequently moved

away from the outer linit by a fixed amount (111 steps including 11

steps of backlash) to a "calibrated" position of 100 steps or 1/64

inches.

ODP$

\b\FING..PARK..FN\\ initializes and calibrates one finger \b\FN\\ as

described above in \b\FING..INIT\\.

ODP$

\b\FING..SET..FN..XP..UNITS\\ sets finger no. \b\FN\\ to the absolute

location specified by \b\XP. UNITS\\ denotes IN, CM, or STEPS.

If \b\UNITS\\ is not specified, CM is taken as the default.

CY1$

\b\FING..SHIF..FN..DX..UNITS\\ shifts finger no. \b\FN\\ by DX units.

If DX is positive, the motion is toward the inner limit.

If DX is negative, the motion is toward the outer limit.

ODP$

\b\FING..OFF\\ turns OFF the power to all finger motors.

ODP$

\b\STAT..FING\\ displays and logs (if LON) the positions and status of

all fingers.

CEN$

\bu\Comments on Finger Movements, Limits etc.\\

ODP$

(1)....The finger travel (outer limit to inner limit) is about 9.5 inches.

ODP$

(2)....The finger in driven by a 32 turn per inch drive screw.

ODP$

(3)....The drive screw in driven by a 200 step per turn stepping motor.

ODP$

(4)....Thus the total travel is about 9.5x32x200 = 60800 steps.

ODP$

(5)....There is one limit switch for each finger at the outer limit.

ODP$

(6)....There is only a single common limit switch at the inner limit.

ODP$

(7)....If either limit is encountered, the finger will be automatically

backed away 400 steps.

ODP$

(8)....The outer limit is arbitrarily defined to be at step# 64000.

ODP$

(9)....The park position is at step# 63600 (400 steps from outer limit).

TPF$

SP1$

SEC$190  Program RMON Operations

CYB$

Operations Performed in Main RMON Loop ------------------------------------

Check for command every --------------------------- 200 ms

Do Elec-Dipole monitor/control every --------------   2 sec

Do Magnet   monitor/control every -----------------   2 sec

Do Magnet   tweaking        every -----------------  60 sec if ON

Do screen display of ED, Mag, Vac, etc every ------  10 sec or as specified

Do logging        of ED, Mag, Vac, etc every ------  60 sec or as specified

Do histogramming  of ED, Mag, Vac, etc every ------  60 sec if ON

BLP$

For Electric Dipoles, there are two modes of operations: The \b\MMON\\ mode

(the default)

and the \b\MMOF\\ mode. In the MMON-mode (the mode-monitor mode), the

software does not allow the power supplies to run in the current-mode while

in the MMOF-mode the object is to permit current-mode operation much of the

time. These two modes are described below - but first some definitions:

CY1$

\b\VDEST..\\is the ultimate voltage to be achieved (ramped to).

\b\VTARG..\\is present target voltage of ramp (can be less than \b\VDEST\\).

\b\VSET...\\is the present voltage specified by the DAC setting.

CYB$

Electric-Dipole operations for both MMON and MMOF modes -------------------

ODP$

(1)....Check for power supply OFF. If OFF, set VDEST, VTARG, VSET to 0.

Power supply will have to be reset manually in RMS room.

ODP$

(2)....Check for bad vacuum status. If bad, set VTARG and VSET to 0.

When vacuum recovers, ramp-up will start from 0.

CYB$

Electric-Dipole operations for MMON-mode only -----------------------------

ODP$

(3)....Check for current control mode. If yes, set VTARG=0 and

start to ramp down. Ramp up will start once voltage mode is

restored.

ODP$

(4)....Check conditioning current. If software limit MXCC is exceeded,

set VTARG to 0 and

start ramp down. Ramp up will start once conditioning current is less than

or equal to MXCC.

ODP$

(5)....If all is normal, set VTARG=VDEST and call RAMPER to set VSET, etc.

ODP$

(6)....Compute HV-supply current limit from minimum of (VSET & VRED)

and the hardware conditioning current limit (CCL) specified.

Write the current control

and voltage control DACs.

CYB$

Electric-Dipole operations for MMOF-mode only -----------------------------

ODP$

(3)....No check on control mode.

ODP$

(4)....Check conditioning current. If software limit MXCC is exceeded,

set VTARG to 0 and

start ramp down. Ramp up will start once conditioning current is less than

or equal to MXCC.

ODP$

(5)....If DWEL = 0 or voltage mode has presisted for DWEL

seconds, start ramp to VTARG.

ODP$

(6)....Compute HV-supply current limit from minimum of (VSET & VRED)

and the hardware conditioning current limit (CCL) specified.

Write the current control

and voltage control DACs.

CEB$

(continued on next page)

TPF$

SP1$

SEC$190  Program RMON Operations (continued)

SP1$

CYB$

Magnet monitor and control operations -------------------------------------

BLP$

If \b\rmon\\ is running and magnet control is enabled

(i.e. \b\MCON\\), the following magnet related functions

are performed.

ODP$

(1)....All magnet \b\set, ramp, and cycle\\ commands are processed.

ODP$

(2)....The magnet power supply currents are managed (via software)

so as to limit the sum of primary currents to no more than 350 AC Amps.

ODP$

(3)....If magnet tweak is enabled (i.e. \b\MTON\\), an attempt

will be made to adjust the magnet current such that the field

measured by the Teslameter is that requested.

TPF$

SP1$

SEC$200  Discussion of Magnet Set, Ramp and Cycle Operations

CYB$

SETTING AMPS (or GAUSS)                                    \b\Display\\

CY1$

(1)....Checks for ON, enabled, legal amps, etc.

CY1$

(2)....If OK, set requested AMPS.                            \b\Set\\

CYB$

RAMPING to AMPS (or GAUSS)

CY1$

(1)....Checks for ON, enabled, legal amps, etc.

CY1$

(2)....Ramps to AMPS at 5 amps/sec                           \b\RAMP\\

CY1$

(3)....Waits 300 seconds.                                    \b\WAIT\\

CY1$

(6)....Displays Set.                                         \b\Set\\

CYB$

CYCLING to AMPS (or GAUSS)

CY1$

(1)....Checks for ON, enabled, legal amps, etc.

CY1$

(2)....Sets current to zero.

CY1$

(3)....Waits 60 seconds.                                     \b\WAIT\\

CY1$

(4)....Ramps to MAXamps at 5 amps/sec.                       \b\RAMP\\

CY1$

(5)....Waits 120 seconds.                                    \b\WAIT\\

CY1$

(6)....Ramps to zero at 5 amps/sec.                          \b\RAMP\\

CY1$

(7)....Waits 120 seconds                                     \b\WAIT\\

CY1$

(8)....Ramps to AMPS at 5 amps/sec.                          \b\RAMP\\

CY1$

(9)....Waits 300 seconds                                     \b\WAIT\\

CY1$

(10)...Displays Set.                                         \b\Set\\

CYB$

CYCLING to Zero

CY1$

(1)....Checks for ON, enabled, legal amps, etc.

CY1$

(2)....Sets current to zero.

CY1$

(3)....Waits 60 seconds.                                     \b\WAIT\\

CY1$

(4)....Ramps to MAXamps at 5 amps/sec.                       \b\RAMP\\

CY1$

(5)....Waits 120 seconds.                                    \b\WAIT\\

CY1$

(6)....Ramps to zero at 5 amps/sec.                          \b\RAMP\\

CY1$

(7)....Waits 300 seconds                                     \b\WAIT\\

CY1$

(8)....Displays Set.                                         \b\Set\\

TPF$

SP1$

SEC$210  Discussion of Magnet Tweak Operations

BLP$

The objective of the magnet tweak operation is to adjust the

magnet current such that the field read \b\GRED\\ is as close as

possible to the field requested \b\GSET\\.

Tweaking can take place under the following conditions:

ODP$

(1)....Tweaking must be enabled via the \b\MTON\\ command.

ODP$

(2)....Magnet must be in a \b\Set, TWEK, or REDY\\ state.

ODP$

(3)....For the \b\Set\\ state, tweaking will be initiated if

\b\GRED & GSET\\ are different.

ODP$

(4)....For the \b\TWEK\\ state, tweaking will continue until the

values \b\GRED & GSET\\ agree or until the maximum number of attempts

(currently 50) has been made.

ODP$

(5)....In the \b\TWEK\\ state, adjustments will be made at 10 sec intervals.

ODP$

(6)....When \b\GRED & GSET\\ agree or the maximum number of tries have

been exhausted, the magnet state will be set to \b\REDY\\.

ODP$

(7)....In the \b\REDY\\ state (with \b\MTON\\), the need for re-tweaking

will be checked at approximately 120 second intervals.

CYB$

Define:  GDIF = absolute value of (GRED-GSET)

ODP$

(8)....Re-tweaking of dipoles is initiated if \b\GDIF\\ is greater than 0.2.

ODP$

(9)....Re-tweaking of quadrupoles and sextapoles is initiated if

\b\GDIF\\ is greater than 2.0.

BLP$

There are two different tweaking methods which are used depending on

the value of \b\GDIF\\. The table below summarizes the conditions for each.

CYB$

GDIF-range    CFAC      Amp-step        Method    Magnet-Type

----------    ----      --------        ------    ------------------

GT    200     0.25         -              1       All

50  - 200     0.50         -              1       All

LT    50      0.75         -              1       All

0.4 - 0.8      -         0.006            2       Dipoles

0.2 - 0.4      -         0.002            2       Dipoles

4.0 - 8.0      -         0.060            2       Quads & Sextapoles

2.0 - 4.0      -         0.020            2       Quads & Sextapoles

---------------------------------------------------------------------------

BLP$

\b\For Method-1\\, the current change required to make \b\GRED = GSET\\ is

estimated. The current change actually made, however, is reduced by the factor

\b\CFAC\\. That is, an under-correction is made which depends on the

magnitude of \b\GDIF\\.

BLP$

\b\For Method-2\\, the current is changed in small fixed-size steps of

magnitude

\b\Amp-step\\ as shown in the table above. Note that the size of the current

step depends on the magnitude of \b\GDIF\\ and the Magnet-Type.

TPF$

SP2$

SEC$220  Miscellaneous Notes and Comments

DBL$

CYB$

For Divider Current - 50 Uamps at 300 kV

For Current Monitor - 10 Volts = 250 Uamps

For Current Control - 10 Volts = 250 Uamps

For Voltage Monitor - 10 Volts = 275 kV

For Voltage Control - 10 Volts = 275 kV

Divider Resistance = 6,000 Megohms

SIN$

ODP$

(1)....Voltage control is either local (via front panel) or remote

but not both at the same time.

ODP$

(2)....Current control is either local (via front panel) or remote

but not both at the same time.

BLP$

Initially, we will limit current by front panel and control voltage

via computer. This gives some hardware protection against software

failures.

Conditioning Current

BLP$

The conditioning current is defined as the current read from the current

monitor which is in excess of that which is calculated to flow "internally"

(in the resistor string) at a given voltage setting.

TPF$

SP1$

SEC$230  Magnet Power Requirements

CYB$

The following table was compiled from the Alpha test report.

Maximum Output       Maximum Input              AC-Amps

----------------     -----------------          -------

Magnet    Ohms  DC-Volts  DC-Amps    AC-Volts  AC-Amps   %Eff   DC-Amps

D1      0.1574        85      540         472     76.0   74.0    0.1410

D2      0.1574        85      540         466     75.0   76.0    0.1390

D3      0.1574        85      540         470     75.0   75.3    0.1390

Q1      0.0643        42      653         481     57.0   58.0    0.0873

Q2      0.0673        31      460         473     28.3   61.4    0.0615

Q3      0.0660        36      540         479     38.0   62.0    0.0704

Q4      0.0636        21      330         480     17.0   49.1    0.0515

Q5      0.0630        21      330         487     17.0   48.2    0.0515

Q6      0.0636        21      330         475     16.8   50.0    0.0509

Q7      0.0673        31      460         474     28.8   60.1    0.0626

S1      0.0851        40      470         474     33.5   68.6    0.0713

S2      0.0851        40      470         474     33.7   68.1    0.0717

-----

496.1

-----------------------------------------------------------------------

Ohms     = Magnet resistance

DC-Volts = Alpha supply output voltage

DC-Amps  = Alpha supply output current (maximum)

AC-Volts = Alpha supply input  voltage

AC-Amps  = Alpha supply input  current

%Eff     = 100 x (DC-Volts x DC-Amps) / (AC-Volts x AC-Amps x SQRT(3))

In general: Alpha AC-Amps = (AC-Amps/DC-Amps) x DC-Amps(required)

and: Total AC-Amps = Sum of Alpha AC-Amps

and: Total AC-Amps must be less than 400 (or maybe 350)

TPF$

SEC$240  Hardware specification file - rmshwd.dat

CYB$

E1    CTRL    3       ;Controller   no.

E1    TC      4       ;TC no.

E1    IG      5       ;Ion-gauge    no.

E1    VSE+    1       ;DAC chan#  for setting voltage of       (+ supply)

E1    VSE-    2       ;DAC chan#  for setting voltage of       (- supply)

E1    CSE+    7       ;DAC chan#  for setting current limit of (+ supply)

E1    CSE-    8       ;DAC chan#  for setting current limit of (- supply)

E1    VRD+    3       ;ADC channel no. for reading voltage of  (+ supply)

E1    CRD+    4       ;ADC channel no. for reading current of  (+ supply)

E1    VRD-    5       ;ADC channel no. for reading voltage of  (- supply)

E1    CRD-    6       ;ADC channel no. for reading current of  (- supply)

E1    VMO+    1       ;Terminal no. for voltage-mode indicator (+ supply)

E1    CMO+    2       ;Terminal no. for current-mode indicator (+ supply)

E1    VMO-    3       ;Terminal no. for voltage mode indicator (- supply)

E1    CMO-    4       ;Terminal no. for current-mode indicator (- supply)

E1    VOF+    9       ;Terminal no. for voltage-OFF  indicator (+ supply)

E1    VON+    10      ;Terminal no. for voltage-ON   indicator (+ supply)

E1    VOF-    11      ;Terminal no. for voltage-OFF  indicator (- supply)

E1    VON-    12      ;Terminal no. for voltage-ON   indicator (- supply)

E1    KVPM    2.0     ;Ramp-up   rate in kV/min

E1    KVPS    2.0     ;Ramp-down rate in kV/sec

E1    MXCC    8.0     ;Max condit current in microamps (software limit)

E1    CCL    10.0     ;Max condit current in microamps (hardware limit)

E1    MXPR    2.0E-6  ;Max pressure (torr) for conditioning

E2    CTRL    4       ;Controller   no.

E2    TC      1       ;TC no.

E2    IG      6       ;Ion-gauge    no.

E2    VSE+    3       ;DAC chan#  for setting voltage of       (+ supply)

E2    VSE-    4       ;DAC chan#  for setting voltage of       (- supply)

E2    CSE+    9       ;DAC chan#  for setting current limit of (+ supply)

E2    CSE-    10      ;DAC chan#  for setting current limit of (- supply)

E2    VRD+    7       ;ADC channel no. for reading voltage of  (+ supply)

E2    CRD+    9       ;ADC channel no. for reading current of  (+ supply)

E2    VRD-    8       ;ADC channel no. for reading voltage of  (- supply)

E2    CRD-    10      ;ADC channel no. for reading current of  (- supply)

E2    VMO+    5       ;Terminal no. for voltage-mode indicator (+ supply)

E2    CMO+    6       ;Terminal no. for current-mode indicator (+ supply)

E2    VMO-    7       ;Terminal no. for voltage mode indicator (- supply)

E2    CMO-    8       ;Terminal no. for current-mode indicator (- supply)

E2    VOF+    13      ;Terminal no. for voltage-OFF  indicator (+ supply)

E2    VON+    14      ;Terminal no. for voltage-ON   indicator (+ supply)

E2    VOF-    15      ;Terminal no. for voltage-OFF  indicator (- supply)

E2    VON-    16      ;Terminal no. for voltage-ON   indicator (- supply)

E2    KVPM    2.0     ;Ramp-up   rate in kV/min

E2    KVPS    2.0     ;Ramp-down rate in kV/sec

E2    MXCC    8.0     ;Max condit current in microamps (software limit)

E2    CCL    10.0     ;Max condit current in microamps (hardware limit)

E2    MXPR    2.0E-6  ;Max pressure (torr) for conditioning

TPF$

SP1$

SEC$240  Hardware specification file - rmshwd.dat (continued)

CYB$

Q1    ACN     8       ;Alpha Controller number

Q1    TCN    11       ;Tesla Controller number

Q1    MXC   650       ;Maximum Current (amps)

Q2    ACN     4       ;Alpha Controller number

Q2    TCN    10       ;Tesla Controller number

Q2    MXC   450       ;Maximum Current (amps)

D1    ACN    10       ;Alpha Controller number

D1    TCN    09       ;Tesla Controller number

D1    MXC   500       ;Maximum Current (amps)

S1    ACN     6       ;Alpha Controller number

S1    TCN    08       ;Tesla Controller number

S1    MXC   450       ;Maximum Current (amps)

Q3    ACN     9       ;Alpha Controller number

Q3    TCN    07       ;Tesla Controller number

Q3    MXC   500       ;Maximum Current (amps)

S2    ACN     7       ;Alpha Controller number

S2    TCN    06       ;Tesla Controller number

S2    MXC   450       ;Maximum Current (amps)

D2    ACN    12       ;Alpha Controller number

D2    TCN    05       ;Tesla Controller number

D2    MXC   500       ;Maximum Current (amps)

Q4    ACN     2       ;Alpha Controller number

Q4    TCN    04       ;Tesla Controller number

Q4    MXC   330       ;Maximum Current (amps)

Q5    ACN     3       ;Alpha Controller number

Q5    TCN    03       ;Tesla Controller number

Q5    MXC   330       ;Maximum Current (amps)

D3    ACN    11       ;Alpha Controller number

D3    TCN    02       ;Tesla Controller number

D3    MXC   500       ;Maximum Current (amps)

Q6    ACN     1       ;Alpha Controller number

Q6    TCN    01       ;Tesla Controller number

Q6    MXC   330       ;Maximum Current (amps)

Q7    ACN     5       ;Alpha Controller number

Q7    TCN    00       ;Tesla Controller number (normally 00)

Q7    MXC   450       ;Maximum Current (amps)

TPF$

SP1$

SEC$240  Hardware specification file - rmshwd.dat (continued)

CYB$

VG11   1      1       ;VAC Ctrl-1, Gauge-1, TC

VG12   1      2       ;VAC Ctrl-1, Gauge-2, TC - Q3 Cryo-1

VG13   1      3       ;VAC Ctrl-1, Gauge-3, TC

VG14   1      4       ;VAC Ctrl-1, Gauge-4, TC

VG15   1      5       ;VAC Ctrl-1, Gauge-5, IG - Target Chamber

VG16   1      6       ;VAC Ctrl-1, Gauge-6, IG - Q3

VG21   2      1       ;VAC Ctrl-2, Gauge-1, TC

VG22   2      2       ;VAC Ctrl-2, Gauge-2, TC

VG23   2      3       ;VAC Ctrl-2, Gauge-3, TC - Q3 Cryo-2

VG24   2      4       ;VAC Ctrl-2, Gauge-4, TC - Drift Cryo

VG25   2      5       ;VAC Ctrl-2, Gauge-5, IG - Drift

VG26   2      6       ;VAC Ctrl-2, Gauge-6, IG - Undefined

VG31   3      1       ;VAC Ctrl-3  Gauge-1, TC

VG32   3      2       ;VAC Ctrl-3, Gauge-2, TC

VG33   3      3       ;VAC Ctrl-3, Gauge-3, TC - ED1 Cryo-1

VG34   3      4       ;VAC Ctrl-3, Gauge-4, TC - ED1 Cryo-2

VG35   3      5       ;VAC Ctrl-3, Gauge-5, IG - Achromat

VG36   3      6       ;VAC Ctrl-3, Gauge-6, IG - ED1

VG41   4      1       ;VAC Ctrl-4, Gauge-1, TC - Need to Change

VG42   4      2       ;VAC Ctrl-4, Gauge-2, TC - Need to Change

VG43   4      3       ;VAC Ctrl-4, Gauge-3, TC - ED2 Cryo-1

VG44   4      4       ;VAC Ctrl-4, Gauge-4, TC - ED2 Cryo-2

VG45   4      5       ;VAC Ctrl-4, Gauge-5, TC - Q6

VG46   4      6       ;VAC Ctrl-4, Gauge-6, TC - ED2

VG51   5      1       ;VAC Ctrl-5, Gauge-1, TC - Unknown

VG52   5      2       ;VAC Ctrl-5, Gauge-2, TC - Unknown

VG53   5      3       ;VAC Ctrl-5, Gauge-3, IG - PSAT

VG54   5      4       ;VAC Ctrl-5, Gauge-4, IG - Unknown

TPF$

SP1$

SEC$240  Hardware specification file - rmshwd.dat (continued)

CYB$

V1u   1  6  3  ;Valve, Ctrl, Gauge, Set-point#

V1d   1  5  2  ;

V2u   2  5  1  ;

V2d   1  6  6  ;

V3u   3  5  1  ;

V3d   2  5  3  ;

V4u   3  5  2  ;

V4d   3  6  3  ;

V5u   3  6  6  ;

V5d   4  6  1  ;

V6u   4  2  4  ;

V6d   4  1  6  ;

VP1u  1  2  7  ;

VP1d  1  6  4  ;

VP2u  2  3  4  ;

VP2d  1  6  5  ;

VP3u  2  4  5  ;

VP3d  2  5  2  ;

VP4u  3  3  7  ;

VP4d  3  6  4  ;

VP5u  3  4  8  ;

VP5d  3  6  5  ;

VP6u  4  3  7  ;

VP6d  4  6  2  ;

VP7u  4  4  8  ;

VP7d  4  6  3  ;

TPF$

SEC$250  Teslameter Parameter Data File - teslapar.dat

CYB$

Q1  IN    N            ;Display Mode

Q1  IG    C            ;General Function

Q1  IR    2            ;Field Range

Q1  IZ    -22.0        ;Calibration Zero

Q1  ID    0            ;Digital Filtering

Q1  IY    20.0         ;Filter Window

Q1  IJ    8.00000E+00  ;Filter Factor

Q1  IK    0.0          ;Sampling Interval

Q2  IN    N            ;Display Mode

Q2  IG    C            ;General Function

Q2  IR    2            ;Field Range

Q2  IZ    -15.0        ;Calibration Zero

Q2  ID    0            ;Digital Filtering

Q2  IY    20.0         ;Filter Window

Q2  IJ    8.00000E+00  ;Filter Factor

Q2  IK    0.0          ;Sampling Interval

D1  IN    N            ;Display Mode

D1  IG    DC           ;General Function

D1  IR    2            ;Field Range

D1  IC    1.00000E+00  ;Calibration Factor

D1  IO    0.0          ;Calibration Offset

D1  IZ    34.4         ;Calibration Zero

D1  IL    1.00000      ;Scale Factor

D1  ID    0            ;Digital Filtering

D1  IY    10.0         ;Filter Window

D1  IJ    4.10000E+01  ;Filter Factor

D1  IK    0.0          ;Sampling Interval

S1  IN    N            ;Display Mode

S1  IG    C            ;General Function

S1  IR    2            ;Field Range

S1  IZ    -8.0         ;Calibration Zero

S1  ID    0            ;Digital Filtering

S1  IY    20.0         ;Filter Window

S1  IJ    8.00000E+00  ;Filter Factor

S1  IK    0.0          ;Sampling Interval

Q3  IN    N            ;Display Mode

Q3  IG    C            ;General Function

Q3  IR    2            ;Field Range

Q3  IZ    -32.0        ;Calibration Zero

Q3  ID    0            ;Digital Filtering

Q3  IY    20.0         ;Filter Window

Q3  IJ    8.00000E+00  ;Filter Factor

Q3  IK    0.0          ;Sampling Interval

S2  IN    N            ;Display Mode

S2  IG    C            ;General Function

S2  IR    2            ;Field Range

S2  IZ    -11.0        ;Calibration Zero

S2  ID    0            ;Digital Filtering

S2  IY    20.0         ;Filter Window

S2  IJ    8.00000E+00  ;Filter Factor

S2  IK    0.0          ;Sampling Interval

TPF$

SEC$250  Teslameter Parameter Data File - teslapar.dat (continued)

CYB$

D2  IN    N            ;Display Mode

D2  IG    DC           ;General Function

D2  IR    2            ;Field Range

D2  IC    1.00000E+00  ;Calibration Factor

D2  IO    0.0          ;Calibration Offset

D2  IZ    30.9         ;Calibration Zero

D2  IL    1.00000      ;Scale Factor

D2  ID    0            ;Digital Filtering

D2  IY    10.0         ;Filter Window

D2  IJ    4.10000E+01  ;Filter Factor

D2  IK    0.0          ;Sampling Interval

Q4  IN    N            ;Display Mode

Q4  IG    C            ;General Function

Q4  IR    2            ;Field Range

Q4  IZ    -10.0        ;Calibration Zero

Q4  ID    0            ;Digital Filtering

Q4  IY    20.0         ;Filter Window

Q4  IJ    8.00000E+00  ;Filter Factor

Q4  IK    0.0          ;Sampling Interval

Q5  IN    N            ;Display Mode

Q5  IG    C            ;General Function

Q5  IR    2            ;Field Range

Q5  IZ    -26.0        ;Calibration Zero

Q5  ID    0            ;Digital Filtering

Q5  IY    20.0         ;Filter Window

Q5  IJ    8.00000E+00  ;Filter Factor

Q5  IK    0.0          ;Sampling Interval

D3  IN    N            ;Display Mode

D3  IG    DC           ;General Function

D3  IR    2            ;Field Range

D3  IC    1.00000E+00  ;Calibration Factor

D3  IO    0.0          ;Calibration Offset

D3  IZ    -28.1        ;Calibration Zero

D3  IL    1.00000      ;Scale Factor

D3  ID    0            ;Digital Filtering

D3  IY    10.0         ;Filter Window

D3  IJ    4.10000E+01  ;Filter Factor

D3  IK    0.0          ;Sampling Interval

Q6  IN    N            ;Display Mode

Q6  IG    C            ;General Function

Q6  IR    2            ;Field Range

Q6  IZ    -7.0         ;Calibration Zero

Q6  ID    0            ;Digital Filtering

Q6  IY    20.0         ;Filter Window

Q6  IJ    8.00000E+00  ;Filter Factor

Q6  IK    0.0          ;Sampling Interval

Q7  IN    N            ;Display Mode

Q7  IG    C            ;General Function

Q7  IR    2            ;Field Range

Q7  IZ    -13.0        ;Calibration Zero

Q7  ID    0            ;Digital Filtering

Q7  IY    20.0         ;Filter Window

Q7  IJ    8.00000E+00  ;Filter Factor

Q7  IK    0.0          ;Sampling Interval

TPF$

SP1$

SEC$260  RMS Calibration Constants - rmscal.dat

BLP$

The electric and magnetic fields required for a specified ion,

energy and q-value are computed by scaling values from a

standard (Amu=100, Mev=100, q=10)

\b\GEOS\\ calculation. These "required fields" may be

modified via the specification of a set of calibration constants

which are used as follows.

CYB$

FIELD(required) = A + (B * TESFAC * FIELD(geos))

BLP$

The following table lists a template \b\rmscal.dat\\ file for the RMS

calibration constants:

CYB$

---------------------------------------------------------------------------

Device       A         B    TESFAC

Q1       89.29  0.901176    1.3173

Q2      -25.21  0.946534    1.2869

D1        0.0   0.982652    1.0000

S1        0.0   1.0         1.3931

Q3        0.0   1.0         1.2744

S2        0.0   1.0         1.4125

D2        0.0   0.984721    1.0000

Q4       -4.32  0.968259    1.2820

Q5       -0.05  0.979575    1.2183

E1        0.03  1.017021    1.0000

D3       -0.10  0.999816    1.0000

E2        0.09  1.002130    1.0000

Q6       -0.68  0.626168    1.2823

Q7        3.78  0.567794    1.2914

CYB$

---------------------------------------------------------------------------

Note:  A is in units of kV or gauss  - depending on the device

B is a multiplication factor

BLP$

\b\TESFAC\\ (the correction factor due to Teslameter positioning) may be

set to 1.0 (or left blank) if B provides the full correction factor required.

---------------------------------------------------------------------------

CEB$

Comments

ODP$

(1)....Initially, all constants are set to the "no correction" values

(i.e. A=0.0, B=1.0, TESFAC=1.0).

ODP$

(2)....An attempt is made to open \b\rmscal.dat\\ in the directory from

which \b\rcon\\ is being run. If the file is not found, the default

constant values are retained.

ODP$

(3)....If \b\rmscal.dat\\ is found, it will be read and any constants

which it contains will replace the default values.

ODP$

(4)....If any syntax errors are encountered while processing \b\rmscal.dat\\,

error messages will be displayed & logged and all default values will

be retained.

TPF$

SEC$270  RMS Mode Table & Discussion - rmsmode.dat

BLP$

The \b\MODE\\ command provides a means of requesting a specific optics

configuration (with associated Knob definitions) at run time. The standard

(default) configuration contained in \b\rmsmode.dat\\ is listed below.

CYB$

Entry  Device  Value

GEOS   Q1     -0.6825

GEOS   Q2      0.3315

GEOS   D1      1.0302118

GEOS   S1      0.4251

GEOS   Q3      0.3607

GEOS   S2      0.3763

GEOS   D2      1.0302118

GEOS   Q4      0.2274

GEOS   Q5     -0.2858

GEOS   E1      166.57731

GEOS   D3      1.0302118

GEOS   E2      166.57731

GEOS   Q6      0.0944

GEOS   Q7     -0.3101

KNOB1  Q4      0.0042

KNOB1  Q5     -0.0022

KNOB2  Q1     -0.0165

KNOB2  Q2      0.0017

KNOB3  S1      0.0500

KNOB3  S2      0.0492

KNOB4  S1     -0.0500

KNOB4  S2      0.0499

KNOB5  Q1     -0.0052

KNOB5  Q2      0.0008

KNOB6  Q4     -0.0005

KNOB6  Q5      0.0003

KNOB6  Q6      0.0175

KNOB6  Q7     -0.0100

KNOB7  Q4     -0.0006

KNOB7  Q5      0.0003

KNOB7  Q6      0.0150

KNOB7  Q7     -0.0253

KNOB8  Q4      0.0037

KNOB8  Q5     -0.0120

KNOB8  Q6     -0.0170

KNOB8  Q7      0.0289

KNOB9  Q1     -0.0016

KNOB9  Q2     -0.0036

KNOB9  S1     -0.0296

KNOB9  S2      0.0296

KNOB9  Q4      0.0038

KNOB9  Q5     -0.0020

CEB$

(continued on next page)

TPF$

SP2$

SEC$270  RMS Mode Table & Discussion - rmsmode.dat (continued)

SP1$

ODP$

(1)....Entries labeled \b\GEOS\\ give the value predicted by \b\GEOS\\

for the device indicated and for Amu=100, MeV=100 and q=10.

ODP$

(2)....Entries labeled \b\KNOBx\\ give the Knob value for the device

indicated where, \b\x \\denotes the Knob-ID.

BLP$

The \b\KNOB\\ command (see SECT# 080) provides a means of specifying

small coupled

changes to be made in two or more optical elements (magnetic or

electrostatic) simultaneously.

\b\KNOBs\\ are only defined for Ion-related commands such as

\b\SETI, RAMI & AJI\\.

BLP$

In the file listed above, 9 Knobs (IDs 1 thru 9) have been specified.

The numbers in the \b\Value\\ column are pole-tip-field

corrections to be added to the basic field value (i.e. for

Amu=100, MeV=100, Q=10). The "corrected" field value is calculated as

follows.

CYB$

I=N

NEWFIELD(J) = BASFIELD(J) + SUM  S(I,J)*K(I,J)  ;Sum over all Knobs, where:

I=1

CY1$

NEWFIELD(J) = New (corrected) field value       (for device-J)

BASFIELD(J) = Basic field value                 (for device-J)

S(I,J)      = Scale constant from KNOB command  (for device-J & Knob-I)

K(I,J)      = Knob-Value from rmsknobs.dat      (for device-J & Knob-I)

SP1$

CEB$

Mode File Processing

BLP$

When a \b\MODE modefile\\ command is issued, the following sequence of

operations take place:

ODP$

(1)....The \b\Geos/Knob\\ data set is set to the internal default values

shown in the table above.

ODP$

(2)....An attempt is made to open \b\rmsmode.dat\\. If that file is

opened and processed without error, its values will replace the internal

defaults.

ODP$

(3)....Finally, an attempt will be made to open and process \b\modefile\\.

If that is successful, any entries contained in \b\modefile\\ will replace

those previously defined.

CY1$

(4)....Finally, one of the following messages will be displayed:

Mode set to:  Default Values  or

Mode set to:  rmsmode.dat     or

Mode set to:  modefile

BLP$

Note: It is not required that \b\modefile\\ contain the complete Geos/Knob

data set. It only needs to contain those entries that are different from

\b\rmsmode.dat\\. If does contain the full data set, that is OK.

TPF$

SP1$

SEC$280  Histogram Table (ID vs Label)

DBL$

CYB$

ID   Label        ID   Label          ID   Label        ID   Label

----------------   ---------------     ---------------  --------------

1   E1_VDEST+   101   E2_VDEST+     151   P_targ*e9

2   E1_VSET+    102   E2_VSET+      152   P_Q3*e9

3   E1_VRED+    103   E2_VRED+      153   P_drift*e9

4   E1_CRED+    104   E2_CRED+      154   Undefined

5   E1_CCUR+    105   E2-CCUR+      155   P_achro*e9

6   E1_CCAV+    106   E2_CCAV+      156   P_ED1*e9

157   P_Q6*e9

11   E1_VDEST-   111   E2_VDEST-     158   P_ED2*e9

12   E1_VSET-    112   E2_VSET-      159   P_psat*e9

13   E1_VRED-    113   E2_VRED-      160   Undefined

14   E1_CRED-    114   E2_CRED-

15   E1_CCUR-    115   E2_CCUR-

16   E1_CCAV-    116   E2_CCAV-

21   E1_torr*e9  121   E2_torr*e9

201   D1_Aset     301   D1_Ared       401   D1_Gset     501   D1_Gred

202   D2_Aset     302   D2_Ared       402   D2_Gset     502   D2_Gred

203   D3_Aset     303   D3_Ared       403   D3_Gset     503   D3_Gred

204   Q1_Aset     304   Q1_Ared       404   Q1_Gset     504   Q1_Gred

205   Q2_Aset     305   Q2_Ared       405   Q2_Gset     505   Q2_Gred

206   Q3_Aset     306   Q3_Ared       406   Q3_Gset     506   Q3_Gred

207   Q4_Aset     307   Q4_Ared       407   Q4_Gset     507   Q4_Gred

208   Q5_Aset     308   Q5_Ared       408   Q5_Gset     508   Q5_Gred

209   Q6_Aset     309   Q6_Ared       409   Q6_Gset     509   Q6_Gred

210   Q7_Aset     310   Q7_Ared       410   Q7_Gset     510   Q7_Gred

211   S1_Aset     311   S1_Ared       411   S1_Gset     511   S1_Gred

212   S2_Aset     312   S2_Ared       412   S2_Gset     512   S2_Gred

SIN$

TPF$

SP1$

SEC$290  What the Hell do I do Now

CY1$

\bu\Problem:  Unable to Communicate with VME\\

BLP$

If \b\rcon & rmon\\ get errors when an attempting to

communicate with the \b\VME\\ system, you will probably need to

boot the vme system. To do this, end both \b\rcon & rmon\\

and type: \b\/usr/hhirf/vme/bootvme\\.

CY1$

\bu\Problem:  Can't start rmon - aborts saying, "RMON already running"\\

BLP$

\b\rmon\\ sets a flag in a shared memory segment when it is started

so that two \b\rmon\\s can't be run at the same time. This flag is

cleared when \b\rmon\\ is terminated normally by a \b\kill\\ command

typed in the rcon window or a \b\ctrl/c\\ typed in the rmon

window. Any other type of termination will leave this flag set. This

flag can be cleared by typing (in the \b\rcon\\ window) the command:

\b\cldf\\.

BLP$

Clearly, you don't want to do this unless you know what you are doing,

otherwise, you can end up with multiple \b\rmon\\s running and a lot

of confusion.

CY1$

\bu\Problem:  damm doesn't access shared memory histograms for rmslog.his\\

BLP$

If \b\damm\\ has rmslog.his open when \b\rmon\\ is killed and

re-started, it will be looking at the wrong shared memory segment.

To cure this, open rmslog.his again by typing: \b\in rmslog.his\\

CY1$

\bu\Problem:  To examine/remove shared memory segments\\

Type: \b\ipcs\\         ;To display existing memory segments & message queues

Type: \b\ipcrm -m ID\\  ;To remove memory segment \b\ID\\

Type: \b\ipcrm -q ID\\  ;To remove message queue  \b\ID\\

BLP$

Where: \b\ID\\ denotes the the number in the ID-column displayed via \b\ipcs\\

as shown in the example below:

CY1$

Message Queues:

T     ID     KEY        MODE       OWNER    GROUP

q      0   50400021 --rw-rw----   milner    users

Shared Memory

T     ID     KEY        MODE       OWNER    GROUP

m      0          0 D-rw-------     root   system

m      1          0 D-rw-------     root   system

m      2          0 D-rw-------     root   system

m   1903   50400020 --rw-rw----   milner    users

m    104          0 --rw-rw-rw-   milner    users

CY1$

\bu\Problem:  Corrupted or lost drr-file\\

BLP$

\b\rmon\\ requires that a proper \b\rmslog.drr\\ exist. If this file has been

corrupted or lost, it can be recreated by typing: \b\/usr/hhirf/chil rmslog\\

This, of course, assumes that \b\rmslog.chl\\ exists for chil to operate on.

If the chl-file is lost, a copy can be gotten from \b\/usr/hhirf/rmslog.chl\\

TPF$

DBL$

CYB$

DATA         GFAC/1.3173,  ! Q1

&                  1.2869,  ! Q2

&                  1.0000,  ! D1

&                  1.3931,  ! S1

&                  1.2744,  ! Q3

&                  1.4125,  ! S2

&                  1.0000,  ! D2

&                  1.2820,  ! Q4

&                  1.2183,  ! Q5

&                  1.0000,  ! D3

&                  1.2823,  ! Q6

&                  1.2914,  ! Q7

&                  3*0.0/

SIN$

QIT$