RPD$075066

CHP$TWIG  A Target Wiggle Control Program - WTM

TPF$

SP2$

CYB$

Sec Page Contents

010   2  General Description

020   3  List of Commands

030   4  Example TWIG Dialog

SP2$

CEN$

\bu\HOW TO GET STARTED\\

CYB$

===========================================================================

Log onto rmsacq2

Type: setenv VME vme1

Type: twig

===========================================================================

TPF$

SP2$

SEC$.010  General Description

BLP$

\b\TWIG\\ is intended to control the "stepping motor driven" target wiggle

device designed and built by Jim Johnson. Several functions or features

are provided, namely:

ODP$

\b\Speed-control\\ via the specification of \b\WPS\\ or wiggles per second.

ODP$

\b\Direction\\ of rotation via the specification of \b\CW or CCW\\, i.e.

Clockwise or Counter-Clockwise rotation.

ODP$

\b\Status-information\\ via the \b\STAT\\ command which displays/logs the

current settings and hardware status.

BLP$

There are generally two modes of action: The \b\GODO & RUN\\ modes.

The \b\GODO\\ mode is for testing only while the \b\RUN\\ mode is for actual

operation.

BLP$

The \b\GODO\\ command allows you to specify a number of steps to do and the

speed (steps/second) with which to do it. I used this to try to determine

if the motor and vacuum feedthrough device under test was loosing steps.

I determined that it was indeed! This means that it is not possible to

predict where the motor will stop if the specified number of steps are

completed. Now, I have been told that if the motor stops

at one of two special orientations that it may not be possible to restart it.

Therefore, one must try to keep it running even if the beam is off

BLP$

The \b\RUN\\ command sets \b\TWIG\\ in a mode in which it keeps the motor

running by repeatedly testing the number of steps done and when the

requested number of steps is 90% completed, it re-issues the move command.

Unless you specify a shorter run time via the \b\RUNT\\ command, it will

set the maximum number of steps that the controller can support, i.e. about

16,000,000 steps. This is about 444 minutes at 3 wiggles per second or

600 steps per second. If you issue a \b\CTRL/C\\ command, the "test & restart"

loop will be interrupted but before it returns to you it will issue a command

to do the maximum number of steps so that you have plenty of time to start

the \b\RUN\\ again before it stops. So, for example, it would be possible

do a \b\CTRL/C\\ to interrupt the "test & restart" loop and specify a new

"wiggles per second" and issue the \b\RUN\\ command again.

BON$

BLP$

Again, since it is not possible to predict where the motor will stop, the idea

is to keep it running until the experiment is over or the target breaks!

BOF$

TPF$

SP2$

SEC$.020  List of Commands

CYB$

Commands Related to Setup =================================================

MOTN N        ;Specifies Motor number (N = 1 to 7)

;(default is N = 4)

CW            ;Specifies that rotation will be CLOCKWISE (default)

CCW           ;Specifies that rotation will be COUNTER-CLOCKWISE

WPS  NUM      ;Specifies NUM wiggles/sec   (0.1 to 3.0 allowed)

;(Default is NUM = 3.0)

Commands Related to Testing ===============================================

GODO NS STPS  ;Says Go do NS steps at STPS steps/second

STOP          ;Stops all the wigglin

TOFF          ;Turns off motor power

STAT          ;Displays current specified parameters & status information

Commands Related to Operation =============================================

MOTN N        ;Specifies Motor number (N = 1 to 7)

;(default is N = 4)

RUNT MIN      ;Specifies run-time in Minutes (0.0 to 1000.0 allowed)

;(If 0.0 is specified, max-value of run-time will be used)

;(RUNT.GT.0.0 is mainly for testing)

;Default is RUNT 0.0, i.e. uses maximum run-time

STAT          ;Displays current specified parameters & status information

RUN           ;Says "start wigglin" or "keep on wigglin"

;It goes into a test-and-restart loop

;It checks "wiggle time" remaining and if it is within 10%

;of stopping, it restarts with run-time specfied by RUNT.

CTRL/C        ;Interrupts wait-and-test loop to allow kewboard input

;Before it returns to you it sets MAXIMUM run-time

STOP          ;Stops all the wigglin

TOFF          ;Turns off motor power

END           ;Does a STOP, TOFF and Ends program TWIG

TPF$

SP1$

SEC$.030  Example TWIG Dialog

CY1$

rmsacq2> \b\setenv VME vme1                               !Must set vme1\\

rmsacq2> \b\twig                                          !Start TWIG\\

Type: h      - for list of HELP code-words & subjects  \b\!Usual Startup\\

Type: h all  - for a more detailed help directory      \b\!HELP\\

Type: h code - for command list for associated subject \b\!Messages\\

TWIG->\b\stat all                                         !Display status\\

---------------------------------------------------------------------------

Selected motor is number          15                   \b\!This is what\\

The current position is            0                   \b\!you get\\

Nobody is at inner limit                               \b\!when the\\

Selected motor is at outer limit                       \b\!stepping motor\\

Selected motor is stopped                              \b\!power\\

Selected motor not ramping                             \b\!has been\\

Down-counter is zero                                   \b\!turned\\

Direction is clockwise                                 \b\!OFF\\

Selected motor stopped normally                        \b\!...\\

---------------------------------------------------------------------------

Mot#  TIME-SET  TIME-DID   STEPS-SET   STEPS-DID   WIGGLES/  STEPS/   STATE

----   minutes   minutes   ---------   ---------     second  second   -----

4      0.00      0.00           0           0        3.0     600    STOP

TWIG->\b\runt 5                                           !Set 5 min loop time\\

TWIG->\b\wps 1                                            !Set 1 wiggle/sec\\

TWIG->\b\run                                              !Run for test only\\

Mot#  TIME-SET  TIME-DID   STEPS-SET   STEPS-DID   WIGGLES/  STEPS/   STATE

----   minutes   minutes   ---------   ---------     second  second   -----

4      5.00      0.25       60000        2999        1.0     200    RUN

4      5.00      0.50       60000        5999        1.0     200    RUN

4      5.00      0.75       60000        8999        1.0     200    RUN

4      5.00      1.00       60000       11999        1.0     200    RUN

4      5.00      1.25       60000       14999        1.0     200    RUN

4      5.00      1.50       60000       17999        1.0     200    RUN

4      5.00      1.75       60000       20999        1.0     200    RUN

4      5.00      2.00       60000       23999        1.0     200    RUN

4      5.00      2.25       60000       26999        1.0     200    RUN

4      5.00      2.50       60000       29998        1.0     200    RUN

Mot#  TIME-SET  TIME-DID   STEPS-SET   STEPS-DID   WIGGLES/  STEPS/   STATE

----   minutes   minutes   ---------   ---------     second  second   -----

4      5.00      2.75       60000       32998        1.0     200    RUN

4      5.00      3.00       60000       35998        1.0     200    RUN

4      5.00      3.25       60000       38998        1.0     200    RUN

4      5.00      3.50       60000       41998        1.0     200    RUN

4      5.00      3.75       60000       44998        1.0     200    RUN

4      5.00      4.00       60000       47998        1.0     200    RUN

4      5.00      4.25       60000       50998        1.0     200    RUN

4      5.00      4.50       60000       53997        1.0     200    RUN

Mot#  TIME-SET  TIME-DID   STEPS-SET   STEPS-DID   WIGGLES/  STEPS/   STATE

----   minutes   minutes   ---------   ---------     second  second   -----

4      5.00      0.25       60000        2999        1.0     200    RUN

4      5.00      0.32       60000        3895        1.0     200    RUN

\b\CTRL/C                                                !Interrupt via CTRL/C\\

TWIG->\b\stat                                            !Display short status\\

Mot#  TIME-SET  TIME-DID   STEPS-SET   STEPS-DID   WIGGLES/  STEPS/   STATE

----   minutes   minutes   ---------   ---------     second  second   -----

4   1333.33      0.34    16000000        4069        1.0     200    RUN

TWIG->\b\stop                                            !STOP motor\\

Mot#  TIME-SET  TIME-DID   STEPS-SET   STEPS-DID   WIGGLES/  STEPS/   STATE

----   minutes   minutes   ---------   ---------     second  second   -----

4   1333.33      0.00    16000000           0        1.0     200    STOP

QIT$