RPD$068066

CHP$Beam-Particle Transit-Time - TRANT

TPF$

BLP$

TRANT is a simple program which calculates transit-times and changes

in transit-time due small changes in beam energy for a system of

drift and acceleration regions such as found in a tandem accelerator.

The input data for TRANT consists of two parts: The first defines the

geometry of the system and the second specifies ions and energies

for which calculations are to be done.

CYB$

Geometry Data ----------------------------------------------------

BLP$

The geometry is specified by up to 100 fixed-format entries of the

following form:

CY1$

0000000001111111111222222222233333 ---- Col. Number

1234567890123456789012345678901234

LABEL       KIND ISTR FLG XM

ODP$

LABEL..is any label that you desire up to 8 characters in length.

ODP$

KIND...can take on one of the following names: DRIF denotes a drift

region, IACC denotes an injector acceleration region, TACC denotes

a tandem acceleration region.

ODP$

ISTR...can take on one of the following names: PRES denotes pre-strip

region and POSS denotes post-strip region.

ODP$

FLG....is usually blank but if non-blank causes program to insert

a line of **** preceding specified region.

ODP$

XM.....is the length of the region in meters.

ODP$

END....entered in the "label field" terminates the "geometry data".

CYB$

Ion Specification Data -------------------------------------------

BLP$

The remaining data are entered free-form. The first two entries defined

should really be computed by the program or belong in the Geometry

section but that's not how it is.

CY1$

TANL = X   ;Specify active acceleration length X(meters) for tandem

INJL = X   ;Specify active acceleration length X(meters) for injector

E0   = X   ;Specify ion energy X(MeV) after extraction

;That is a zero following the E not an OH

VINJ = X   ;Specify injector voltage X(MV)

VTAN = X   ;Specify tandem terminal voltage X(MV)

AMU  = X   ;Specify ion mass X(amu)

Q    = X   ;Specify post stripper charge state X

GO         ;Says do the calculation

BLP$

Only those quantities which are to be changed need to be entered

for sucessive calculations "GOs".

CEN$

(see next page for definition of output quantities)

TPF$

CYB$

Definition of Output Quantities ----------------------------------

BLP$

The quantities AMU, E0, VINJ, VTAN & Q are printed at the top of the

page. The columns labeled REGION, ---TYPE---, and XL(M) reproduce the

geometry input data. The remaining columns are defined as follows:

RND$09

ODP$

E(MeV)...gives the beam energy (in MeV) at the entrance (beginning) to

the region specified.

ODP$

T(NS)....is the transit-time (in nanoseconds) to the beginning of the

region being defined.

ODP$

KV/NS....is the number of keV that the energy of a beam particle must

be changed in order that the arrival time at the "target" (end of the

last region specified) is shifted by 1 nanosecond. This is the useful

quantity.

ODP$

DKV4.....is just 4 times KV/NS.

ODP$

DKV125...is just 125 times KV/NS.

ODP$

DKV4/Q...is just DKV4 divided by Q.

QIT$